3GPP TSG-RAN WG4 Meeting # 113	R4-2419046

# Maastricht, Netherland, August 19 – 23, 2024

Source:	Huawei, HiSilicon, China Telecom, China Unicom

Title:	TP for TR 37.719-11-10 to introduce DC_5A_SUL_n78A-n89A

Agenda item:	6.2.5

Document for:	Approval

1 Background

This contribution provides text proposal on the NSA SUL band combination DC_5A_SUL_n78A-n89A.

2 Text Proposal

---Start of changes---

6.Y	DC_5_SUL_n78-n89

6.Y.1	Configuration

Table 6.Y.1-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_SUL_n78A-n89A | DC_5A_n78A DC_5A_n89A_ULSUP-TDM_n78A |

6.Y.2	Maximum output power

Table 6.Y.2-1: Maximum output power for inter-band EN-DC

| DC configuration | Power class X (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n78A | 23 | +2/-3 |
| DC_5A_n89A_ULSUP-TDM_n78A | 23 | +2/-3 |

6.Y.3	Spurious emission band UE co-existence

Table 6.Y.3-1: Spurious emissions for inter-band EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5_n78 DC_5_n89_ULSUP-TDM_n78 | Frequency range | 859 | - | 869 | -27 | 1 |  |

6.Y.4	MSD

Referring to the DC_5A_n78A, the MSD due to the IMD4 for DC_5A_SUL_n78A-n89A can be specified.

Table 6.Y.4-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
| DC_5A_SUL_n78A-n89A | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |

6.Y.5	∆TIB and ∆RIB values

Table 6.Y.5-1: ΔTIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_5_SUL_n78-n89 | 0.6 | 0.8 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

Table 6.Y.5-2: ΔRIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_5_SUL_n78-n89 | 0.2 | 0.5 | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

---End of changes---

Reference

[1]	RP-241786, “Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands”, Nokia