# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419056

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon, MediaTek, Spreadtrum Communications

Title: 	(NR_NTN_solutions-Core) Discussion on the clarification of UE speed application for the real scenario

Agenda Item:	4.2.2

Document for:	Approval

Introduction

In last meeting, one issue [1] was raised about how to clarify the frequency error requirements for the real scenario. In the end, we have a way forward [2] to further discuss this issue.

- Issue 1-4: Frequency error Agreement: Discuss changes common to non-Redcap NTN and (e)RedCap NTN NR UE frequency error in maintenance agenda. (e)RedCap UE frequency error requirement may be impacted by this maintenance discussion.

Currently, the frequency error requirements for NTN UE are specified below.

- 6.4.1 Frequency error The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

Generally, the ± 0.1 PPM frequency error requirement is only specified for RF components performance in RAN4 RF. However, for NTN UE, the UL pre-compensated behaviour and performance is also included. In Rel-17, the frequency error of the UL doppler pre-compensated behaviour was ignored based on the assumption that an NR NTN UE is capable of acquiring the exact GNSS location and satellite ephemeris.

But there is no clear requirement on UL Doppler pre-compensated performance in the real scenario, e.g. UE has a large velocity. If an NR NTN UE has to maintain the same frequency error requirements, that means NR NTN UE has to improve the update period of GNSS, which will lead to the higher power consumption for NR NTN UE.

Currently, constant doppler frequency shift doesn’t reflect the real scenario any more. Thus, in this paper, we’d like to clarify this issue.

Discussion on Rel-17 frequency error requirements

In the RAN4#98bis-e meeting, RAN4 concluded the frequency error requirements by sending LS [3] to RAN1 based on the companies’ contributions [4] [5] [6].

Before RAN4 concluded such requirements, there is an assumption or condition, which was captured into the status report in RAN#90e [7].

RAN1 Agreements on “UL time and frequency synchronization” - An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link.

It’s highlighted that An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link.

Based on this consensus or assumption, RAN4 specified the frequency error requirements in the approved TP [8] and highlighted that the location of the UE is explicitly provided to the UE from the test equipment.

Observation 1: RAN4 concluded the frequency error requirements including doppler pre-compensation error as 0.1ppm based on the assumption that an NR NTN UE is capable of acquiring the exact GNSS location and satellite ephemeris.

From the real system and UE implementation perspective, it’s very hard to assume that UE shall be always capable of acquiring the exact GNSS location and satellite ephemeris. After going through the Rel-17 discussion in NTN topic, RAN4 never discuss the frequency error requirements for NTN UE when UE aren’t capable of acquiring the exact GNSS location and satellite ephemeris. However, in the RAN4#109 meeting, the CR [9] for Rel-18 NGSO testing work removed this clarification below as it’s just for test case.

- 6.4.1 Frequency error The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

Observation 2: RAN4 never discuss the frequency error requirements for NTN UE when UE can’t acquire the exact GNSS location and satellite ephemeris.

However, in case reader misunderstand that current 0.1ppm frequency error requirements for NTN UE are applicable to any scenarios without pre-condition, it’s necessary to clarify that current 0.1ppm frequency error requirements for NTN UE including NTN RedCap UE are only applicable when NTN UE is capable of acquiring the exact GNSS location and satellite ephemeris.

Discussion on GNSS issue

Generally, current NR system is designed to cover 250km/h UE velocity as basic capability without any UL Doppler pre-compensation. For NR NTN, the frequency pre-compensation is specified in clause 16.14.2.2 of TS 38.300 below.

- 16.14.2.2 Timing Advance and Frequency Pre-compensation For the serving cell, the network broadcast valid ephemeris information and Common TA parameters. The UE shall have valid GNSS position as well as ephemeris and Common TA before connecting to an NTN cell. To achieve synchronisation, before and during connection to an NTN cell, the UE shall compute the RTT between UE and the RP based on the GNSS position, the ephemeris, and the Common TA parameters (see clause 4.2 in TS 38.213 [38]), and autonomously pre-compensate the TTA for the RTT between the UE and the RP as illustrated in Figure 16.14.2.1-1 (see clause 4.3 of TS 38.211 [52]). The UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris. If the UE does not have a valid GNSS position and/or valid ephemeris and Common TA, it shall not transmit until both are regained. In connected mode, the UE shall be able to continuously update the Timing Advance and frequency pre-compensation. The UE may be configured to report Timing Advance during Random Access procedures or in connected mode. In connected mode, event-triggered reporting of the Timing Advance is supported.

Observation 3: The UE shall compute the frequency Doppler shift of the service link by considering UE position and the ephemeris referring to the clause 16.14.2.2 of TS 38.300.

Generally, the ± 0.1 PPM frequency error requirement is only specified for RF components’ performance in RAN4 RF. However, RAN4 didn’t discuss the performance requirements of UE UL frequency pre-compensation.

Observation 4: Generally, the ± 0.1 PPM frequency error requirement is only specified for RF components’ performance in RAN4 RF. However, RAN4 didn’t discuss the performance requirements of UE UL frequency pre-compensation in Rel-17.

For a real scenario illustrated below, it is assumed that a UE is moving with 250km/h velocity from Position 1 at T1 to Position 2 at T2 within approximate one minute.

figure 1 the real scenario illustration for 250km/h UE velocity

If UE GNSS position is updated larger than every one minute, the UE frequency pre-compensation error is about fd_error=(Vsat×sinθ-Vsat×cos90°)/λ=Vsat×sinarctan4.1600/c×f0=0.17ppm (Vsat = 7559m/s, LEO600)

Based on this model, we calculate the UE doppler pre-compensation error for different UE speed and different GNSS update periods below.

table 1 UE doppler pre-compensation error for different UE speeds and different GNSS update periods

| additional frequency error/ppm |  |  |  |  |  |  |  | GNSS update period / second |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10km/h UE | 20km/h UE | 80km/h UE | 100km/h UE | 120km/h UE | 250km/h UE | 500km/h UE | 1000km/h UE |  |
| 0.000 | 0.000 | 0.001 | 0.001 | 0.001 | 0.003 | 0.006 | 0.012 | 1 |
| 0.001 | 0.001 | 0.005 | 0.006 | 0.007 | 0.015 | 0.029 | 0.058 | 5 |
| 0.001 | 0.002 | 0.009 | 0.012 | 0.014 | 0.029 | 0.058 | 0.117 | 10 |
| 0.002 | 0.005 | 0.019 | 0.023 | 0.028 | 0.058 | 0.117 | 0.233 | 20 |
| 0.003 | 0.007 | 0.028 | 0.035 | 0.042 | 0.087 | 0.175 | 0.350 | 30 |
| 0.005 | 0.009 | 0.037 | 0.047 | 0.056 | 0.117 | 0.233 | 0.467 | 40 |
| 0.006 | 0.012 | 0.047 | 0.058 | 0.070 | 0.146 | 0.292 | 0.583 | 50 |
| 0.007 | 0.014 | 0.056 | 0.070 | 0.084 | 0.175 | 0.350 | 0.700 | 60 |
| 0.008 | 0.016 | 0.065 | 0.082 | 0.098 | 0.204 | 0.408 | 0.816 | 70 |
| 0.009 | 0.019 | 0.075 | 0.093 | 0.112 | 0.233 | 0.467 | 0.933 | 80 |
| 0.010 | 0.021 | 0.084 | 0.105 | 0.126 | 0.262 | 0.525 | 1.049 | 90 |

As we all know, the shorter GNSS update period UEs have to maintain, the higher power consumption UEs have to bear.

Observation 5: The shorter GNSS update period can help UE acquire the exact GNSS location timely. However, the shorter GNSS update period UEs have to maintain, the higher power consumption UEs have to bear.

In some areas without terrestrial IMT network coverage, it’s still very important to reduce UE power consumption and keep connection with satellite by reducing the GNSS update frequency.

If NTN UE has to maintain the 0.1ppm frequency error for all kinds for UE speed scenarios, NTN UE has to keep one second GNSS update period. This kind of implementation with high GNSS update frequency will lead to much higher UE power consumption.

Based on the calculation above, for 10km/h UE speed scenario, the additional frequency error from doppler pre-compensation will be less than 0.01ppm even if 90s GNSS update frequency is assumed.

Observation 6: Based on the calculation, for 10km/h UE speed scenario, the additional frequency error from doppler pre-compensation will be less than 0.01ppm even if 90s GNSS update period is assumed.

Additionally, in the most of scenarios for handheld UE communicating with satellite, the UE speed is less than 10km/h. As RAN4 have to balance the UE speed scenarios, GNSS update period and accuracy of GNSS location, it’s better to clarify the UE speed scenarios for current frequency error requirements.

Proposal 1: To clarify that current 0.1ppm frequency error requirements for handheld NR NTN UE are only applicable to UE speed less than or equal to 10km/h.

Summary

Observation 1: RAN4 concluded the frequency error requirements including doppler pre-compensation error as 0.1ppm based on the assumption that an NR NTN UE is capable of acquiring the exact GNSS location and satellite ephemeris.

Observation 2: RAN4 never discuss the frequency error requirements for NTN UE when UE can’t acquire the exact GNSS location and satellite ephemeris.

Observation 3: The UE shall compute the frequency Doppler shift of the service link by considering UE position and the ephemeris referring to the clause 16.14.2.2 of TS 38.300.

Observation 4: Generally, the ± 0.1 PPM frequency error requirement is only specified for RF components’ performance in RAN4 RF. However, RAN4 didn’t discuss the performance requirements of UE UL frequency pre-compensation in Rel-17.

Observation 5: The shorter GNSS update period can help UE acquire the exact GNSS location timely. However, the shorter GNSS update period UEs have to maintain, the higher power consumption UEs have to bear.

Observation 6: Based on the calculation, for 10km/h UE speed scenario, the additional frequency error from doppler pre-compensation will be less than 0.01ppm even if 90s GNSS update period is assumed.

Proposal 1: To clarify that current 0.1ppm frequency error requirements for handheld NR NTN UE are only applicable to UE speed less than or equal to 10km/h.

- References

[1] R4-2409336, Discussion on the other potential impacts for Rel-19 NTN RedCap UE, Huawei, HiSilicon

[2] R4-2410574, WF on Rel-19 phase 3 NR-NTN UE RF requirements, Qualcomm

[3] R4-2106174, LS reply on NTN UL time and frequency synchronization requirements, RAN4

[4] R4-2104762, Discussion on RF requirements for NTN UE, CATT

[5] R4-2106900, Reply LS to RAN4 on NTN UL time and frequency synchronization requirements (frequency), Ericsson

[6] R4-2106361, Discussion on UE UL frequency synchronization requirements in NTN, MediaTek Inc.

[7]	RP-20257, NR NTN Status Report to RP#90e, December 2020

[8] R4-2211220, TP for 38.101-5 on frequency error, Qualcomm

[9] R4-2321032, CR to 38.101-5 on clarification for NR NTN UE RF and Demod requirements test conditions, Apple, Ericsson, Samsung, MediaTek Inc