3GPP TSG-RAN WG4 Meeting # 113	R4-2419070

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on coexistence study for NR NTN Ku Band

Agenda Item:	7.9.2

Document for:	Approval

Introduction

In the RAN4#112 and RAN4#112bis meeting, RAN4 reached some initial consensus on adjacent coexistence studies for Ku band in the Way forward [1] [2].

- RAN4#112 meeting: - Issue 2-2: Coexistence scenarios Adopt the interference scenarios in [Intelsat] R4-2411507 for the coexistence studies - Issue 2-4: Coexistence frequencies - Use 11GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation for Ku-band. - Issue 2-8: Coexistence TN channel model For TN channel model, reuse existing version of TR 38.901. - Issue 2-9: Coexistence TN channel model For NTN channel model, reuse parameters from TR 38.811 and collect the following missing information: - Shadow fading and clutter loss tables for the reference frequencies - Tropospheric scintillation. - Issue 2-10: TN Parameters As starting point, reuse the antenna parameters and BS/UE ACLR/ACS from: - The 10.0-10.5 GHz frequency range (TR 38.921) for the reference frequency of 11 GHz - The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz. - RAN4#112bis meeting: - Sub-topic 2-1: SAN equivalent antenna aperture - Use 9.1 m DL, 7.14 UL (GEO ), 0.91 m DL, 0.714 UL (LEO DL 600 & 1200) - Issue 2-8: Coexistence TN channel model For TN channel model, reuse existing version of TR 38.901. - Sub-topic 2-2: SAN antenna pattern Use section 6.4.1 from 38.811 - Sub-topic 2-3: SAN Tx antenna maximum gain Use 58.5 dBi (GEO), 38.5 dBi (LEO 600 & 1200) - Sub-topic 2-4: SAN Rx antenna maximum gain Use 58.5 dB (GEO), 38.5 dB (LEO 600 & 1200) - Sub-topic 2-5: SAN antenna beamwidth Use 0.177 (GEO), 1.768 (LEO 600 & 1200) - Sub-topic 2-7: SAN max EIRP 43 dBw/MHz (GEO), 13 dBw/MHz (LEO 1200), 7 dBW/MHz (LEO 600) using 100 MHz bandwidth - Sub-topic 2-9: VSAT equivalent antenna aperture Use 60 cm - Sub-topic 2-10: VSAT polarization Assume circular first and if necessary derive the linear result - Sub-topic 2-11: VSAT max Tx antenna gain Use 36.6 dBi (option 1 2W), 36.2 dBi (option 2 10W) - Sub-topic 2-12: VSAT max Rx antenna gain 34.3 dBi - Sub-topic 2-15: VSAT noise figure Type 1 2.5 dB for VSAT LEO and GEO Type 2 6 dB for VSAT to LEO Other options are not precluded - Sub-topic 2-16: VSAT max Tx power 40 dBm (10 W) - Sub-topic 2-21: Channel model 38.811 is the channel model for satellite used for Ka. It can be scaled for Ku. For TN use 38.863 and change the frequency to 11 & 14 GHz - Sub-topic 2-22: TN parameters NTN to NTN won’t be considered. Continue with the TN - NTN coexistence study noting that the applicability of the VSAT ACS simulation result may have to be relaxed for the core requirement derivation. Use the IMT parameters from TR38.922 version 0.3.0 for the IMT study on 15 GHz as a starting point.

In this paper, we’d like to share our views and studies to continue the discussion of adjacent coexistence study assumption in Ku band.

Discussion

Phased array antenna model for VSAT

In last meeting, RAN4 reached the following way forward [3] to encourage companies to contribute the flat-panel or phase-array antenna patterns and modeling.

• Encourage Ku-band VSAT vendors to contribute the flat-panel or phase-array antenna patterns and modeling.

Thus, we’d like to propose the phase-array antenna patterns and modeling for Ku band VSAT in detail. The VSAT phased array antenna characteristics in Ku band is proposed in the table 2-1. The coordinate system for VSAT itself is shown in the figure 1.

Table 2-1 VSAT phased array antenna characteristics

| Ku band VSAT phased array antenna |  |  |
| --- | --- | --- |
| 1 | Ku band VSAT phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 2) | 0 |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 4) | 60*60 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.01m (almost 0.5 of wavelength for in 14GHz and 11GHz) |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 4.44 dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | normal direction | Toward X+ axis |

Note 1:	Void

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element).

Note 4:	All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane.

Figure 1 the geometry distribution of VSAT with phased array antenna model

In Table 2-2, the parameters used by the parameterized array antenna model are described.

Table 2-2: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Y axis element HPBW | 3dB | Degrees |
| Z axis element HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements in Y axis and Z axis | (M, N) | Integer |
| Y axis element separation | dy | m |
| Z axis element separation | dz | m |
| Y axis Electrical scan angle | Yescan | Degrees |
| Z axis Electrical scan angle | Zescan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 2-3.

Table 2-3: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dzλcosθ+n-1dyλsinθsinφ wm,n=1MNexpj2πm-1dzλsinθZescan-n-1dyλcosθZescansinφYescan | dBi |

Proposal 1: To use the parameterized phased array antenna model for VSAT when performing the coexistence studies.

10GHz VS 15GHz TN parameters

In the last two meetings, we have the following way forward for TN parameters.

RAN4#112bis meeting: As starting point, reuse the antenna parameters and BS/UE ACLR/ACS from: • The 10.0-10.5 GHz frequency range (TR 38.921) for the reference frequency of 11 GHz • The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz. RAN4#113 meeting: Use the IMT parameters from TR38.922 version 0.3.0 for the IMT study on 15 GHz as a starting point.

However, it seems that some TN/BS parameters are different between TR 38.921 (10GHz) and TR 38.922 (15GHz). Currently, the BS parameters for different frequency range are shown below.

|  |  | Macro suburban/urban | Macro Sub-urban | Urban macro |
| --- | --- | --- | --- | --- |
| 0 | Frequency range | 7.125 – 8.4 GHz (Referring to TR 38.922) | 10~10.5GHz (Referring to TR 38.921) | 14.8 – 15.35 GHz (Referring to TR 38.922) |
| 1.1 | Antenna pattern | Clause 5.2.3.2.4 of TR 38.803 (Sub-array antenna model) | Clause 8.1 of TR 38.921 (Non-sub-array antenna model) | Clause 5.2.3.2.4 of TR 38.803 (Sub-array antenna model) |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 x 16 | 16 x 8 | 16 x 24 |
| 1.7 | Horizontal/Vertical radiating sub-array or element spacing (Note 5) | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 2.8 of wavelength for V (Note 5) |
| 1.7a | Number of element rows in sub-array | 3 | 1 | 3 |
| 1.7b | Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength for V | NA | 0.7 of wavelength of V |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | NA | 3 |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 22 | 22 | 17.14 |
| 1.10 | Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 |
| 1.12 | Mechanical down-tilt (degrees) | 6 | 6 | 6 |

It’s observed that the parameters for 10GHz are not better than the new agreed 7~8GHz. Thus, it’s better to discuss whether the 10GHz can be updated to refer to the new agreed 7~8GHz or 15GHz TN parameters.

Proposal 2: the TN BS parameters in 10GHz can be updated, referring to the new agreed 7~8GHz or 15GHz TN parameters.

For the UE type in 10~14GHz, there are some observation in the clause 8.2 of TR 38.921. The UE in 10 - 10.5 GHz range will most likely have a conducted interface with an assumed isotropic radiation pattern antenna and no beamforming.

- 8.2 UE antenna characteristics The outcome of the RAN4 study for collecting technical background information relevant for the frequency range 7 to 24 GHz indicated that the frequency range 7.25-[10-13] GHz would have “FR1 like” requirements, and as such we can assume that in the 10 - 10.5 GHz range this applies. The UE will most likely therefore have a conducted interface with an assumed isotropic radiation pattern antenna and no beamforming.

In the TR 38.922 v0.3.0, both FR1-like and FR2-like TN UE antenna pattern were captured.

- 6.1.2.3.3 UE antenna element pattern - 6.1.2.3.3.1 FR1 like In this approach, a UE will most likely have a conducted interface with an assumed isotropic radiation pattern antenna with [0 or 5] dB receive diversity gain and no beamforming. - 6.1.2.3.3.2 FR2 like Table 6.1.2.3.3.2-1: UE antenna element pattern Parameter; Values. Antenna element vertical radiation pattern (dB); AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=25dB. Antenna element horizontal radiation pattern (dB); AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=25dB. Combining method for 3D antenna element pattern (dB); A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am. Maximum directional gain of an antenna element GE,max; 5 dBi. (Mg, Ng, M, N, P); (1, 1, 2, 2, 2). (dv, dh); (0.5λ, 0.5λ). UE orientation; Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees. NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees..

In our understanding, “FR1-like” TN UE can be assumed for NTN Ku band coexistence study to simplify the simulation.

Proposal 3: For Ku band adjacent channel coexistence study, no beamforming “FR1 like” can be assumed for TN UE for both 11GHz and 14GHz.

Propagation model

In the last two meetings, we have the following Way forward for propagation model.

- RAN4#112 meeting: - Issue 2-9: Coexistence TN channel model For NTN channel model, reuse parameters from TR 38.811 and collect the following missing information: - Shadow fading and clutter loss tables for the reference frequencies - Tropospheric scintillation. - RAN4#112bis meeting: - Sub-topic 2-21: Channel model 38.811 is the channel model for satellite used for Ka. It can be scaled for Ku. For TN use 38.863 and change the frequency to 11 & 14 GHz

However, if we looked at the assumption for propagation in the Ka band, the previous way forward seems not clear.

Table 6a.2.5-1: Propogation models between NTN and TN Link; Propagation model. TN BS to Fixed VSAT on roof; Free space path loss. TN BS to L-ESIM at 1.5 m; UMa as in 3GPP TR 38.803. TN BS to TN UE; UMa as in 3GPP TR 38.803. TN UE to Fixed VSAT on roof; UMa as in 3GPP TR 38.803 (BS is to be replaced with VSAT). TN UE to L-ESIM; Umi. Satellite to TN BS/UE; 3GPP TR 38.821. Satellite to VSAT/ESIM; 3GPP TR 38.821. TN BS to Satellite; 3GPP TR 38.821. Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. Note2: TN BS height is 25m.

It seems that we can only consider the scenarios that VSAT/ESIM is located on the roof as worst case, so the Propagation models between NTN and TN can be simplified.

Proposal 4: to simplify the Propagation models between NTN and TN as below.

| Link | Propagation model |
| --- | --- |
| TN BS to Fixed VSAT on roof | Free space path loss |
| TN BS to TN UE | UMa as in 3GPP TR 38.803 |
| TN UE to Fixed VSAT on roof | UMa as in 3GPP TR 38.803 (BS is to be replaced with VSAT) |
| Satellite to TN BS/UE | 3GPP TR 38.821 |
| Satellite to VSAT/ESIM | 3GPP TR 38.821 |
| TN BS to Satellite | 3GPP TR 38.821 |
| Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. Note2: TN BS height is 25m Note3: The height of fixed VSAT is 25m. |  |

Summary

Proposal 1: To use the parameterized phased array antenna model for VSAT when performing the coexistence studies.

Table 2-1 VSAT phased array antenna characteristics

| Ku band VSAT phased array antenna |  |  |
| --- | --- | --- |
| 1 | Ku band VSAT phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | Table 2-2 and 2-3 (TR 38.921) |
| 1.2 | Element gain (dBi) (Note 2) | 0 |
| 1.3 | Y axis / Z axis element 3 dB beam width of single element (degree) | 90° for Y axis 90° for Z axis |
| 1.4 | Y axis / Z axis element front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Y axis × Z axis)(Note 4) | 60*60 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Y axis / Z axis radiating element spacing | 0.01m (almost 0.5 of wavelength for in 14GHz and 11GHz) |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 4.44 dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | normal direction | Toward X+ axis |

Note 1:	Void

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element).

Note 4:	All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane.

Figure 1 the geometry distribution of VSAT with phased array antenna model

Table 2-2: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Y axis element HPBW | 3dB | Degrees |
| Z axis element HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements in Y axis and Z axis | (M, N) | Integer |
| Y axis element separation | dy | m |
| Z axis element separation | dz | m |
| Y axis Electrical scan angle | Yescan | Degrees |
| Z axis Electrical scan angle | Zescan | Degrees |

Table 2-3: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dzλcosθ+n-1dyλsinθsinφ wm,n=1MNexpj2πm-1dzλsinθZescan-n-1dyλcosθZescansinφYescan | dBi |

Proposal 2: the TN BS parameters in 10GHz can be updated, referring to the new agreed 7~8GHz or 15GHz TN parameters.

|  |  | Macro suburban/urban | Macro Sub-urban | Urban macro |
| --- | --- | --- | --- | --- |
| 0 | Frequency range | 7.125 – 8.4 GHz (Referring to TR 38.922) | 10~10.5GHz (Referring to TR 38.921) | 14.8 – 15.35 GHz (Referring to TR 38.922) |
| 1.1 | Antenna pattern | Clause 5.2.3.2.4 of TR 38.803 (Sub-array antenna model) | Clause 8.1 of TR 38.921 (Non-sub-array antenna model) | Clause 5.2.3.2.4 of TR 38.803 (Sub-array antenna model) |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 x 16 | 16 x 8 | 16 x 24 |
| 1.7 | Horizontal/Vertical radiating sub-array or element spacing (Note 5) | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 2.8 of wavelength for V (Note 5) |
| 1.7a | Number of element rows in sub-array | 3 | 1 | 3 |
| 1.7b | Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength for V | NA | 0.7 of wavelength of V |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | NA | 3 |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 22 | 22 | 17.14 |
| 1.10 | Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 |
| 1.12 | Mechanical down-tilt (degrees) | 6 | 6 | 6 |

Proposal 3: For Ku band adjacent channel coexistence study, no beamforming “FR1 like” can be assumed for TN UE for both 11GHz and 14GHz.

Proposal 4: to simplify the Propagation models between NTN and TN as below.

| Link | Propagation model |
| --- | --- |
| TN BS to Fixed VSAT on roof | Free space path loss |
| TN BS to TN UE | UMa as in 3GPP TR 38.803 |
| TN UE to Fixed VSAT on roof | UMa as in 3GPP TR 38.803 (BS is to be replaced with VSAT) |
| Satellite to TN BS/UE | 3GPP TR 38.821 |
| Satellite to VSAT/ESIM | 3GPP TR 38.821 |
| TN BS to Satellite | 3GPP TR 38.821 |
| Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. Note2: TN BS height is 25m Note3: The height of fixed VSAT is 10m. |  |

- References

[1] R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[2] R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[3] R4-2416563, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

[4] TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz, V17.1.0

[5] TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz, V0.3.0