3GPP TSG-RAN WG4#113	R4-2419083

Orlando, US, November 18th – 22nd, 2024

Agenda item:	7.5.2.2

Source: 	Qualcomm France

Title: 	Non-collocated Intra-band NR CA/EN-DC Signaling

Document for:	Approval

- Introduction

Considerations on other aspects for Non-collocated NR CA/EN-DC UE are provided in this contribution.

- WF in RAN4#112-bis

WF [1]:

## < Issue 2-1: How to differentiate UE Capability Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC >

Agreement:

- Legacy capabilities, such as MIMO-CapabilityDL-r10 on E-UTRA carriers

## < Issue 2-2: UE Capability Type 4a(6Rx/UE) for NR-CA >

Agreement:

- Do not consider Type 4a for NR-CA

## < Issue 2-3: BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated) >

Way Forward:

- Continue further discussion on BS signalling in the next meeting.

## < Issue 2-4: BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) >

Way Forward:

- Continue further discussion on BS signalling in the next meeting.

## < Issue 2-5: Switch between Type 4a/4b and single CC operation >

Agreement:

- No new NW signaling needed from Type 4 to single carrier fallback.

## < Issue 2-6: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b >

Way Forward:

- Continue further discussion on BS signalling in the next meeting.

## < Issue 2-7: When to inform RAN2 the demand on new UE capability(s) and new BS signaling >

Agreement:

- After RAN4 finish the work according the WID including all UE capability and NW signalling, the RAN4 will send the decisions related into RAN2 on one go.

- Discussion

< Issue 2-3: BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated) >

It would be beneficial to define new BS signalling for this case, to maintain consistency with Type 2 signaling. An issue when the NW is Rel-18 and UE is Rel-19 was discussed by interested companies in previous meeting. We will return to later later in this submission.

Proposal 1: Specify BS signalling for NR CA and EN-DC, e.g. “nonCollocatedTypeNR-CA-r19” and

“nonCollocatedTypeMRDC-r19”

< Issue 2-4: BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) >

UE supporting Type 4a/4b could optionally support 6L/8L per CC in collocated mode (Type 1). The default is anyhow 4L/CC for Type 1.

UE can move from Type 1 to Type 4 with BS signaling, as NW can configure UE which is in Type 1 with 6L/8L with 2< maxMIMO-layers ≥4 and Type 4 BS signaling.

Going the other way, UE in Type 4 with 4L can be configured by the NW to Type 1 with  2< maxMIMO-layers ≥4, and the NW can configure UE to 6L/8L.

There is no new signaling required for that as UE can already in current specifications indicate maxMIMO-Layers and can reconfigure between different number of MIMO layers when in Type 1.

Proposal 2: No need for BS signalling to switch between Type 4a/4b and Type 1 with 6L/8L per CC

< Issue 2-6: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b >

In the proposal below, we want to clarify everyone is on the same page, i.e. “maxMIMO-layers” is the measure configured by the NW; not the value indicated by UE in its capabilities. Case when maxMIMO-layers >4 is clear, UE is Type 1.

When maxMIMO-layers ≤2:

- NW and UE both Rel-18

  - No issue, as UE indicates only Rel-18 capabilities, NW uses BS signaling accordingly

  - Case where Rel-18 BS signaling elements are present => Type 1

  - Case where Rel-18 BS signaling elements are absent = > Type 2

- NW Rel-18, UE Rel-19

  - No issue, as UE indicates only Rel-18 capabilities, NW uses BS signaling accordingly

  - Case where Rel-18 BS signaling elements are present => Type 1

  - Case where Rel-18 BS signaling elements are absent = > Type 2

- NW Rel-19, UE Rel-18

  - No issue, as UE indicates only Rel-18 capabilities, NW uses BS signaling accordingly

  - Case where Rel-18 BS signaling elements are present => Type 1

  - Case where Rel-18 BS signaling elements are absent = > Type 2

- NW and UE both Rel-19

  - Case where Rel-18 BS signaling elements are present => Type 1

  - Case where both Rel-18 BS signaling elements are absent = > Type 2

So overall the situation is as follows.

-UE is type 1 if nonCollocatedTypeMRDC and nonCollocatedTypeNR-CA for Rel-18 are present

-UE is type 2 if nonCollocatedTypeMRDC and nonCollocatedTypeNR-CA for Rel-18 are absent

Case when when 2< maxMIMO-layers ≥4 needs a bit of work.

- NW and UE both Rel-18

  - UE is Type 1 because for rel-18 feature >2 MIMO layers means collocated

- NW Rel-18, UE Rel-19

  - This case needs special attention:

    - UE indicates both Rel-18 and Rel-19 capabilities and MaxMIMO-layers>2

    - NW does not understand Rel-19 capabilities but Rel-18 only

    - With this, NW intention with maxMIMO-layers>2 is to say Type 1, but if UE sees both Rel-18 and Rel-19 capabilities absent, it assumes NW is asking for “Type 4”

    - We suggest that RAN4 agrees Rel-19 BS signaling elements are present when NW supports Rel-19, with a distinction for Type 1 and Type 4. When Rel-19 BS elements are not present, UE behaves according to Rel-18 BS signaling

- NW Rel-19, UE Rel-18

  - UE is Type 1 because for Rel-18 feature >2 MIMO layers means collocated

- NW and UE both Rel-19

  - Case where Rel-19 BS signaling elements are present => Type 1 or Type 4 depending on distinction

  - Cases where Rel-19 BS signaling elements are absent = > Type 1

Proposal 3: UE behaviour during initial connection is assumed as follows for a UE which supports intraBandNR-CA-non-collocated-r19 and/or interBandMRDC-WithOverlapDL-Bands-r19. Please note that “maxMIMO-layers” is the measure configured by the NW; not the value indicated by UE in its capabilities.

When maxMIMO-layers >4

- UE is Type 1

When 2< maxMIMO-layers ≥4

- UE is Type 1 or Type 4 depending on always present Rel-19 BS signaling element distinction

  - NOTE: RAN2 has the authority to modify this, as long as case of Rel-18 NW vs Rel-19 UE remains resolved

- UE is Type 1 when Rel-19 BS signaling is absent i.e. UE follows Rel-18 BS signaling

When maxMIMO-layers ≤2

- UE is type 1 when Rel-18 BS signaling elements are present

- UE is type 2 when Rel-18 BS signaling elements are absent

- Conclusion

Remaining aspects for Non-collocated Rel-19 Intra-band Non-contiguous CA/EN-DC were considered with the following observations and proposals.

Proposal 1: Specify BS signalling for NR CA and EN-DC, e.g. “nonCollocatedTypeNR-CA-r19” and

“nonCollocatedTypeMRDC-r19”

Proposal 2: No need for BS signalling to switch between Type 4a/4b and Type 1 with 6L/8L per CC

Proposal 3: UE behaviour during initial connection is assumed as follows for a UE which supports intraBandNR-CA-non-collocated-r19 and/or interBandMRDC-WithOverlapDL-Bands-r19. Please note that “maxMIMO-layers” is the measure configured by the NW; not the value indicated by UE in its capabilities.

When maxMIMO-layers >4

- UE is Type 1

When 2< maxMIMO-layers ≥4

- UE is Type 1 or Type 4 depending on always present Rel-19 BS signaling element distinction

  - NOTE: RAN2 has the authority to modify this, as long as case of Rel-18 NW vs Rel-19 UE remains resolved

- UE is Type 1 when Rel-19 BS signaling is absent i.e. UE follows Rel-18 BS signaling

When maxMIMO-layers ≤2

- UE is type 1 when Rel-18 BS signaling elements are present

- UE is type 2 when Rel-18 BS signaling elements are absent

- Reference

[1] R4-2417115, “WF on rest issues for intra-band non-collocated EN-DC/NR-CA”, KDDI