3GPP TSG RAN WG4 Meeting #113	R4-2419089

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.26.4.2

Source:	vivo

Title:	Discussion on RRM impacts on DL coverage for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 further analyzed the potential RRM impacts on the objectives of downlink coverage enhancements, support of regenerative payload. The considerations from companies and the latest progress were summarized in WF [1]. Based on the progress, the issues to be settled mainly focus on the aspect of DL coverage. In this contribution, we would like to provide further analysis of the potential RRM impacts on DL coverage based on the latest progress in RAN1.

- 2.	Discussion

  - Downlink coverage enhancements

In the last meeting, RAN4 initially discussed the potential RRM impacts if SSB periodicity enhancement is introduced.  The agreement is shown below.

### Issue 1-1: SSB periodicity enhancement Agreement: - RAN4 to wait for RAN1’s definitive decision on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements. FFS on the detailed list of RRM requirements to be affected, e.g., - uplink timing requirements, RRM requirements during transmission between different SSB periodicities, default assumption on the SSB and SMTC periodicities in handover/satellite switch with re-sync/RRC re-establishment/RRC connection release with redirection, cell identification, measurements, etc.

On the enhancement of SSB periodicity extension, as per the latest progress in RAN1[2], for the initial access procedure, the maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. RAN1 will proceed with further feasibility evaluation on whether 320ms can be supported as the maximum of the additional default value.

Agreement in RAN1#118 meeting Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

When the maximum of the additional default value is 160ms, we understand that there is no explicit impact on the core requirement. However, if 320ms is supported as the maximum of the additional default value, it would bring some impacts on RRM requirements..

Here we provide some relevant requirements as examples of how to modify the relevant RRM requirements:

- For timing error requirements for NTN, it requires that UE shall meet Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. If SSB periodicity is extended larger than 160ms, there may be no SSB available during the last 160ms. Consequently, the current Te_NTN requirement shall be re-considered.

- For the gradual timing adjustment, one of the adjustment rules is that the maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200ms, shall be Tq_NTN per 200 ms. When SSB periodicity is extended to 320ms or 640ms, the frequency shift may be larger, which may result in the current maximum aggregate adjustment rate not being sufficient to compensate for the frequency offset.

- For the requirements on SSB-based RLM/BFD, since the current maximum SSB periodicity can be configured is 160ms, the cases of evaluation period are divided by DRX configuration and the boundary is set as 320ms. If SSB periodicity is extended to 640ms, the boundary of evaluation/measurement period shall be updated accordingly.

If RAN1 agrees to introduce 320ms as the maximum additional default value, the detailed list of RRM requirements provided in the agreement can be the baseline for further checking. The corresponding discussion can be initiated after the agreement is reached from RAN1.

Proposal 1: RAN4 to wait for more progress from other WGs on whether 320ms can be supported as the maximum of the additional default value.

### Issue 1-3: RRM impacts due to Satellite Tx Beam Hopping Agreement: - RAN4 to wait for more progress from RAN1/2 on the detailed solutions to support a satellite T/Rx beam sweeping pattern, e.g. to adopt the concept of Rel-18 cell DTX/DRX, before discussing whether and how to modify the relevant RRM requirements.

Regarding the DRX/DTX aspects, it is still under discussion in other working groups and related to the final decision on the concrete maximum default value. The potential impacts of RAN4 can wait until RAN1/RAN2 figure out a clear solution.

Proposal 2: RAN4 to wait for more conclusions from RAN1/2 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES.

- Summary

Proposal 1: RAN4 to wait for more progress from other WGs on whether 320ms can be supported as the maximum of the additional default value.

Proposal 2: RAN4 to wait for more conclusions from RAN1/2 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES.

- References

- R4-2416866, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm Incorporated

- R1-2407767, FL Summary #1: NR-NTN downlink coverage enhancements, CMCC