3GPP TSG RAN WG4 Meeting #113	R4-2419090

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.26.4.1

Source:	vivo

Title:	Discussion on RRM requirements on (e)RedCap for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 further analyzed the potential RRM impacts on Rel-19 NR NTN WI, which mainly included the objectives of downlink coverage enhancements, support of regenerative payload and Redcap NTN. The considerations from companies and the latest progress on (e)RedCap NTN were summarized in WF [1]. As per the arrangement on agenda, in this contribution, we would like to provide our further analysis on aspects on RRM requirements on (e)RedCap NTN.

- 2.	Discussion

  - General consideration on RRM requirements

    - What RRM requirements are defined for (e)RedCap UE with FR1-NTN

      - Further discussion on the agreed requirements

#### Issue 2-2-1: Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN Online agreement <Agreement>: - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

In RAN4#112 meeting, it was agreed that RAN4 will not define requirements for Inter-RAT measurements in idle/inactive/connected mode. Specifically, these requirements are only defined for (e)Redcap under NR TN to E-UTRA TN scenarios. Following this logic, it is not quite necessary to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) either since the inter-RAT mobility between NR NTN to E-UTRA TN is not that typical compared with NR NTN to NR TN and NR TN to E-UTRA TN.

Proposal 1: RAN4 not to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

      - Requirements to be further determined

#### Issue 2-2-3: The Requirements to be further determined Online agreement <Agreement>: - Define NR Rel-17 and Rel-18 Conditional Handover requirements. - Include location-based CHO and time-based CHO. - Support earth moving cell scenario - Define RACH-less Handover requirements. - Define SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state. - FFS TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - FFS: MDT requirements in RRC_IDLE and RRC_INACTIVE state.

For SDT requirements, it was agreed that PUR requirements on IoT NTN may be referred. In our understanding, this agreement can be confirmed and the PUR requirements on IoT NTN can be reused for RedCap NTN. However, given that the discussion in terms of TA validation condition for transmission using PUR is still FFS in IoT NTN topic, SDT requirements for RedCap NTN can be defined after further conclusion on the following FFS for the validation conditions for PUR for IoT NTN.

#### TS36.133 clause 4.7A.4 Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access #### 4.7A.4.3 Requirements on TA validation for transmission using PUR The UE is allowed to transmit using PUR provided that TA is valid. [ FFS for the validation conditions] The UE shall determine the uplink timing, TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

Proposal 2: For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable)

On the Minimization of Drive Tests (MDT), currently we already have the corresponding requirements defined for NTN scenario. From the deployment point of view, the data logged for MDT in NTN and (e)RedCap NTN may not be quite different. The necessity of further supporting MDT capability for Redcap UE over NTN is unconvinced.

Proposal 3: RAN4 not to define MDT requirements in RRC_IDLE and RRC_INACTIVE state

    - Features to be considered for (e)RedCap UE with FR1-NTN

#### Issue 2-3-1: RedCap R17 relaxed measurement Online agreement <Agreement>: - FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline. - If defined, the requirements apply provided that target cell is GSO

Regarding the relaxation on cell reselection measurement, it was discussed in both NTN and Red-cap work items. The scenario on R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE is also covered by Redcap. We believe it is straightforward to support them in (e)RedCap NTN.

For the Rel-17 RedCap relaxed measurement requirements for cell reselection, the criteria for stationary scenarios were newly introduced. The main difference between the stationary criterion and low mobility criterion is that the threshold for triggering relaxed measurement may be configured differently. One issue raised by some companies in the last meeting is that, due to the near-far effect in NTN, the variation of RSRP may not be obvious when UE is in the center of the cell and at the edge of cell. It caused that these two scenarios are quite similar even though the thresholds are configured differently.

In our view, considering the relaxation with low mobility has been introduced for R17 NR NTN, it means the criterion based on RSRP measurement still may be workable in some cases for GSO deployment. It is no harm to further distinguish stationary and low mobility scenarios to perform respective relaxations on measurement. Besides, for the sake of the flexibility on separate configurations for UE with stationary and low mobility, we still see the benefit of introducing the Rel-17 RedCap relaxed measurement requirements for (e)RedCap NTN.

Proposal 4: RAN4 to define both R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) and Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding relaxed requirements defined for RedCap and NTN UE can be the baseline.

#### Issue 2-3-2: eDRX configuration Online agreement <Agreement>: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO.

For earth-moving LEO deployment, considering the UE is not required to meet the requirements for 2.56s DRX cycle length in NTN. We prefer to add the corresponding applicability rules that eDRX cycle is not applicable for earth-moving LEO deployment. And for earth-fixed LEO deployment, RAN4 can further discuss the value of eDRX cycle to be supported. The maximum value of the eDRX cycle set as 10.24s can be the starting point.

Proposal 5: For eDRX configuration, RAN4 to agree that

- for earth-moving LEO deployment, requirements are not applicable for eDRX cycle.

- for earth-fixed LEO deployment, the maximum value of eDRX cycle to be supported is up to 10.24s

#### Issue 2-3-3: Multiple SMTCs <Way Forward>: - Proposal 1 (HW): RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands. - Option 1: make it optional for RedCap UE to support 2 SMTC per MO. - Option 2: change the requirements so that measurement for different SMTCs are not parallel. - Proposal 2 (ZTE): Multiple SMTCs shall be reused for RedCap UEs in NTN scenario. - Proposal 2a (Ericsson): RedCap for NTN supports (at the least) 2 SMTC. - Proposal 3 (OPPO): RAN4 to not consider the support of multiple SMTCs for (e)RedCap UE with FR1-NTN bands. - Proposal 4 (QC): NTN RRM requirements defined for simultaneous measurements on multiple satellites are not applicable for RedCap NTN UE.

In Rel-17 NTN, it was introduced multiple SMTCs for satellite measurement, which is to support at least 2 SMTCs as mandatory and optionally support a maximum of 4 SMTCs. While for RedCap, it mainly aims to support UE complexity reduction reduction features. From this point of view, our preference is not to support the maximum number of SMTCs as 4 but limit it by supporting 2SMTCs as an optional capability for (e)Redcap NTN capable UE.

Proposal 6: For supporting of multiple SMTCs, RAN4 to make it optional to support 2 SMTCs per MO for (e)Redcap UE with FR1-NTN bands.

#### Issue 2-3-6: FH in PRS measurement for NW verified location requirements <Way Forward>: - Proposal 1 (HW): RAN4 to discuss whether to support FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN bands. - Proposal 2 (Ericsson, OPPO): Not support FH in PRS measurement for RedCap for NTN.

For (e)RedCap NTN, it was agreed that PRS-RSRPP measurement would not be defined and only NW verified location is supported. The necessity of introducing FH in PRS measurement for high accuracy is not evident.

Proposal 7: Not support FH in PRS measurement for RedCap for NTN

  - Reduction in the number of UE Rx branches

#### Issue 3-2-2: NR HO requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN - Proposals - For NR RACH-based HO requirements, - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs. - For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs. - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused. - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO - Further discuss the requirements for RACH-less HO, Time/location-based CHO with/without L3 measurement and Satellite switching with re-sync.

For RACH-based HO for 2Rx (e)RedCap NTN UEs, it is straightforward to use the legacy RACH-based HO in FR1-NTN requirements. And for 1Rx (e)RedCap NTN UEs, the corresponding requirements for normal HO have never been considered in NTN scenario, the same relaxation for 1 Rx RedCap UE in TN network can be reused.

Proposal 8: For RACH-based HO requirements,

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

    - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO

    - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO

For RACH-less HO, currently the Tsearch defined in NTN is the same as RACH-based HO. Naturally, the same relaxation by extending cell search delay in normal HO for 1Rx (e)RedCap UEs can be applied.

Proposal 9: For RACH-less HO requirements,

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-less based HO requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-less based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs

  - The definition of Tsearch is the same as the one defined for RACH-based HO For 1Rx (e)RedCap UEs with FR1-NTN bands

For Satellite switching with re-sync, the requirement was generally inherited from RACH-based HO. The main difference in terms of Tsearch is that the time is based on SSB periodicity of the source satellite and ssb-TimeOffset configured by NW. Further, since the procedure of satellite switching is limited for unchanged PCI case, only the intra-frequency measurement is considered. For supporting 1 Rx antenna, we can consider one more SSB periodicity on the top of Tfirst_SSB. This rule can be applied for both hard satellite switch with re-sync and soft satellite switch with re-sync. To be specific, the relaxation on Tsearch for 1 Rx antenna is defined as follows:

- Relax by TSSB: Tsearch = Tfirst_SSB + TSSB ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite

TS38.133 clause 6.1C.3.2.2 Interruption time for hard satellite switch with re-sync the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + Tprocessing + T∆ + Tmargin ms Otherwise, no interruption time requirement is applied. Where: - Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when UE starts synchronizing with target satellite. If the target cell Es/Iot ≥ -2 dB, then Tsearch = [Tfirst_SSB] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE.

Proposal 10: For Satellite switching with re-sync requirements (including both hard satellite switch with re-sync and soft satellite switch with re-sync),

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy Satellite switching with re-sync requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB:

  - Tsearch = Tfirst_SSB + TSSB ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite

For Time/location-based CHO with L3 measurement, currently there is no requirement on CHO defined for (e)RedCap UE. To support it for (e)RedCap NTN UE, the legacy CHO delay requirements for FR1 NTN can be the baseline for defining the corresponding requirements for 2 Rx and 1 Rx antenna. The CHO delay for FR1 NTN is shown as follows:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Generally, the definition and corresponding value of each delay component can be reused except for Tmeasure. For RedCap incapable UE, the measurement time delay for identifying a new detectable intra/inter-frequency cell follows the identification delay as Tidentify_intra_with_index /Tidentify_intra_without_index (for intra-frequency) or Tidentify_inter_with_index /Tidentify_inter_without_index (for inter-frequency). While for (e)RedCap UE, it owns the specific identification delay defined in clause 9.2B and 9.3B. The part Tmeasure shall be adapted for (e)RedCap NTN by replacing the identification delay for NTN with the identification delay for RedCap. To be specific, the adaptations to the measurement delay for CHO include:

- For intra-frequency measurement, replace Tidentify_intra_with_index/Tidentify_intra_without_index with Tidentify_intra_with_index_RedCap /Tidentify_intra_without_index_RedCap

- For inter-frequency measurement, replace Tidentify_inter_with_index /Tidentify_inter_without_index with Tidentify_inter_with_index_RedCap /Tidentify_inter_without_index_RedCap

Where for the 2 Rx and 1 Rx UE, the Tidentify_intra_with_index_RedCap /Tidentify_intra_without_index_RedCap and Tidentify_inter_with_index_RedCap /Tidentify_inter_without_index_RedCap correspond to its own time period defined in clause 9.2B and 9.3B.

Proposal 11: For Time/location-based CHO with L3 measurement, RAN4 to reuse legacy Time/location-based CHO with L3 measurement requirements for normal UEs with the following adaptation:

- The part Tmeasure shall be adapted for (e)RedCap NTN by replacing the identification delay for NTN with the identification delay for RedCap,

- For intra-frequency measurement, replace Tidentify_intra_with_index/Tidentify_intra_without_index with Tidentify_intra_with_index_RedCap /Tidentify_intra_without_index_RedCap

- For inter-frequency measurement, replace Tidentify_inter_with_index /Tidentify_inter_without_index with Tidentify_inter_with_index_RedCap /Tidentify_inter_without_index_RedCap

- Where for the 2 Rx and 1 Rx UE, the Tidentify_intra_with_index_RedCap /Tidentify_intra_without_index_RedCap and Tidentify_inter_with_index_RedCap /Tidentify_inter_without_index_RedCap correspond to its own time period defined in clause 9.2B and 9.3B.

And for Time/location-based CHO without L3 measurement, considering for this scenario CHO will be triggered only based on time/location criteria and no further measurement needed, the delay requirements are common for both 2Rx and 1Rx (e)RedCap UEs with FR1-NTN bands. The legacy Time/location-based CHO without L3 measurement defined for NTN can be reused.

Proposal 12: For Time/location-based CHO without L3 measurement,

- RAN4 to reuse the legacy FR1-NTN Time/location-based CHO without L3 measurement requirements for normal UEs.

- The requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

Issue 3-3-1: Timing requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN - The existing timing requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands. - UE transmit timing - UE timer accuracy - Timing Advance - The timing requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

The current timing requirements defined for NTN are including:

- UE transmit timing

- UE timer accuracy

- Timing advance

For UE timer accuracy, the corresponding requirements defined in NTN and Red-cap are the same. For UE transmit timing requirements and timing advance requirements, in NTN, the NTN-specific factors are additionally taken into account. As a Red-cap NTN UE, the same principles should be followed. To sum up, the timing requirements for NTN can be reused to define the corresponding requirements for Red-cap NTN.

Proposal 13: For timing requirement for Red-cap NTN, RAN4 to reuse the existing timing requirements for NTN

#### Issue 3-4-2: Other requirements in signaling characteristics for 1Rx/2Rx (e)RedCap UEs with FR1-NTN - The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands: - Active BWP switch delay, - active TCI state switch delay - UE specific CBW change requirements - Pathloss reference signal switching delay - The above requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

For the requirements on Radio Link Monitoring, for 2Rx, the existing requirements for NTN can be reused. And for 1Rx the current parameters and requirements for 1Rx defined in Red-cap can be the reference and make necessary modifications to adapt NTN scenario.

For the Link Recovery Procedures, a similar analysis in RLM can be referred.

For Active BWP switch delay, the main difference between NTN and Red-cap is that the NR slot length 0.125ms is additionally considered in Red-cap. For Red-cap NTN, we considered the minimum applicable sets of these two features. Thus, the current requirements on Active BWP switch delay for NTN can be reused.

For Pathloss reference signal switching delay, it was defined for NTN and the corresponding requirements can be also applied for Red-cap NTN.

Proposal 14: The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands:

- Active BWP switch delay,

- active TCI state switch delay

- UE specific CBW change requirements

- Pathloss reference signal switching delay

Issue 3-5-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused. - For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred, and the same relaxation for accuracy should be reused.

For Network verified UE location, the corresponding requirements for NTN were discussed in Rel-18. Basically, the measurement period and measurement accuracy requirements can be reused. Specifically, the requirement to be newly introduced is UE Rx-Tx accuracy requirement for 1Rx. For this, a similar discussion occurred in Red-cap positioning. The corresponding accuracy can be referred.

Proposal 15: For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused. And for 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred to, and the same relaxation for accuracy should be reused

- Summary

Proposal 1: RAN4 not to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Proposal 2: For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable)

Proposal 3: RAN4 not to define MDT requirements in RRC_IDLE and RRC_INACTIVE state

Proposal 4: RAN4 to define both R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) and Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding relaxed requirements defined for RedCap and NTN UE can be the baseline.

Proposal 5: For eDRX configuration, RAN4 to agree that

- for earth-moving LEO deployment, requirements are not applicable for eDRX cycle.

- for earth-fixed LEO deployment, the maximum value of eDRX cycle to be supported is up to 10.24s

Proposal 6: For supporting of multiple SMTCs, RAN4 to make it optional to support 2 SMTCs per MO for (e)Redcap UE with FR1-NTN bands.

Proposal 7: Not support FH in PRS measurement for RedCap for NTN

Proposal 8: For RACH-based HO requirements,

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

    - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO

    - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO

Proposal 9: For RACH-less HO requirements,

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-less based HO requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-less based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs

  - The definition of Tsearch is the same as the one defined for RACH-based HO For 1Rx (e)RedCap UEs with FR1-NTN bands

Proposal 10: For Satellite switching with re-sync requirements (including both hard satellite switch with re-sync and soft satellite switch with re-sync),

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy Satellite switching with re-sync requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB:

  - Tsearch = Tfirst_SSB + TSSB ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite

Proposal 11: For Time/location-based CHO with L3 measurement, RAN4 to reuse legacy Time/location-based CHO with L3 measurement requirements for normal UEs with the following adaptation:

- The part Tmeasure shall be adapted for (e)RedCap NTN by replacing the identification delay for NTN with the identification delay for RedCap,

- For intra-frequency measurement, replace Tidentify_intra_with_index/Tidentify_intra_without_index with Tidentify_intra_with_index_RedCap /Tidentify_intra_without_index_RedCap

- For inter-frequency measurement, replace Tidentify_inter_with_index /Tidentify_inter_without_index with Tidentify_inter_with_index_RedCap /Tidentify_inter_without_index_RedCap

- Where for the 2 Rx and 1 Rx UE, the Tidentify_intra_with_index_RedCap /Tidentify_intra_without_index_RedCap and Tidentify_inter_with_index_RedCap /Tidentify_inter_without_index_RedCap correspond to its own time period defined in clause 9.2B and 9.3B.

Proposal 12: For Time/location-based CHO without L3 measurement,

- RAN4 to reuse the legacy FR1-NTN Time/location-based CHO without L3 measurement requirements for normal UEs.

- The requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

Proposal 13: For timing requirement for Red-cap NTN, RAN4 to reuse the existing timing requirements for NTN

Proposal 14: The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands:

- Active BWP switch delay,

- active TCI state switch delay

- UE specific CBW change requirements

- Pathloss reference signal switching delay

Proposal 15: For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused. And for 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred to, and the same relaxation for accuracy should be reused

- References

- R4-2416865, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT