3GPP TSG-RAN WG4 Meeting # 113	R4- 24xxxxx

Orlando, US, 18th – 22nd November, 2024

Source: 	China Telecom

Title: 	Views on Power domain enhancements for single carrier

Agenda Item:	7.1.1.2.1

Document for:	Discussion

Introduction

In RAN4#112-bis, we saw the divergence among companies about the scenarios and approaches for MPR reduction but still had some progress about outer-inner converting. The focus of discussion concentrated on scenarios application and IBE/OOBE/SE region.

So in this contribution, we bring the operator's perspective and request of UL power boosting from network.

Discussion

General clarification

Our spectrum for NR include band n1 and n78, which is co-constructed and co-shared in China, and thus no coexistence issue for these two bands. Considering enough gap at the left of band n1 and n78 so it is typically belonging to scenario1. Those gap space could be utilized for OOBE relaxation to reduce MPR by shifting ACLR/SEM integrating range which is what we've been discussing in the previous meeting.

So to achieve better uplink coverage performance, we support to convert outer RB allocation to inner RB allocation by shifting OOBE to the unused gap band to reduce MPR.

Proposal1: Support scenario 1 that shifts the OOBE integral range to the unused band for reducing MPR.

Potential solutions discussion

- IBE/OOBE/SE boundary and range

The main conflict in this part can be captured as below in WF[1].

- Issue 1-2-3: Where to use IBE in the larger BS CBW - Issue 1-2-4: Boundary to apply ACLR and SEM - Issue 1-2-5: Boundary to apply SE - Issue 1-2-6: Which CBW is utilized as the basis for the OOBE requirement and the applicable boundary

To relax the requirement and reduce MPR, the principle of determining those values should be use actual UE CBW for integration and extended UE CBW for application. Since introduced the bandwidth extension, we must utilize the shifting to calculate ACLR/SEM/SE with lower PSD. As a consequence, the OOBE, including ACLR/SEM, should be integrated with UE CBW and applied from each edge of extended UE CBW. And the application range of SE should base on extended UE CBW.

Proposal2: The OOBE, including ACLR/SEM, should be integrated with UE CBW and applied from each edge of extended UE CBW.

Proposal3: The application range of SE should base on extended UE CBW.

For the IBE, there are indeed some cases the extended UE CBW including the guard band, and the UE CBW can still be extended in case of full RB allocation, which all hinge on the scenarios and NW configuration. The purpose of bandwidth extension is relaxing OOB requirement for MPR reduction but we should use IBE for the shifting range in exchange to make sure the emission compliance.

As a consequence:

- IBE should be used in the guard band of UE CBW if it’s included in extended UE CBW;

- And for full RB allocation the extended range depends on actual spectrum allocation and NW indication, but if the extended UE CBW exists, the IBE should be applied.

Proposal4: IBE should be used in the guard band of UE CBW if it’s included in extended UE CBW.

Proposal5: For full RB allocation, if the extended UE CBW exists, the IBE should be applied.

- extended CBW ratiox

The main conflict about ratio can be captured as below in WF[1].

Issue 1-2-7: Ratio of extended CBW to UE CBW and to larger BS channel BW - WF - To discuss the following aspects for the extended UE CBW: - Whether the extended CBW with new boundary of FOOB should comply with the recommendation by ITU-R on necessary bandwidth in terms of SE for the original UE BW - Whether default or fixed extension, e.g. 1/2 UE CBW could be stipulated in the spec or the extension ratio could be a UE capability to fulfil the conversion of outer to inner - Whether the extended UE CBW could belong to the regular defined channel bandwidth - Whether some premise should be established for the extension size, e.g. - minimum extended CBW compared to original UE CBW - minimum BS CBW compared to UE CBW - frequency separation of the RB allocation to the BS edge

Since the scenarios are different among regions and operators, the gap band for extension is diverse too, e.g. our band n1 can only extend 15MHz on the left while n78 can extend enough space for maximum UE CBW (50MHz shifting for 100MHz CBW). And there are also scenarios that could do extension on each side. So obviously, we should make sure the ratio of extended CBW is flexible and configurable and not preclude any other premise.

Proposal6: Make sure the ratio of extended CBW is flexible and configurable and not preclude any other premise.

Last but not the least, UE CBW extension is just used for outer-inner converting to relax MPR requirement but cannot transmit any information. The extended UE CBW is not the regular defined channel bandwidth.

Proposal7: The extended UE CBW is not the regular defined channel bandwidth.

- Approaches clarification

In pursuit of a simpler and more acceptable way, we should only change the definition of inner-outer RB allocation rather than the MPR table in TS 38.101-1. And the change should satisfy scenarios distinguish, different spectrum allocation status, RedCap or non-RedCap UE, and symmetric or asymmetric CBW extension as summarized in WF[1]. So the effective method is introducing configurable parameters in RB allocation determination.

Proposal8: Introduce configurable parameters in RB allocation determination.

For signaling aspects, the BS should configure the parameters (i.e. the shifting on one or both side) to UE from NW perspective. And the parameters should distinguish scenarios and single or multi-CC.

Proposal9: The BS should configure the shifting on one or both side to UE from NW perspective, and the parameters should distinguish scenarios and single or multi-CC.

Conclusion

Proposal1: Support scenario 1 that shifts the OOBE integral range to the unused band for reducing MPR.

Proposal2: The OOBE, including ACLR/SEM, should be integrated with UE CBW and applied from each edge of extended UE CBW.

Proposal3: The application range of SE should base on extended UE CBW.

Proposal4: IBE should be used in the guard band of UE CBW if it’s included in extended UE CBW.

Proposal5: For full RB allocation, if the extended UE CBW exists, the IBE should be applied.

Proposal6: Make sure the ratio of extended CBW is flexible and configurable and not preclude any other premise.

Proposal7: The extended UE CBW is not the regular defined channel bandwidth.

Proposal8: Introduce configurable parameters in RB allocation determination.

Proposal9: The BS should configure the shifting on one or both side to UE from NW perspective, and the parameters should distinguish scenarios and single or multi-CC.

References

- R4-2417204, WF on power domain enhancement, RAN4#112-bis, Huawei, HiSilicon