3GPP TSG RAN WG4 Meeting #113	R4-2419116

Orlando, FL, USA 18 Nov. 2024 – 22 Nov. 2024

Agenda Item:	7.10.4

Source: 	Ericsson

Title:	Discussion on Rel-19 ATG RRM Enhancement Issues

Document for:	Discussion

# Introduction

In the last two meetings, we discussed and agreed on several general issues regarding the enhancement of the ATG network. The workflows for RAN4#112-bis [1] and RAN4#112 [2] have been approved. In this contribution, we further discuss the impact on RRM and potential enhancements for ATG CA.

# Discussion

In this section, we discuss the important open issues for this meeting.

## General Issues

## UE antenna type for inter-band CA

In RAN4#112 meeting all the below UE antenna type for inter-band CA is agreed. However, it is agreed that case 4 is kept in bracket with the note that if some of the cases will be precluded in RF discussion, they will also be removed for RRM requirement discussion. In the RAN4#112-bis meeting, it is agreed to check whether case 4 is excluded or not explicitly discussed in the RF session. To our understanding, case 4 was not explicitly discussed in the RF session.

Issue 1-1-2: UE antenna type for inter-band CA Agreement: - Refine the agreement from last meeting as: - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed. Further check which understanding is correct: - Understanding #1: Case 4 is excluded in RF session. - Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

Case 4 was not explicitly discussed in the RF session.

## Signalling for UE Antenna Type

In the WF [1], 5 different UE types are agreed. For ATG CA, the UE must measure and report the results to the network. Compared to the legacy scenario without ATG operation, with Rel-19 ATG UE supporting different types of antennas, UE can perform measurements on different serving cells using different types of antennas. However, the network has no prior knowledge about what type of antennas will be used by the UE for measurement.  As the measurement delays depend on the type of antennas used, the serving network needs more information regarding how much time it will take to complete a measurement and when to expect measurement reporting. Therefore, the network needs to know which type of antenna panel combinations are used to perform measurements when the network configures the measurements. For example, based on this information the NW can plan its operation.

If ATG UE can perform measurements on different serving cells using different types of antennas, it is necessary to provide assistance information to the network regarding which antenna panel combination is used on the UE side.

## UL TX Switching

To improve UL capacity, 3GPP introduced UL Tx switching capability. As the ATG enhancement WID [3] is specifically limited to co-located ATG DL CA to improve the ATG DL capacity performance, we think UL carrier switching is out of scope for Rel-19 WI. Hence, there is no need to introduce UL carrier switching for ATG in Rel-19.

UL carrier switching is out of scope for Rel-19 WI; there is no need to introduce UL carrier switching for ATG in Rel-19.

## Maximum Receive Time Difference (MRTD)

In the RAN4#112-bis meeting, we presented the receive time difference (RTD) outlined in clause 2.3.1 of R4-2416009. The paper indicated that the RTD could be as low as 3.545 µs for inter-band co-located CA. However, after further study, we realized that the time taken for signals to travel from the base station to the receiver across different paths or frequencies may vary depending on the allocated channel bands. Therefore, setting an MRTD value of 3 µs for ATG UE is not feasible.

MRTD value of 3 µs for ATG UE is not feasible.

Additionally, there is currently no new UE type defined for ATG. The NR UE FR1 MRTD requirement is 33 µs, which encompasses TAE, dispersion, and propagation delay differences. We believe that this 33 µs MRTD can be reused for ATG UE, as it accounts for all relevant RTD scenarios.

RAN4 should reuse the legacy 33µs requirement to define the MRTD requirement for ATG inter-band CA.

## Signalling Characteristics

## SCell Activation Delay

From RAN4#111, Fukuoka meeting agreements, we can see that the RAN4 RF agreed there are no limitations on antenna types for ATG CA. The text from the meeting minutes is copied below:

Sub-topic 3-1 antenna type for inter-band CA Issue 3-1: clarify the antenna type for each band for inter-band CA Agreement: - No limitation on antenna types for ATG CA - FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39 - FFS on whether new capability is needed for ATG CA

In accordance with the agreement, we agreed for inter-band CA to consider the following cases:

1. ATG UE with omnidirectional antennas on both PCell and SCell.

2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell.

3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell.

[4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.]

5. ATG UE with an antenna array on both PCell and SCell with separate antenna panels.

To better understand the impact of different antenna types on the SCell activation requirement, let's examine the SCell activation time requirements.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. - TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms. If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms, otherwise. - Otherwise - TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise. otherwise, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell: - the SCell is contiguous to an active serving cell in the same band, and - A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell - its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB; If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band. For a UE supporting scellWithoutSSB-InterBandCA-r18if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The [EPRE] difference at the UE is smaller than or equal to [12] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell. where the reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured. Editor notes: FFS whether and how to capture the wording “after the compensation for AGC”. Tactivation_time is - Tfirst_TRS + TTRS +5 ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17 - Tfirst_ATRS + Tgap + TATRS + 5 ms if aperiodic CSI-RS resources are configured for Scell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17

As marked in the boxed text, the activation delay of SCell may vary depending on the combination of different antenna types in PCell and SCell. Hence, SCell activation should be further investigated for ATG CA in Rel-19.

SCell activation requirements shall be further investigated for potential impact, considering the UE antenna types.

## SCell Addition/Activation mechanism

During the RAN4#112-bis meeting, a company suggested exploring distance-triggered SCell addition and activation. However, section 38.133 already has multiple mechanisms for SCell activation and addition. Therefore, we do not see a solid reason to introduce another mechanism. Furthermore, introducing distance triggered SCell addition mechanism may also require some work from RAN2 but this is WI is limited to RAN4 only.Hence,

RAN4 should not consider distance triggered SCell addition and/or activation; existing activation methods are sufficient.

## Interruption Requirements

As we agreed to consider the following cases for inter-band CA:

1. ATG UE with omnidirectional antennas on both PCell and SCell.

2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell.

3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell.

[4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.]

5. ATG UE with an antenna array on both PCell and SCell with separate antenna panels.

We need to study the impact of this UE antenna pattern on the interruption requirements. We may need to update the following two tables of clause 8.2.2.2.3.

Table 8.2.2.2.2-1: Interruption length X2 for SCell activation/deactivation for inter-band CA NR Slot length (ms) of victim cell; Interruption length X2 (slots). 0; 1; 1. 1; 0.5; 1. 2; 0.25; Both aggressor cell and victim cell are on FR2; 2. Either aggressor cell or victim cell is on FR1; 3. 3; 0.125; Aggressor cell is on FR2; 4. Aggressor cell is on FR1; 5. 5; 0.03125; Aggressor cell is on FR1; 17. 6; 0.015625; Aggressor cell is on FR1; 33. Table 8.2.2.2.2-2: Interruption duration for SCell activation/deactivation for intra-band CA NR Slot length (ms); Interruption length (slots). 0; 1; 1 + TSMTC_duration * Nslotsubframe,μ. 1; 0.5; 1 + TSMTC_duration * Nslotsubframe,μ. 2; 0.25; 2 + TSMTC_duration * Nslotsubframe,μ. 3; 0.125; 4 + TSMTC_duration * Nslotsubframe,μ. 5; 0.03125; 16+ TSMTC_duration * Nslotsubframe,μ. 6; 0.015625; 32+ TSMTC_duration * Nslotsubframe,μ. NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms; - the longest SMTC duration among all active servingNOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]..

Interruption requirements shall be further investigated for potential impact, considering the UE antenna types.

## Active BWP switch delay on multiple CCs

The legacy Active BWP switch delay requirements are specified in clause 8.6 of TS 38.133. clause 8.6D defined the active BWP sweitcvh delay for signle CC.As per our understanding, legacy NR requirements can be reused for ATG DL CA. Hence, RAN4 should reuse the legacy requirements relevant to support ATG DL CA. For RRC based BWP switch delay on multiple CCs , as NR-DC is not in the scope, only simultaneous BWP switch delay scenario need to be considered. In all other cases, both simultaneous and non-simultaneous cases should be considered.

For Active BWP switch delay on multiple CCs, introduce following requirements, which can reuse the legacy:

- Simultaneous and non-simultaneous DCI based BWP switch delay on multiple CCs

- Simultaneous RRC based BWP switch delay on multiple CCs

- Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

## Measurement Procedure

    - Scheduling Availability/Restrictions

Rel-18 specifies different scheduling availabilities for ATG intra/inter-frequency measurement with or without gap for PCell. Clause 9.2D.5.3, 9.3D.9.3 However, as no limitations on antenna types for ATG CA were agreed upon in the RAN4#111 meeting, we need to study the possible impact of different antenna combinations on scheduling availability for ATG CA.

The scheduling availability extension to intra-band/inter-band CA, RAN4 to further study the impact of UE antenna patterns on scheduling availability ATG CA.

- Furthermore, we propose the following restrictions considering the UE antenna type.

For Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement following restrictions should be considered for inta-band carrier aggregation:

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

For Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement following restrictions should be considered for inter-band carrier aggregation:

- For ATG UE with omnidirectional antennas on the bands of PCC and SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

- For ATG UE with the antenna array on PCell and one or more omni-directional antenna on SCell, there are no scheduling restrictions on FR1 SCel configured in other bands than the bands in which PCell is configured.

- For ATG UE with the omni-directional antenna on PCC and antenna array on SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

- For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

- For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

RAN4 should discuss and further study the following Scheduling restriction options:

- Option1: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement.

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- Option 2: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement. When inter-band carrier aggregation in FR1 is configured,

  - For ATG UE with omnidirectional antennas on the bands of PCC and SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

  - For ATG UE with the antenna array on PCell and omni-directional antenna on SCell, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

  - For ATG UE with the omni-directional antenna on PCC and antenna array on SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

  - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

  - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

Similarly, for the scheduling availability of UE performing measurements on FR1 TDD band, the following restrictions should be considered for intra-band carrier aggregation:

- When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

For the scheduling availability of UE performing measurements on FR1 TDD band, the following restrictions should be considered for inter-band carrier aggregation:

- When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

Hence, The restrictions defined for Rel-18 legacy NR CA on FR1 TDD band ban be reused for ATG CA.

RAN4 should reuse the legacy scheduling restriction for FR1 TDD band for ATG CA.

## Beam Sweeping Factor and L3/L1 measurement sharing factor

For intra-frequency measurement, N1 and Klayer1_measurement are specified for ATG in clauses 9.2D.5 and 9.2D.6 of TS 38.133. We can reuse the defined requirements for ATG as they cover both antenna types.

Ran4 should reuse the Beam Sweeping Factor N1 and L3/L1 measurement sharing factor Klayer1_measurement  defined for ATG UE.

## Sharing factor between L1 measurement and RRM measurement

In the RAN4#112-bis meeting, some companies proposed to consider relaxing of the sharing factor Klayer1_measurement and Psharing factor between L1 measurement and RRM measurement. However, to the best of our understanding, there is no feasible impact to address that is specific to ATG UE CA case. Hence, we propose to use the legacy sharing factor Klayer1_measurement and Psharing factor between L1 measurement and RRM measurement.

RAN4 to reuse the legacy sharing factor Klayer1_measurement and Psharing factor between L1 and RRM measurements.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for ATG UE DL CA and made the following observations and proposals.

Observation 1	Case 4 was not explicitly discussed in the RF session.

Observation 2	MRTD value of 3 µs for ATG UE is not feasible.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	If ATG UE can perform measurements on different serving cells using different types of antennas, it is necessary to provide assistance information to the network regarding which antenna panel combination is used on the UE side.

Proposal 2	UL carrier switching is out of scope for Rel-19 WI; there is no need to introduce UL carrier switching for ATG in Rel-19.

Proposal 3	RAN4 should reuse the legacy 33µs requirement to define the MRTD requirement for ATG inter-band CA.

Proposal 4	SCell activation requirements shall be further investigated for potential impact, considering the UE antenna types.

Proposal 5	RAN4 should not consider distance triggered SCell addition and/or activation; existing activation methods are sufficient.

Proposal 6	Interruption requirements shall be further investigated for potential impact, considering the UE antenna types.

Proposal 7	For Active BWP switch delay on multiple CCs, introduce following requirements, which can reuse the legacy:

 Simultaneous and non-simultaneous DCI based BWP switch delay on multiple CCs

 Simultaneous RRC based BWP switch delay on multiple CCs

 Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

Proposal 8	The scheduling availability extension to intra-band/inter-band CA, RAN4 to further study the impact of UE antenna patterns on scheduling availability ATG CA.

Furthermore, we propose the following restrictions considering the UE antenna type.

Proposal 9	RAN4 should discuss and further study the following Scheduling restriction options:

Option1: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement.

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

Option 2: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement. When inter-band carrier aggregation in FR1 is configured,

o For ATG UE with omnidirectional antennas on the bands of PCC and SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

o For ATG UE with the antenna array on PCell and omni-directional antenna on SCell, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

o For ATG UE with the omni-directional antenna on PCC and antenna array on SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

o For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overla with the aforementioned restricted symbols.

o For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

Proposal 10	RAN4 should reuse the legacy scheduling restriction for FR1 TDD band for ATG CA.

Proposal 11	Ran4 should reuse the Beam Sweeping Factor N1 and L3/L1 measurement sharing factor Klayer1_measurement  defined for ATG UE.

Proposal 12	RAN4 to reuse the legacy sharing factor Klayer1_measurement and Psharing factor between L1 and RRM measurements.

# References

- R4-2416852, “WF on RRM requirements for [112bis][208] NR_ATG_enh,” CMCC

- R4-2413959, “WF on RRM requirements for NR_ATG_enh”, CMCC

- RP- 240839, “New WID on Enhancements for Air-to-ground network for NR,” Apple

- 3GPP TS 38.133: "NR; Requirements for support of radio resource management"

- R4-2416009, “Further Discussion on Rel-19 ATG CA RRM requirements, Ericsson”