3GPP TSG-RAN WG4 Meeting #113	R4-2419140

Orlando, US, 18 – 22 November, 2024

Title: 	Discussion on performance requirements for CPP

Source: 	Huawei, HiSilicon

Agenda item:	5.22.2.6

Document for:	Discussion

Introduction

RRM performance requirements for CPP are discussed in RAN4#112-bis, and the outcomes are captured in [1]. Based on the WF, RAN4 need to further discuss the margin due to frequency error.

In this paper, we will provide our views on remaining issues in performance requirements for CPP.

Discussion

The impact of frequency error on the accuracy performance of CP measurement has been discussed for many meetings. Based on discussions in RAN4#112-bis, we understand there are two different issues discussed by different companies.

Issue 1: frequency error at TRP transmission

Similar to UE, TRP is also allowed a frequency error in its transmission compared to the assigned frequency as defined in clause 6.5.1 of 38.104. As different TRPs has different frequency error, the impact on the phase calculation cannot cancel out at UE side when UE derives the RSCPD or relative RSCP.

However, RAN4 is defining UE requirements, and it should be based on ideal TRP transmission assumption because UE is not expected to compensate the imperfection at TRP side. In this case, the impact of frequency error at TRP transmission will be handled by LMF with double differential method. We do not see a need to add a margin to the UE measurement accuracy requirements for TRP imperfection.

In the test, we are also not going to model the frequency error at TRP transmission. Of course, if the TE cannot guarantee zero frequency error, then some test tolerance can be added, but this is RAN5 scope.

Proposal 1: RAN4 not to define margin to CPP accuracy requirements for frequency error at TRP transmission.

Issue 2: frequency error at UE reception

UE is allowed a frequency error in its reception compared to the nominal frequency where it is configured to receive PRS. This frequency error is common for all the TRPs, so we considered that it can cancel out when UE derives the RSCPD or relative RSCP. However, after some further checking, we understand the same frequency error may have different impact to the phase calculation for different TRPs because there are some non-linear operations in the phase calculation when combining all REs across all symbols, e.g. arc tan.

To address the impact of frequency error, a margin is needed which depends on the symbol duration of PRS resource, i.e. the longer duration is, the larger margin is required. Considering the margin needs to be reasonably small, we suggest to clarify that CP accuracy requirements do not apply for comb size 12. This is because with comb size 12, the symbols duration would be 12 symbols, and the margin would be quite large which makes CP measurement meaningless. For the other comb sizes, we suggest to add a margin of [5] degree to the simulated BB accuracy. It is noted that we do not see resource with repetition as a problem as UE does not necessarily need to combine all the symbols across all the repetitions to calculate a phase.

Besides, some additional margin still needs to be added for the expected RSTD uncertainty. This is because even the two resources are in the same set of symbols after accounting for the expected RSTD, there can still be RTD at UE side due to expected RSTD uncertainty, and this RTD will cause error in RSCPD or relative RSCP with frequency error.

Based on the maximum RSTD uncertainty of 32us in FR1 and 8us in FR2, and assuming PFL center frequency is 4GHz in FR1 and 40GHz in FR2, the additional margin on the phase accuracy would be 9.2 deg for FR1 and 23 deg for FR2. It is noted that in the calculation, we assume in the worst case TRP1 is with +32us RTD compared to the reference resource, and TRP2 is with -32us RTD compared to the reference resource, so the total RTD between TRP1 and TRP2 is 2*32=64us.

Proposal 2: RAN4 to clarify that CPP measurement accuracy do not apply for comb size 12.

Proposal 3: For defining CP measurement accuracy requirements, add additional margin on top of simulation results to account for the impact of carrier frequency error.

- 14.2 deg for FR1

- 28 deg for FR2

Conclusions

In this paper we provided our views on remaining issues in performance requirements for CPP.

Proposal 1: RAN4 not to define margin to CPP accuracy requirements for frequency error at TRP transmission.

Proposal 2: RAN4 to clarify that CPP measurement accuracy do not apply for comb size 12.

Proposal 3: For defining CP measurement accuracy requirements, add additional margin on top of simulation results to account for the impact of carrier frequency error.

- 14.2 deg for FR1

- 28 deg for FR2

Reference

- R4-2416870, WF on RRM requirements for NR_pos_enh2_Part2, CATT