3GPP TSG-RAN4 Meeting #113	R4-2419155

# Orlando, US, 18th – 22nd November 2024

Source:	LG Electronics

Title:	HPUE for intra-band UL CA

Agenda item:	7.1.1.1.2

Document for:	Discussion

# Introduction

In this paper, we provide our views on MPR of HPUE for intra-band contiguous/non-contiguous UL CA.

# Discussion

In last 2 RAN4 meetings, the following WFs were approved [1, 2]. This paper discusses the requirement of HPUE for intra-band UL CA.

## [1] ## 1.1 MPR assumption - Keep the following last meeting’s agreement for intra-band non-contiguous ULCA - Define MPR/A-MPR requirements based on equal PSD assumption for intra-band contiguous ULCA - It does not preclude companies to bring evaluation for unequal PSD - MPRc = MPR, which means no change of spec ## [2] ## 1.1 MPR evaluation methodology and assumption Online agreement: - MPR and CANS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on TxD architecture and may account for PSD imbalance with up to 6dB. - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB - PC1.5 contiguous intra-band ULCA based on dualPA architecture with two LOs is not specified in R19 - MPR studies for PC1.5 non-contiguous intra-band ULCA focusses on dualPA architecture with two LOs and may account for PSD imbalance with up to 6dB. - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB - PC1.5 non-contiguous intra-band ULCA based on TxD architecture is not specified in R19

## MPR evaluation of PC1.5 UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx

For PC1.5 contiguous intra-band ULCA, RAN4 agreed to define MPR/A-MPR requirements based on equal PSD assumption for intra-band contiguous ULCA.

For PC1.5 non-contiguous intra-band ULCA, RAN4 agreed to keep the previous agreement.

- focus on dualPA architecture with 2 LOs and whether to define the requirements based on equal PSD or PSD imbalance with 6dB.

Figure 2-1 shows the PC1.5 dualPA-architecture for intra-band non-contiguous UL CA.

Figure 2-1. PC1.5 dualPA-Architecture (2PA2LO) : Equal PSD vs up to 6dB PSD imbalance

The existing MPR requirement was specified based on equal PSD, such as, MPR(A) in Figure 2-1. If up to 6dB PSD imbalance, such as, MPR(B), and MPR(C) in Figure 2-1, is meaningful in real field and to be specified as MPR, it needs to be considered with following condition.

- 6 dB ≤ { PCMAX,CC1 -  10*log10( LCRB1*SCS1)} - { PCMAX,CC2  -  10*log10( LCRB2*SCS2) }  ≤ 6 dB

It can be as follows.

{ PCMAX,CC1 -  PCMAX,CC2} – 6 ≤ 10*log10( LCRB1*SCS1LCRB2*SCS2) ≤ { PCMAX,CC1 -  PCMAX,CC2} + 6

Here, the upper bound of Pcmax,c is 26dBm per CC.

#### Proposal 1: For dualPA-Architecture of PC1.5 intra-band non-contiguous UL CA, consider following condition if up to 6dB PSD imbalance is assumed for MPR requirement.

{ PCMAX,CC1 -  PCMAX,CC2} – 6 ≤ 10*log10( LCRB1*SCS1LCRB2*SCS2) ≤ { PCMAX,CC1 -  PCMAX,CC2} + 6

The upper bound of  PCMAX,CC1 and PCMAX,CC2 is 26dBm.

To see MPR impact by equal PSD and  PSD imbalance in Figure 2-1, the following assumption in Table 2-1 and scenarios in Table 2-2 are considered for preliminary simulation.

- Table 2-1. Simulation assumption

| Bandwidth | per CC: 10MHz/20MHz aggregated CBW: 10MHz +10 MHz |
| --- | --- |
| Aggregated CBW | 10MHz + Gap10MHz + 10MHz |
| Maximum output power for aggregated CBW | 29dBm |
| Numerology | 15 kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |

Table 2-2: PC1.5 intra-band NC CA MPR evaluation scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | B | Frequency of FIM3,low_block,low & FIM3,high_block,high | PSD |
| --- | --- | --- | --- | --- | --- | --- |
| 10MHz + Gap10MHz+10MHz | 1 | 2RB41 | 8RB0 | 1.8 | SEMfreq_-13 | Equal |
|  | 2 |  |  |  |  | CC1 > CC2, 6dB |
|  | 3 |  |  |  |  | CC1 < CC2, 6dB |
|  | 4 | 10RB41 | 10RB0 | 3.6 |  | Equal |
|  | 5 |  |  |  |  | CC1 > CC2, 6dB |
|  | 6 |  |  |  |  | CC1 < CC2, 6dB |
|  | 7 | 10RB41 | 20RB0 | 5.4 |  | Equal |
|  | 8 |  |  |  |  | CC1 > CC2, 6dB |
|  | 9 |  |  |  |  | CC1 < CC2, 6dB |
|  | 10 | 10RB41 | 30RB0 | 7.2 |  | Equal |
|  | 11 |  |  |  |  | CC1 > CC2, 6dB |
|  | 12 |  |  |  |  | CC1 < CC2, 6dB |
|  | 13 | 2RB0 | 8RB43 | 1.8 | SEfreq_-30 | Equal |
|  | 14 |  |  |  |  | CC1 > CC2, 6dB |
|  | 15 |  |  |  |  | CC1 < CC2, 6dB |
|  | 16 | 10RB0 | 10RB41 | 3.6 |  | Equal |
|  | 17 |  |  |  |  | CC1 > CC2, 6dB |
|  | 18 |  |  |  |  | CC1 < CC2, 6dB |
|  | 19 | 10RB0 | 20RB31 | 5.4 |  | Equal |
|  | 20 |  |  |  |  | CC1 > CC2, 6dB |
|  | 21 |  |  |  |  | CC1 < CC2, 6dB |
|  | 22 | 10RB0 | 30RB21 | 7.2 |  | Equal |
|  | 23 |  |  |  |  | CC1 > CC2, 6dB |
|  | 24 |  |  |  |  | CC1 < CC2, 6dB |

Table 2-3 and Figure 2-2 shows the simulation results for PC1.5 intra-band NC CA with dualPA-architecture(2x26dBm).

Table 2-3: Preliminary MPR simulation result for PC1.5 intra-band NC CA with dualPA-architecture

| Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 'QPSK' | 3.2 | 2.0 | 3.8 | 1.2 | 2.8 | 2.9 | 2.7 | 2.0 | 4.8 | 4.7 | 2.6 | 5.7 |
| '16QAM' | 3.9 | 2.0 | 4.8 | 2.1 | 3.9 | 3.9 | 3.1 | 3.1 | 4.8 | 4.7 | 2.6 | 5.7 |
| '64QAM' | 5.0 | 3.1 | 5.8 | 3.1 | 5.0 | 5.0 | 4.2 | 4.2 | 5.5 | 4.7 | 3.4 | 5.7 |
| '256QAM' | 6.7 | 4.7 | 7.4 | 4.7 | 6.7 | 6.7 | 5.9 | 5.9 | 7.2 | 6.4 | 5.5 | 7.3 |
| Scenario # | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| 'QPSK' | 10.0 | 9.5 | 10.2 | 7.5 | 9.0 | 9.4 | 7.3 | 7.8 | 8.5 | 7.3 | 6.4 | 8.2 |
| '16QAM' | 9.5 | 9.5 | 10.2 | 7.5 | 9.0 | 9.4 | 7.3 | 7.7 | 8.5 | 7.3 | 6.4 | 8.2 |
| '64QAM' | 9.5 | 9.5 | 10.2 | 7.5 | 9.0 | 9.4 | 7.3 | 7.8 | 8.5 | 7.3 | 6.4 | 8.2 |
| '256QAM' | 9.5 | 9.5 | 10.2 | 7.5 | 9.0 | 9.5 | 7.3 | 7.8 | 8.5 | 7.3 | 6.4 | 8.2 |

In Figure 2-2, the difference of MPR between equal PSD and 6dB PSD imbalance is up to 2 dB for SEM of -13dBm/MHz, and up to 2dB for SE of -30dBm/MHz.

Figure 2-2: Preliminary MPR of PC1.5 intra-band NC CA with dualPA-architecture (Equal, CC1>CC2 (6dB), CC1<CC2(6dB))

#### Observation 1: Difference of MPR between equal PSD and 6dB PSD imbalance is up to 2 dB for SEM of -13dBm/MHz.

#### Observation 2: Difference of MPR between equal PSD and 6dB PSD imbalance is up to 2 dB for SE of -30dBm/MHz.

Based the observations,

#### Proposal 2: Define MPR requirement based on 6dB PSD imbalance for PC1.5 intra-band NC CA. Here, it is assumed that lower Lcrb between CCs is 6dB higher PSD.

Table 2-4 and Figure 2-3 shows the simulation results for PC2 intra-band NC CA with dualPA-architecture(2x23dBm).

Table 2-4: Preliminary MPR simulation result for PC2 intra-band NC CA with dualPA-architecture(2x23dBm)

| Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 'QPSK' | 2.4 | 1.3 | 3.6 | 0.7 | 2.7 | 2.4 | 2.5 | 1.6 | 4.6 | 4.1 | 1.8 | 5.5 |
| '16QAM' | 3.7 | 1.9 | 4.6 | 1.9 | 3.7 | 3.7 | 2.9 | 2.9 | 4.6 | 4.1 | 2.1 | 5.5 |
| '64QAM' | 4.8 | 2.9 | 5.6 | 2.9 | 4.8 | 4.8 | 4.0 | 4.0 | 5.4 | 4.5 | 3.2 | 5.5 |
| '256QAM' | 6.5 | 4.6 | 7.3 | 4.6 | 6.5 | 6.5 | 5.7 | 5.7 | 7.0 | 6.2 | 5.3 | 7.2 |
| Scenario # | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| 'QPSK' | 8.4 | 6.9 | 8.6 | 5.9 | 7.9 | 7.9 | 6.2 | 6.7 | 7.5 | 6.2 | 5.3 | 7.2 |
| '16QAM' | 8.4 | 6.9 | 8.6 | 5.9 | 7.9 | 7.9 | 6.2 | 6.7 | 7.5 | 6.3 | 5.3 | 7.2 |
| '64QAM' | 7.9 | 6.9 | 8.7 | 5.9 | 7.9 | 7.9 | 6.2 | 6.7 | 7.5 | 6.2 | 5.3 | 7.2 |
| '256QAM' | 7.9 | 6.9 | 8.7 | 5.9 | 7.9 | 7.9 | 6.2 | 6.7 | 7.5 | 6.2 | 5.3 | 7.2 |

In Figure 2-3, the difference of MPR between equal PSD and 6dB PSD imbalance is up to 2 dB for SEM of -13dBm/MHz, and up to 2dB for SE of -30dBm/MHz.

Figure 2-3: Preliminary MPR of PC2 intra-band NC CA with dualPA-architecture (2x23dBm) (Equal, CC1>CC2 (6dB), CC1<CC2(6dB))

# Use of generally known IM3 equations to extend the already existing TX CA MPR requirements for new PA combinations / power classes

For TX CA the transmitter performance and MPR have been defined in 38.101-1 for 20+20= 23dBm (PC3) and 23+23= 26dBm (PC2) and to ensure that both SEM and out-of-band performance requirements are met. The applicability of the existing requirements also for case with unequal PSD has been debated in RAN4.

We have provided our views to this in the proposals 1-4 above and see the need to further improve the coverage of the already existing requirements. However, when it comes to defining MPR for new CA PA combinations and power classes it may be possible to avoid time consuming simulations and analysis for each case by introducing a delta value to a selected reference. This delta value can be defined using the single carrier ACLR requirement as driving factor for PA and TX line-up dimensioning and linearity, in a bit similar manner as single carrier ACLR is used as calibration method in the MPR simulation.

## IM3 analysis

When two tones or carriers are transmitted with non-linear amplifier the level of spurious emissions, IM3 products, follow the well-known formula.

OIP3= Pout + 1/2*P, where P is the difference between the level two tone signal components and IM3 products.

OIP3=PTone+12∙∆P

Figure 3-1. Definition of OIP3 with equal tone or carrier power

There are also well known IM3 formulas for the case that power of the tones (or carriers) is not equal and there are shown below.

OIP3=2∙PLow+PHigh-IM3Low2     OIP3=2∙PHigh+PLow-IM3High2

Figure 3-2. Definition of OIP3 with unequal tone or carrier power

It is also known that P for two-tone signal is also a relatively good estimate for the ACLR with fully allocated (all RBs are transmitted) NR signal. Therefore, we can use the minimum ACLR performance requirement to estimate the OIP3 for the PA (or TX line up) that is required to support certain output power level with targeted linearity. Although the post PA losses are not relevant in case of relative comparison 2dB IL in RFFE between the PA and antenna is assumed here. IL higher than 2dB is often used in the analysis done for UEs supporting multiple modes and bands, but as the RFFE IL is not impacting the outcome of the relative analysis, lower IL= 2dB is used here. Block diagram of the simplified TX model that is used is shown below.

Figure 3-3. Simplified TX architecture

The 38.101-1 defines following ACLR requirement for different UE power classes:

ACLR and PA output power can be used in OIP3 equation to derive estimates for the required PA OIP3 as shown in table below.

|  | Power class 1 | Power class 2 | Power class 3 | Power class 5 |
| --- | --- | --- | --- | --- |
| Pout (dBm) | 31 | 26 | 23 | 20 |
| Post PA losses (dB) | 2 | 2 | 2 | 2 |
| PA Pout (dBm) | 33 | 28 | 25 | 22 |
| deltaP (dB) | 37 | 31 | 30 | 27 |
| PA OIP3 (dBm) | 48.5 | 40.5 | 37 | 32.5 |

The OIP3 can then be added back to same equations to estimate the level of IM3 products in case of two TX line-ups being used to generate high-output power non-contiguous ULCA signal. This simplified method does not take into account all transmitter imperfections that impact the level of emissions, but can be used as relative metric as shown below.

Figure 3-4. Examples with 23+23 and 26+26dBm PAs

Same approach can then be applied also to other cases where two transmit line-ups are used to generate UL CA signal using the formulas for both equal and un-equal power between the tones or carriers. This result indicates that there should not be a need to define MPR values individually for each non-contiguous CA power class (or PA combination), but a delta value to the already existing requirements can be used.

The results of such analysis are collected into the table below, where column MA,delta shows the additional MPR that is needed vs. the reference, which is here PC2 (23+23=26dBm) MPR values that are already defined in the 38.101-1. Total MPR would then be MA,total= MA + MA,delta.

|  | C1 | C2 | C1+C2 | MA,delta |
| --- | --- | --- | --- | --- |
| 23+23= 26dBm | 23 | 23 | 26.0 | 0 (Reference) |
| 23+26= 27.8dBm | 23 | 26 | 27.8 | 0.6 |
| 26+26= 29dBm | 26 | 26 | 29.0 | 1.0 |
| 26+31= 32.2dBm | 26 | 31 | 32.2 | 0.8 |
| 31+31= 34dBm | 31 | 31 | 34.0 | 0.5 |
| 20+20= 23dBm | 20 | 20 | 23.0 | 0.0 |
| 20+23= 24.8dBm | 20 | 23 | 24.8 | 0.0 |

#### Proposal 3: Study possibility to avoid MPR simulations and analysis for each CA output power combination through intoducing MA,delta value (MA,total= MA + MA,delta) ,which is used to offset the reference MPR requirement to new CA output power levels and PA combinations.

# Conclusion

In this contribution, we proposed our view on MPR of HPUE for intra-band UL CA. The proposals are as follows.

#### Proposal 1: For dualPA-Architecture of PC1.5 intra-band non-contiguous UL CA, consider following condition if up to 6dB PSD imbalance is assumed for MPR requirement.

{ PCMAX,CC1 -  PCMAX,CC2} – 6 ≤ 10*log10( LCRB1*SCS1LCRB2*SCS2) ≤ { PCMAX,CC1 -  PCMAX,CC2} + 6

The upper bound of  PCMAX,CC1 and PCMAX,CC2 is 26dBm.

#### Observation 1: Difference of MPR between equal PSD and 6dB PSD imbalance is up to 2 dB for SEM of -13dBm/MHz.

#### Observation 2: Difference of MPR between equal PSD and 6dB PSD imbalance is up to 2dB for SE of -30dBm/MHz.

#### Proposal 2: Define MPR requirement based on 6dB PSD imbalance for PC1.5 intra-band NC CA. Here, it is assumed that lower Lcrb between CCs is 6dB higher PSD.

#### Proposal 3: Study possibility to avoid MPR simulations and analysis for each CA output power combination through intoducing MA,delta value (MA,total= MA + MA,delta) ,which is used to offset the reference MPR requirement to new CA output power levels and PA combinations.

# References

[1] R4-2417098, “WF on HPUE RF requirements”, Samsung, Skyworks, Nokia

[2] R4-2414277, “WF on HPUE for CA in TN”, Samsung

[3] R4-2410750, “WF on higher power UE”, Samsung

[4] RP-240828, “New WID : UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, ATT