# 3GPP TSG-RAN WG4 Meeting #113                                                                R4-2419156

# Orlando, US, 18th – 22nd November, 2024

Title: 	Remaining RF issues for supporting 3MHz CBW in NTN bands

Source: 	Huawei, HiSilicon

Agenda item:	7.8.3.2

Document for:	Discussion

# Introduction

RAN4 112-bis meeting the WF [1] was agreed, and some RF requirements are remained as FFS in this document, those open issues will be addressed.

# Discussion

Issue 2-1-3: A-MPR - Way forward - RAN4 to evaluate A-MPR for 3MHz channel BW for NS_02N, NS_03N, NS_04N, NS_05N and NS_24;

In the last meeting we proposed the AMPR evaluations for NS_03N, NS_04N, NS_05N and NS_24[2]. In this meeting we evaluated NS_02N for n255 with the same calibration. It was revealed that no A-MPR was needed, as shown in Figure 1. It should be mentioned that in our evaluation the case with 1MHz MBW was considered as it has stricter requirements than the case with 700Hz MBW.

Figure 1 No A-MPR is required for NS_02N at n255 with 3MHz. The grey colour represents the back off values lower than MPR value.

Proposal 1: For 3MHz at n255, no AMPR is needed when NS_02N is applied. For NS_03N, NS_04N, NS_05N and NS_24 our evaluations were proposed in R4-2416255.

Issue 2-1-9: General spurious emission - Way forward - FFS

Proposal 2: For general spurious emission, as was specified in TN, the 3 MHz should be added into Table 6.5.3.1-1 with an OOB boundary of 6 MHz.

Issue 2-1-10: Additional spurious emissions - Way forward - Option 1: New requirements need define for NS_02N, 03N, 04N, 05N - Option 2: For the additional spurious emissions requirements, add 3 MHz CBW into - Table 6.5.3.3.2-1: Additional requirements for "NS_02N"; - Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N"; - Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N". - Add Table 6.5.3.3.5-1 Additional out-of-band requirements for "NS_24" - Option 3: Introduce 3 MHz channel bandwidth for all network signalling values

Proposal 3: Per our evaluations, option 2 is more comprehensive as for NS_02N, NS_03N and NS_04N, 3 MHz needs to be listed and a table needs to be added for additional out-of-band requirements for NS_24.

Issue 2-2-1: Reference sensitivity - Way forward - Option 1: 2.2dB improvement compared with 5 MHz CBW; - Option 2: 1.7dB improvement compared with 5 MHz CBW;

In TS36.101-1 36% of the bands take 1.7dB improvement and 60% of the bands take 2.2dB improvement, so the majority requires 2.2dB. Moreover, it is true that due to IIP2, some bands such as B2, B3and B12 (all with small duplex distance) had 1.7dB improvement as indicated in R4-103187 and R4-103188. However the duplex distance between the NTN bands are much larger than those bands. Therefore Option 1 is our preference.

Proposal 4: As the duplex distance of NTN bands are large enough, 2.2dB improvement compared with 5 MHz CBW shall be considered.

Issue 2-2-3: ACS - Way forward - Option 1: Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE. - Option 2: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

Same as TN specification, the ACS can be updated to include 3MHz in TS 38.101-5 Table 7.5-1 and Table 7.5-3.

Proposal 5: Option 2: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

# Conclusions

Proposal 1: For 3MHz at n255, no AMPR is needed when NS_02N is applied. For NS_03N, NS_04N, NS_05N and NS_24 our evaluations were proposed in R4-2416255.

Proposal 2: For general spurious emission, as was specified in TN, the 3 MHz should be added into Table 6.5.3.1-1 with an OOB boundary of 6 MHz.

Proposal 3: Per our evaluations, option 2 is more comprehensive as for NS_02N, NS_03N and NS_04N, 3 MHz needs to be listed and a table needs to be added for additional out-of-band requirements for NS_24.

Proposal 4: As the duplex distance of NTN bands are large enough, 2.2dB improvement compared with 5 MHz CBW shall be considered.

Proposal 5: Option 2: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

# Reference

[1]	“R4-2416562, Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi.”

[2]	“R4-2416255, A-MPR evaluations of NS_03N, NS_N04N, NS_05N and NS_24 for 3MHz CBW for NTN communication, Huawei, HiSilicon.”