# 3GPP TSG-RAN WG4 Meeting # 113                                    R4-2419165

# Orlando, US, 18th – 22nd November, 2024

# Title: 	Discussion on MIMO layer evaluation for 6Rx UE

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.1.3.2

# Document for:	Approval

# 1 Introduction

In RAN #103 meeting, the new WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was approved in [1], wherein the objectives related to 6Rx for handheld UE and FWA UE are listed as below:

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

During the last meeting, some conclusions has been reached for 6Rx UE MIMO layer [2]. In the following section, we will provide the detailed discussions for MIMO layer evaluation with 6Rx.

# 2 Discussion

6-layer Support

Way Forward: RAN4 to further discuss the following options (options are not necessarily mutually exclusive). - Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios. - Option 2: 6-layer support is feasible for FWA devices. - Option 3: Companies to further discuss and align on the feasibility criteria and simulation assumptions by RAN4#113 to evaluate 6-Layer feasibility for handhelds using the following as a starting point. - Feasibility criteria - Consider 4-layer vs 6-layer throughput performance. - Simulation assumptions - Only consider link-level simulation for performance evaluations. - Updated simulation assumptions are provided in Annex A for discussions on alignment at RAN4#113 and can be used by companies to provide simulation results at RAN4#113 for discussion. Companies to clarify and justify (e.g., where the number comes from) the settings used in the simulation assumptions for BS correlation matrix and Tx EVM. - Option 4: 6-layer support is not feasible for handheld UEs. - Option 5: Companies to further discuss the following aspect of 6-layer support. - If the handheld feasibility is confirmed, then the handheld and FWA devices will share the same 6-layer requirements. - If the handheld feasibility is not confirmed, 6-layer requirements will be specified for FWA device only.

In last meeting, RAN4 discussed the detailed simulation assumptions for 6Rx, as shown in the appendix. Regarding the assumptions, the most critical is the UE correlation matrix, which was also the subject of extensive discussion at the time. Ultimately, RAN4 presented two candidate options for evaluation. It is obvious that there are still significant difference between these two correlation matrices. We understand that the antenna correlation depends on the implementation, being related to the design and the placement of the antennas, as well as the type of the handle UE. However, the gap between these varying implementations might lead to divergent in the evaluation results, which is also related to BS correlation matrix. There is no doubt that the conclusion of a feasibility study for 6 MIMO layers would also vary.

Observation 1. There are significant differences between the antenna correlation matrices.

Regarding antenna correlation matrix, from legacy testing perspective, RAN4 only defined low, medium A and B, high MIMO correlation matrices for ULA and X-pol from LTE. From our understanding, the correlation matrix currently employed by RAN4 is based on the Kronecker channel model, where the correlation between the transmitter and receiver can be decoupled. Consequently, the channel spatial matrix can be decomposed into the Kronecker form of the transmitter side and receiver side correlation matrices, we merely need to calculate the corresponding correlation matrices according to parameters α and β, which defined in TS38.101-4 section B.2.

And now, we cam directly derive the correlation matrix based on the results of actual measurements. We understand that is due to the specific implementation method. However, different types have varying implementation approaches, which also means that the antenna correlation matrices are not same. From RAN4 testability perspective, this will significantly increase the complexity and burden of testing. To avoid the potential testing issues, RAN4 might need to seek a compromise for addressing the antenna correlation issues. On the other side, if requirements ultimately defined based on practical correlations, will future requirements for multiple antennas also need to be specified according to implementation details rather than following the conventional methods of RAN4.

Observation 2. Different UE types have different implementation methods, which also have different antenna correlation.

We also evaluated the performance of the 4-layer and 6 MIMO layer with different antenna correlation and adaptive MCS, which as shown in following Table 1. Compared to 4 layers, we observed some gain of 6 layers when employing adaptive MCS. Based on our simulation results, we believe that there are certain benefits to consider 6 layers.

Proposal 1. From our simulation results, 6 MIMO Layers shall be considered as an optional feature.

Table 2-1: Throughput curves for adaptive MCS

| (1) TDD 40M/30k 8T6R adaptive MCS TDLA30-10 with option 1 correlation | (2) TDD 40M/30k 8T6R adaptive MCS TDLA30-10 with option 2 correlation |
| --- | --- |

# 3 Conclusion

In this contribution, we give some discussions on the MIMO layers related to 6Rx, The conclusions are:

Observation 1. There are significant differences between the antenna correlation matrices.

Observation 2. Different UE types have different implementation methods, which also have different antenna correlation.

Proposal 1. From our simulation results, 6 MIMO Layers shall be considered as an optional feature.

# 4 References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei).

- R4-2414285, WF on 6Rx UE requirements, AT&T.

# 5 Annex

| Table: Feasibility Study Link-Level Simulation AssumptionsParameters |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30 |
| Antenna configuration |  | {8Tx, 6Rx} as baseline Interested companies can submit results with {32Tx, 6Rx} and specify the precoding type |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,6}, interested companies can bring {5} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 Interested companies can bring MCS Table 1 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx; companies are encouraged to provide details on the SVD update periodicity |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) Option 2: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) Option 4: Rel-15 2+2(4L), Rel-15 2+2(6L) [To be further discussed online] |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx, companies are encouraged to provide details on the SVD update periodicity |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Option 4: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 Option 5: 3GPP MIMO correlation matrix with beta=0.00718 (Option 5 is parametric and to be justified since Options 1-4 are based on measurements) Note: For the feasibility analysis, antenna efficiency can be considered. Companies are encouraged to provide the antenna efficiency numbers. |
| BS correlation matrix |  | Option 1: 3GPP MIMO correlation matrix with Alpha = 0.1 Option 2: 3GPP MIMO correlation matrix with Alpha = 0.0 Other options are not precluded. Matrices requested from interested BS vendors for RAN4#113 |