# 3GPP TSG-RAN WG4 Meeting #113                               R4-24xxxxx

# Orlando, US, 18th – 22nd November, 2024

# Title: 	Simulation results for BS demodulation performance for MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.16.3

# Document for:	Discussion

# 1 Introduction

In RAN #104 meeting, the revised WID of NR demodulation performance requirements for NR BS with MMSE-IRC receiver was approved in [1], wherein the objectives related to NR BS MMSE-IRC receiver are listed as below:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we give some initial simulation results for NR BS MMSE-IRC receiver.

- Discussion

In RAN4 #112 meeting, some simulation assumptions are reached. It is pointed in RAN4 # 112bis meeting that the performance gain of 1 or 2 interferers for 2Rx needs to be checked[2].

Number of interferers - For 4Rx and 8Rx, consider 2 interferers. - Check the performance gain of 1 or 2 interferers for 2Rx and make decision based on simulation result.

We provide our performance comparison with the simulation assumptions listed in Table 2-1.

Table 2-1 Simulation assumption for NR BS MMSE-IRC vs MMSE

| Parameter |  | Tested signal | Interferer 1 (Note 1) | Interferer 2 (Note 1) |
| --- | --- | --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 | N/A | N/A |
| RV sequence |  | 0, 2, 3, 1 | N/A | N/A |
| DIP (Note 2) | Set 1 | N/A | -1.11 | -10.91 |
|  | Set 2 | N/A | -0.43 | -13.78 |
| Interference model |  | N/A | 16QAM randomly modulated data | 16QAM randomly modulated data |
| Cyclic Prefix |  | Normal |  |  |
| Duplex mode |  | FDD |  |  |
| Network type |  | Synchronous scenario |  |  |
| Antenna configuration |  | 1 Tx 2/4/8 Rx |  |  |
| Number of interferers |  | 1 interferer for 2 Rx 2 interferers for 2 Rx |  |  |
| SCS and BW |  | 15kHz SCS, 5MHz |  |  |
| MCS |  | MCS 13 |  |  |
| Waveform |  | CP-OFDM |  |  |
| Receiver type |  | MMSE MMSE-IRC |  |  |
| PUSCH mapping type |  | Type A |  |  |
| Test metric |  | 70% of maximum throughput |  |  |
| Propagation condition |  | TDLA30-10: serving cell, neighbour cell |  |  |
| DMRS | DM-RS configuration type | 1 |  |  |
|  | DM-RS duration | single-symbol DM-RS |  |  |
|  | Additional DM-RS position | pos1 |  |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |  |
|  | DM-RS port(s) | 0 |  |  |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |  |  |

Table 2-2 summarizes the demodulation performance with 70% max throughput operating SNR with the performance curve appended in the Annex.

Table 2-2 Performance gain of 1 or 2 interferers for 2Rx .

| index | Scenario | channel | Number of interfer | SCS, BW | Mapping type | SNR(dB) MMSE | SNR(dB) MMSE-IRC | Gain(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | HomNet | TDLA30-10 | 1 | 15kHz,5MHz | Type A | 13.2822 | 9.8388 | 3.4 |
| 2 | HomNet | TDLA30-10 | 2 | 15kHz,5MHz | Type A | 13.9481 | 11.6140 | 2.3 |
| 3 | HetNet | TDLA30-10 | 1 | 15kHz,5MHz | Type A | 18.0165 | 11.2519 | 6.8 |
| 4 | HetNet | TDLA30-10 | 2 | 15kHz,5MHz | Type A | 18.4098 | 13.3944 | 5.0 |

# 3	Reference

- RP-241297, Revised WID on NR demodulation performance: Phase 5, .

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, China Telcom

# Annex

- Figure A-1 HomNet typeA MCS 13 TDLA30-10 1 interferer

- Figure A-2 HomNet typeA MCS 13 TDLA30-10 2 interferers

- Figure A-3 HetNet typeA MCS 13 TDLA30-10 1 interferer

- Figure A-4 HetNet typeA MCS 13 TDLA30-10 2 interferers