3GPP TSG-RAN WG4 Meeting #113	 R4-2419194

# Orlando , US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on detected cell requirement during state transition and Idle mode/Inactive for UE supporting eEMR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Based on the agreements achieved in previous meetings Agreement in 106bis: - Agreement - UE is allowed to reuse existing measurement, including legacy measurement for cell re-selection and EMR. - Agreements: - If only existing measurement, including legacy measurement for cell re-selection and EMR are used, existing cell re-selection requirements (4.2, 38.133) and idle mode CA/DC measurement requirements (4.4, 38.133) can be reused. Agreement in 112bis: Agreement: - the SSB detectability applies from the earliest possible eEMR measurement (e.g., the configured measIdleValidityDuration or measReselectionValidityDuration before Msg1 transmission) to the end of the defined SCell activation latency (i.e. a valid CQI reporting). - the SSB measured remains detectable according to the IDLE/Inactive mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.. - Based on the agreements and the conclusions achieved in # 112-bis on the detectability for Rel-18 eEMR based fast SCell activation, the Cell/SSBs measured shall remain detectable in RRC connected mode, and during state transition and Idle/Inactive mode - The current detected cell requirement during state transition and Idle/Inactive mode for the idle mode CA/DC measurement can not cover cell re-selection measurent case at least since - “The timer T331 running limitation” does not need to consider in cell re-selection based early measurement - Distinct and dedicate configurations: measValidationReportReselectionMeasurements-r18 and measReselectionValidityDuration-r18 shall be considered for cell re-selection measurement for fast CA/DC setup - The existing detected cell requirement during state transition and Idle mode for CA/DC measurement does not cover: 1) UE configured with idleInactiveNR-MeasReport-r17; 2) Rel-18 eEMR |
| Summary of change: |  | - Add the detected cell requirement during state transition and Idle/Inactive mode for cell re-selection based early measurement for fast CA/DC setup to ensure the known condition, i.e., detectability condition for Rel-18 EMR based fast SCell activation are applicable to non-EMR idle mode measurement case - Update the existing detected cell requirement during state transition and Idle mode for CA/DC measurement to guarantee the requirement can be applied to UE configured with idleInactiveNR-MeasReport-r17 and Rel-18 eEMR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The detected cell requirements during state transition and Idle mode/Inactive for cell re-selection measurement for fast CA/DC setup are missing - The existing detected cell requirements during state transition and Idle mode/Inactive for UE supporting eEMR are not suitable for Rel-18 EMR based fast SCell activation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.X (new), 4.4.2.1, 5.1.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change # 1>

4.2.2.X Detected cell requirement during state transition and Idle mode

This subclause defines the requirements for the detected cell status transfer over the RRC state transitions for cell re-selection measurement when UE transitions from RRC Connected mode to idle mode and after the UE has entered idle mode and transitions to RRC Connected mode.

The requirements are applicable for UE supporting measValidationReportReselectionMeasurements-r18 and configured with measReselectionCarrierListNR-r18 by higher layers.

Upon releasing the connection and if the UE has been configured with measReselectionCarrierListNR-r18 measurement reporting, the following requirements apply:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during the Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies in measReselectionCarrierListNR-r18 with [for early measurement reporting] dedicated RRC signaling, and

-	The detected cell is among the list of cells or on a carrier frequencies provided in measReselectionCarrierListNR-r18, and

-	The detected cell and SSBs remains detectable until UE reconnect to the network, and [and transmits the early measurement report, and]

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

[-	A cell which is detected in the Idle mode, shall remain detected during, and after the UE transitions to the RRC connected mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies in measReselectionCarrierListNR-r18 with dedicated RRC signaling or via SIB configuration and

-	The detected cell is among the list of cells or on a carrier frequencies provided in measReselectionCarrierListNR-r18, and

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.]

[A cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.1 and Annex B.1.2 in [15] for a corresponding Band. An inter-frequency cell is considered detectable according to the conditions in Annex B.1.2 and B.1.3 for a corresponding band. An SSB of an inter-frequency cell is considered detectable according to SSB_RP and SSB Ês/Iot defined in Annex B.1.2 and B.1.3 for a corresponding Band.]

## <End of Change #1>

## <Start of Change # 2>

4.4.2	Measurement Requirements

For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement reporting provided T331 has not expired, the serving cell is supporting idle mode CA/DC measurement reporting and the serving cell is in the validity area.

4.4.2.1	Detected cell requirement during state transition and Idle mode

This subclause defines the requirements for the detected cell status for the idle mode CA/DC measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode. The requirements are applicable to an NE-DC and NR carrier aggregation capable UE which has been configured with one or more of following, one or more SCells, one E-UTRAN PSCell or one or more downlink E-UTRAN SCells during the Connected mode, which supports idleInactiveNR-MeasReport-r16, idleInactiveEUTRA-MeasReport-r16 or idleInactiveNR-MeasReport-r17, and which supports measValidationReportEMR-r18 for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16. The requirements are applicable for SCell(s) and E-UTRAN FDD and TDD PSCell and SCells.

Upon releasing the connection and if the UE has been configured with idle mode CA measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle mode and during Idle mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signaling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signaling, and

-	The detected cell and SSBs remains detectable until UE reconnect to the network and transmits the early measurement report, and

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-RAT E-UTRAN cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.1 and Annex B.1.2 in [15] for a corresponding Band. An inter-frequency cell is considered detectable according to the conditions in Annex B.1.2 and B.1.3 for a corresponding band. An SSB of an inter-frequency cell is considered detectable according to SSB_RP and SSB Ês/Iot defined in Annex B.1.2 and B.1.3 for a corresponding Band. Upon releasing the connection and if the UE has been configured with idle mode CA measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle mode and during Idle mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signaling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signaling, and

-	The detected cell and SSBs remains detectable until UE reconnect to the network and transmits the early measurement report, and

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-RAT E-UTRAN cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.1 and Annex B.1.2 in [15] for a corresponding Band. An inter-frequency cell is considered detectable according to the conditions in Annex B.1.2 and B.1.3 for a corresponding band. An SSB of an inter-frequency cell is considered detectable according to SSB_RP and SSB Ês/Iot defined in Annex B.1.2 and B.1.3 for a corresponding Band.

## <End of Change #2>

## <Start of Change # 3>

## <End of Change # 3>

5.1.2.X Detected cell requirement during state transition and inactive mode

The requirements in clause 4.2.2.X shall apply.