3GPP TSG-RAN WG4 Meeting#113                              R4-2419219

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:	7.20.1

Source: 	ZTE Corporation

Title: 	Discussion on system parameters for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. During the last RAN4 meeting, we have some initial discussions on work plan and general aspects for SBFD BS, however there are still lots of open issues not addressed yet. In this contribution, we would like to share further views on these remaining issues.

- Issue 1-1-1: Minimum channel bandwidth for SBFD (X MHz) Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = 40 as the default value - For band n39, X = 20MHz is used instead. - Issue 1-1-3: Specification implementation for band/channel bandwidth support for SBFD Agreement: - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD. - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD - FFS the band for NR-U shall be supported - Issue 1-1-4: Transmission bandwidth configuration for SBFD Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues. - Issue 1-1-5: DL/UL subband configurations support for SBFD Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case. Sub-topic 1-2: Other general issues for SBFD requirements - Issue 1-2-1: BS RF Specification structure for SBFD requirements Way Forward: - FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements

Discussion

- Issue 1-1-3: Specification implementation for band/channel bandwidth support for SBFD

During last RAN4 meeting, there were some discussions whether NR-U bands should be supported for SBFD feature or not and there were no consensus reached yet. This will have direct impacts on lot of RF requirement since NR-U BS RF requirements are defined differently from the BS RF requirement in the licensed spectrum due to its regulatory mask and also interleaving PRB design from RAN1. Based on the above considerations, we propose to clearly clarify whether NR-U bands in FR1 should be supported or not.

Proposal 1: propose to clearly clarify whether NR-U bands in FR1 should be supported or not.

- Issue 1-1-4: Transmission bandwidth configuration for SBFD

Based on the discussion in the previous RAN4 meeting, it was discussed whether to have informative figure for SBFD transmission bandwidth configuration. As shown in the following Figure 1.2.3-1, similar as the legacy BS PRB utilization, this informative figure might be useful for the clarification in the specification.

Figure 1.2.3-1: SBFD BS PRB utilization

In addition, according to the SCS restriction in TS 38.331 specification, the SCS for active DL BWP and active UL BWP should be the same, we also highlighted in the following diagram to align the numerology in DL sub-band and UL sub-band.

Proposal 2: SCS for DL and UL sub-band should be the same.

- Issue 1-1-5: DL/UL subband configurations support for SBFD

- In the previous RAN4 meeting, we have some initial discussions on how to specify the DL/UL subband configuration for SBFD. Option 2b should be best option to move forward with the following reasons:

- DL sub-band: if DL sub-band is specified explicitly and leave the GB and UL sub-band configuration up to the manufacture declaration, then it is not possible to specify the SBFD BS receiver requirement properly. In addition, DL sub-band configuration is mainly used for transmitter requirement definition or conformance testing which is somehow similar as test model specified in TS38.104. Whether to specify the in-channel emission requirement or in-channel leakage requirement is also ongoing. In other words, the receiver requirement cannot be addressed for DL sub-band configuration only.

- UL sub-band: the main reasons to specify the UL sub-band explicitly is due to the BS receiver requirement. It’s well known that lots of BS receiver requirement (e.g. REFSENS FRC) is highly dependent on the UL PRB configurations, corresponding MCS and reference signal configuration. If UL sub-band is missing and leave it up to the manufacture declaration, then it is not possible to define the receiver RF requirement appropriately. The details could be found as following especially for FRC design for UL sub-band.

During last RAN4 meeting, there were some discussions on alternative 1 or alternative 2 for FR1 WA SBFD DL configuration, from our understanding, alternative 1 is not reasonable since the DL-UL traffic pattern might be different from different operators perspective, if we have only one DL-UL pattern or DL-UL-DL pattern, then it would be very restrictive for the practical deployment. In other words, for FR1 WA BS, we prefer to have multiple DL-UL sub-band configuration instead of single one.

Proposal 3: to support the multiple DL sub-band sizes for SBFD BS to accommodate the operators’ deployment request.

    - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested

      - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS.

In addition,regarding the specific UL sub-band configuration for certain carrier bandwidth also need more discussions if option 2b is agreeable. For example, for 100MHz carrier bandwidth, whether all sub-band configuration e.g.20, 25, 30, 35, 40, 45,50, 60, 70,80,90MHz should be specified or only part of carrier bandwidth should be specified to simplify the specification.

From our understanding, it might be better to have all sub-band configuration to be specified with both FRC and power levels defined, otherwise some of RAN4 specification might be not aligned with the practical deployment especially considering the analog filtering design which is tightly related with certain DL-UL sub-band configuration, therefore in order to have more complete specification and allow for the alignment between specification and the practical deployment, all sub-band configuration should be specified.

Proposal 4:all UL sub-band configuration (e..g.reusing from the existing carrier bandwidth, 20~90MHz) should be specified for SBFD receiver requirements together with FRC and power levels. The detailed MCS, reference signal pattern for licensed band could align with WF R4-1801031. For unlicensed bands, the interleaved PRB pattern should be also configured.

- Issue 1-2-5: Restriction on guardband size for SBFD

In the previous RAN4 meeting, there were some discussions on the guard band size for SBFD. From our understanding, there are two kinds of guard band concept as following at least:

- Guard band at the carrier edge (e.g. in red color) which is supposed to the same as legacy GB design;

- Guard band between DL sub-band and UL sub-band (e..g in orange color);

- For the guard band between DL sub-band and UL sub-band, this is highly dependent on BS class, DL/UL sub-band configuration etc. For example, for FR1 wide area SBFD BS, due to the potential analog RF filtering between DL and UL sub-band, then guard band size would be expected to be larger, however for other BS class, the guard band might be quite limited e.g. limited to 1 PRB or even less than 1 PRBs. Whether to define maximum guard band size based on the worst assumption is also questionable since the analog RF filtering is still debatable so far for FR1 WA SBFD BS. In addition, guard band between different spectrum block is not something new in TS 38.104 specification. As extracted from the existing TS 38.104 specification, the guard band between different numerologies is up to the implementation dependent. This kind of design principle could be also reused for guard band between DL-UL sub-band configuration of SBFD BS.

Figure 1.2.5-1. DL/UL sub-band configuration

Figure 5.3.3-2: Guard band definition when transmitting multiple numerologies

NOTE:	Figure 5.3.3-2 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

Proposal 5: propose to guard band between DL and UL sub-band is up to the declaration from the implementation perspective.

- Issue 2-1-3: Location of transient period between SBFD and non-SBFD

Based on the discussions in the previous meetings, the location of transient period between SFBD and non-SBFD symbols is not determined yet and the necessity of this requirement is also debatable so far. At least from our understanding, the transition period should be located within SBFD symbols instead of non-SBFD symbols to avoid the negative impacts on the legacy UE reception. It is well known that, during the BS Tx OFF-ON or ON-OFF, the transmitter signal quality within the transition period cannot be guaranteed properly. Usually from UE receiver demodulation perspective, the corrupted soft information should be dropped before feeding into the LPDC/Polar decoder. For the SBFD aware UE, UE could know the location of the transition period by the TDD DL-UL configuration and additional SBFD configuration in RRC inf, then the corrupted soft information could be dropped within the estimated window length and baseband performance could be ensured somehow. If the transition period is located within the non-SBFD symbols or across the non-SBFD symbols and SBFD symbols, then the performance degradation for non-SBFD symbols will be expected. In short, we propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols.

Figure 2.1.3-1. the location of transition period between SBFD and non-SBFD symbols

Proposal 6: propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols.

Conclusions

In this contribution, we want to share some initial views on general aspects for SBFD BS and proposals/observations are made as following:

Proposal 1: propose to clearly clarify whether NR-U bands in FR1 should be supported or not.

Proposal 2: SCS for DL and UL sub-band should be the same.

Proposal 3: to support the multiple DL sub-band sizes for SBFD BS to accommodate the operators’ deployment request.

Proposal 4:all UL sub-band configuration (e..g.reusing from the existing carrier bandwidth, 20~90MHz) should be specified for SBFD receiver requirements together with FRC and power levels. The detailed MCS, reference signal pattern for licensed band could align with WF R4-1801031. For unlicensed bands, the interleaved PRB pattern should be also configured.

Proposal 5: propose to guard band between DL and UL sub-band is up to the declaration from the implementation perspective.

Proposal 6: propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2406107, Way Forward for [110bis][313] NR_duplex_evo

- R4-2409958, Way forward for [110bis][313] NR_duplex_evo, Approved

- R4-2413514	Way Forward for [112][307] NR_duplex_evo_General, Approved.

- R4-2413514	Way Forward for [112][307] NR_duplex_evo_General

- R4-1801031, WF on NR BS RF FRCs, approved.