3GPP TSG-RAN WG4 Meeting #113 	R4-2419239

Orlando, USA, Nov 18 – 22, 2024

Source: 	Huawei, HiSilicon

Title: 	Further discussion on Expected EIRP test

Agenda Item:	7.11.4.1

Document for:	Approval

# 1 Introduction

In RAN4#112bis the WF [1] was approved which had a number of open issues regarding the conformance testing of the EEIRP requirement.

Issue 2-1: The formulation of EEIRP mask

Issue 2-2: CAR and mechanical tilt related issue

Issue 2-3: Test signals and test model

Issue 2-4: Test beams for EEIRP mask

Issue 2-5: Measurement grid over elevation angle and azimuth angle

Issue 2-6: Conformance testing for different mechanical tilts

Issue 2-7: RF channels for the conformance testing

Issue 2-8: Test procedures

Issue 2-10: Measurement uncertainty

Issue 2-11: CI

Issue 2-12: Others

In this paper we will discuss our views on each of these issues.

# 2 Discussion

## 2.1	Issue 2-1: The formulation of EEIRP mask

There is a discussion about the way the EEIRP equations are written and the various variables defined. Currently there is use of radians in some places and degrees in others. Whilst of course the equations should be correct and understandable these types of mixing of terms is not uncommon and generally can be identified as such rather than a real error, it should not be necessary to choose degrees or radians universally as long as it is made clear the unit being used.

As a preference of course real world steering angles are currently defined as degrees and this should remain as they are more useful in the applications we deal with (steering rages, tilts etc).

## 2.2	Issue 2-4: Test beams for EEIRP mask

The following agreements and WF were recorded in [1]

- The description of test beams shall be derived from a formula within the beam steering range.

- [Equally weighting factors] to be assumed for test beams;

- For EEIRP evaluation, at least consider the antenna array assumptions on 7GHz reply LS to ITU-R; other antenna configurations (e.g. one smaller and one larger array sizes) are not precluded.

There has been some offline alignment between the antennas and the test beam sets to be further investigated.

Our results use an electrical steering range with an 8deg mechanical tilt. This was initially based on the usable steering range of the antennas we investigated, although it seems 0-16deg is the case used by others (although this appears to be greater than the usable range of ANT4?). The relative performance of the test direction sets seems comparable so we have maintained these conditions to provide an additional data point.

Errors are given with respect to a 10k random direction data set for both a rectangular steering range and ellipse steering range assumption. It can be noted that there are small but notable errors (0.54dB rms average) between our reference case and the regular 2057 point reference which has been used in some results.

Table 2.2-1 shows the antennas under consideration, in this case ANT4 was investigated.

Figure 2.2-1 shows the test direction sets.

Table 2.2-2 shows the results for the +/-5deg steering range with 8 deg mechanical tilt

Table 2.2-1: Antenna parameters

Figure 2.2-1. Test beam direction sets

Table 2.2-2: Results for +/-5deg steering range 8deg tilt

We believe the Ellipse steering range case is the most important as it more closely resembles real products, however ideally the selected test beam direction set will have a small error for both the elliptical and rectangular assumption. As such minimising both seems appropriate.

The results do not suggest that the addition test time required to implement the larger test sets is justified, there are a number of the 9 point test beam sets which perform comparably (3x3A, 9 point diamond), the 17 point ellipse pattern is also good but favours the rectangular distribution.

Of the 9-point patterns the 9 point diamond is well balanced in terms of performance across the distributions, 3x3A is optimum for the ellipse and still good for the rectangular reference. Either of these will probably be acceptable however based on the ellipse performance we still favour the 3x3A pattern.

Proposal 1: The 3x3A test pattern is selected.

## 2.3	Issue 2-5: Measurement grid over elevation angle and azimuth angle

For the WF we have the following

For elevation angle:  maximum step size as 1 degree

For azimuth angle: maximum step size as  [1 to  5] degree

For the azimuth step size we have noticed the following:

Figure 2.3-1: EEIRP error vs azimuth grid step size

The graphs clearly show that the accuracy is directly related to the beam width (which differs from elevation case where it is not dependent) as such agreeing an absolute step size is not optimum. For narrow beams it may be to large and introduce inaccuracies and for wide beams it will introduce unnecessary measurement time.

Whilst for the range of antennas and beam widths investigated the upper end of the suggested step size (5deg) would just about be sufficient, that would not be the case if a wider antenna with a narrow beam width were deployed.

These results are almost the same as was found for the TRP grid size (for the same reason) as such it is proposed to adopt the same approach as for TRP as described in 38.141-2 annex I2.2.

Which is effectively half the beam width (with by using antennas dimensions or beam width it’s a similar result)

Proposal 2: For azimuth step size use the same approach as TRP (effectively half the beam width)

## 2.4	Issue 2-6: Conformance testing for different mechanical tilts

In the last meeting in [2] we provided some analysis where both the mechanical tilt and the electrical steering range were varied.

The metric used in this case was the minimum margin over the specification.

Figure 2.4-1. left: Margin vs mechanical down tilt, right margin vs electrical steering range

In figure 2.4-1 in the left hand graph the mechanical tilt is varied for various electrical steering ranges. In all cases it can be seen that as mechanical tilt increases the margin gets larger.

Observation 1: Worst case margin is when mechanical tilt in minimum

In figure 2.1-1 in the right hand graph the electrical steering range is varied (for a fixed mechanical tilt). It is clear that as the steering range increases the margin reduces

Observation 2: Worst case margin is when electrical steering range is maximum.

It seems clear that the worst-case condition is when mechanical tilt is minimum (closest to horizon) and the electrical steering range is maximum. As such this condition should be declared and tested.

Proposal 3: The minimum mechanical down tilt and the maximum electrical steering range shall be declared.

In real products we should allow for that fact that these 2 conditions may not be declared simultaneously as such the following caveat should be added.

Proposal 4: If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt

## 2.5	Issue 2-7: RF channels for the conformance testing

The WF proposes to further discuss the following options:

  - Option 1: B, T

  - Option 2: M.

  - Option 3: T

In [3] we looked at a number of antennas and varied the wavelength by 10% (+/-5%) to approximate the effect of frequency variation across the n106 band. The EEIRP results were compared to those recorded for the middle channel.

We found the errors were small (consistent with the error between the test direction sets and the reference) and also that they had no clear pattern with respect to frequency. It was not clear that T had less margin than M or B.

As such no clear worst case exists, within the level of the errors, measuring T or even T&B does not offer a worst case set that guarantees performance at M.

As the level of error is consent with the MU levels we are calculating measuring a single channel seems sufficient.

As it is not clear that T or B is worst case using the middle channel is preferable.

Proposal 5: Use Option 2: M

## 2.6	Issue 2-10: Measurement uncertainty

Measurement uncertainty has 2 parts that need to be discussed:

- The normal MU of the OTA measurement, the chamber accuracy etc

- The uncertainty of the EEIRP averaging assumptions – measurement grid sizes, beam steering test patterns etc.

The 1st part should be relatively simple as we have done this a number of times, the mythology and many of the error definitions and uncertainty assumptions are documented in the TR 39.941. We can use this as a good starting point to estimate the MU for each of the directional power measurements associated with the EEIRP measurement.

The 2nd part is more difficult. We had a similar issue when looking at TRP measurements and grid sizes, the solution was to define an uncertainty called TRP summation error which accounted for errors between using an infinite set of spherical measurement grid points and a reduced set which provided a reasonable measurement time.

For EEIRP this issue is extended somewhat as we have both a measurement grid (split into sections above the horizon) and also a set of random beam directions which must be averaged.

The requirement from Resolution 220 which covers both of the EEIRP averaging steps is:

The averaging processes in steps 1 and 2 shall allow for accurate averaging of the expected e.i.r.p. (e.g. to the confidence interval of 95%).

Unfortunately “accurate averaging” is not a clear requirement. Potentially the example of 95% CI can be used to approximate an accuracy requirement but is not clear how this should be done ? This is further discussed below in another open issue, however considering the term “accurate averaging”.

The target of the RAN4 conformance specifications and test methods is to provide a measurement technique which is practical and sufficiently accurate considering the tools available. Generally test equipment and calibration accuracy is estimated based on a 95% confidence interval, generally assuming random errors (i.e. normal) are the source of inaccuracies, where error distributions are not normal the distribution is estimated and corrections made so it can be applied to the sum.

The target of the conformance work therefore would be to do the same task and apply it to the EEIRP requirement so that Measurement Uncertainty is minimised (within practical reason) i.e. it is accurate. Part of this would be to ensure the 2 averaging procedures (grid step size and test beam directions) allow for this accuracy and the calculated MU is quoted at a 95% CI.

If we use a similar technique to the one used for TRP where a TRP summation error term was applied to the MU calculation (EEIRP averaging error maybe ?) there is of course still the issue of what is a reasonable value ofr that error. For TRP it was 0.75dB.

If the errors are considered orthogonal and can be added rms (as we generally assume) then if we have an estimate of the test equipment error then the effect of the [EEIRP averaging error] can be put into context.

#### 2.6.1	Measurement uncertainty calculations

The EEIRP measurement is very similar to the in-band TRP measurements, as such the in-band emission MU budgets can be used as a starting point for the EEIRP budgets.

Additional sources of uncertainty are:

Dynamic range uncertainty

One major difference with EEIRP however is that it is necessary to measure and add power on the same frequency as the wanted signal when the wanted signal is present and steered in another direction. Existing measurements either are measuring on adjacent channels (where frequency isolation can be used) or where the wanted signal is also being measured and is dominant. As such we are relying on the chamber spatial isolation to remove the effect of the wanted signal. Reflections and refractions of the main beam will limit the minimum power level that can be accurately measured.

This error can be referred to as dynamic range uncertainty.

The lowest EEIRP requirement is 15dBm/MHz

Currently the dynamic range contribution is 0.51dB, in order to maintain that level of accuracy in the above scenario an isolation of approximately 58dB would be required:

Pint=10*log101063.7-5810+101510=15.5dBm/MHz i.e. 0.5dB error

EEIRP sampling Error

As is being discussed the ideal definition and a reasonably sampled test case will result in potential uncertainties.

These will occur due to the differences between the test beam directions set and infinite set of random beam directions and also due to errors caused by the sampling grid.

Currently the analysis of the test beam direction sets we are using various different metrics to compare the accuracy of the beam sets. Its not clear what is the exact metric (or on whose data it will be derived) would be suitable to add into the MU budget (its distribution etc, its standard deviation etc), however this can be further discussed.

EEIRP sampling error seems like an appropriate name for this parameter a potential definition is provided below:

EEIRP sample error – Error due to the sampling of the 2 averaging processes used to define EEIRP. The sampling error covers errors due to difference in power distribution due to the test beam direction set and an infinite set of random beam directions as well as errors due to the sampling grid.

The value and the distribution assumption for the EEIRP sampling error needs to be further discussed. When adding uncertainties to the MU calculation the sum is done after converting the standard deviation to an equivalent normal standard deviation so they can be added.

So that we can provide an example in the tables below we have assumed a worst-case error of 1.2dB as placeholder, if we assume a gaussian distribution then this is added in as a standard deviation of 0.6dB. This is done only to demonstrate an approximate value at this stage, the exact nature of the error and the optimum way to add it in needs to be further discussed

These 2 errors have been added to the MU budget as follows:

| Table 11.2.2.3-1: IAC MU value derivation for EIRP measurement of OTA BS output power |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-1 | Positioning misalignment between the BS and the reference antenna |  |  | 0.03 | Rectangular | 1.73 | 1 |  |  |  |  | 0.02 |  |
| A1-2 | Pointing misalignment between the BS and the receiving antenna |  |  | 0.3 | Rectangular | 1.73 | 1 |  |  |  |  | 0.17 |  |
| A1-3 | Quality of quiet zone |  |  | 0.1 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4a | Polarization mismatch between the BS and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5a | Mutual coupling between the BS and the receiving antenna |  |  | 0 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.26 | Gaussian | 1.00 | 1 |  |  |  |  | 0.26 |  |
| A1-7 | Impedance mismatch in the receiving chain |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A1-8 | Random uncertainty |  |  | 0.1 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| A2-21 | Measurement system dynamic range uncertainty |  |  | 0.51 | Gaussian | 1 | 1 |  |  |  |  | 0.51 |  |
| *new | [EEIRP sampling error] |  |  | [0.6] | [Gaussian] | 1 | 1 |  |  |  |  | [0.6] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-9 | Impedance mismatch between the receiving antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-11 | Impedance mismatch between the reference antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-3 | Quality of quiet zone (normal test conditions) |  |  | 0.10 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4b | Polarization mismatch between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5b | Mutual coupling between the reference antenna and the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A1-12 | Influence of the reference antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| A1-13 | Reference antenna feed cable loss measurement uncertainty |  |  | 0.06 | Gaussian | 1.00 | 1 |  |  |  |  | 0.06 |  |
| A1-14 | Influence of the receiving antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A1-15 | Uncertainty of the absolute gain of the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [0.95] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [1.87] |
| Table 11.2.3.3-1: CATR MU value derivation for OTA BS output power measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) |  |  | 0.30 | Rectangular | 1.73 | 1 |  |  |  |  | 0.173 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.26 | Gaussian | 1.00 | 1 |  |  |  |  | 0.260 |  |
| A2-2a | Standing wave between BS and test range antenna |  |  | 0.21 | U-shaped | 1.41 | 1 |  |  |  |  | 0.148 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.001 |  |
| A2-4a | QZ ripple experienced by BS |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.093 |  |
| A2-12 | Frequency flatness of test system |  |  | 0.25 | Gaussian | 1.00 | 1 |  |  |  |  | 0.250 |  |
| A2-17 | Measurement system dynamic range uncertainty |  |  | 0.51 | Gaussian | 1 | 1 |  |  |  |  | 0.51 |  |
| *new | [EEIRP sampling error] |  |  | [0.6] | [Gaussian] | 1 | 1 |  |  |  |  | [0.6] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A2-6 | Insertion loss of receiver chain |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.00 |  |
| A2-7 | Influence of the calibration antenna feed cable |  |  | 0.02 | U-shaped | 1.41 | 1 |  |  |  |  | 0.02 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A2-8 | Misalignment positioning system |  |  | 0.00 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.00 |  |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) |  |  | 0.50 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.25 |  |
| A2-9 | Rotary joints |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.03 |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  |  | 0.09 | U-shaped | 1.41 | 1 |  |  |  |  | 0.06 |  |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) |  |  | 0.01 | Gaussian | 1.00 | 1 |  |  |  |  | 0.01 |  |
| A2-11 | Switching uncertainty |  |  | 0.26 | Rectangular | 1.73 | 1 |  |  |  |  | 0.15 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [1.03] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [2.03] |

As with the TRP cases the CATR is slightly higher error so we can concentrate on this when considering the final MU value.

#### 2.6.2	Measurement uncertainty summary

In this section we have tried to discuss measurement uncertainty without offering a definitive explanation of the accuracy statement in resolution 220.

Using a similar approach as to that done for TRP the measurement test system accuracy has been evaluated and presented.

As an example (we are not currently proposing this number) a reasonable EEIRP averaging error value has been suggested based on the test system HW accuracy and other similar OTA TRP measurements.

## 2.7	Issue 2-11: CI

Confidence interval was discussed along with measurement uncertainty in the previous sub-clause. In this case confidence interval is important as it is given as an example of accuracy in resolution 220. As follows:

The averaging processes in steps 1 and 2 shall allow for accurate averaging of the expected e.i.r.p. (e.g. to the confidence interval of 95%).

It seems clear that this requirement is specifying an accuracy of the averaging requirement, unfortunately an actual accuracy figure is not given. The example of 95% CI indicates that the accuracy should be related to a distribution, if we had a distribution then the standard deviation and the 95% CI could be used to estimate a required accuracy value.

It is not clear however what the data set is (or should be in this instance), the 2 averages in the requirement do not have a statistical distribution they are integrals with a single correct answer.

There are 2 averages in the resolution

•	The measurement grid points around the sphere

•	The number of beam directions within the steering range

Both averages need to be done to form an EEIRP value, a single measurement point on the grid or a single beam direction are not a sample of EEIRP but are EIRP. The distribution of EIRP under these conditions does not relate to the accuracy of the EEIRP measurement.

Observation 3: Individual EIRP measurements are not samples of EEIRP and their distribution does not indicate EEIRP accuracy.

It has been suggested that a distribution can be formed by effectively taking the EEIRP an applying it to a single beam direction within the steering range and using that as a sample. Intuitively because the beam is quite narrow and moves over a wide steering range it seems the samples using this procedure will be very different but as the data is available from the simulator it can be analysed, the histograms of each EEIRP angular bin are shown in figure 2.10-1 (note. not all of the bins are shown for brevity):

Figure 2.7-1. Histogram of EIRP samples in selected EEIRP bins – 10k samples

It can be seen that the distributions are not generally normal and also, they differ considerably

If we look at the average and standard deviation for each

| Elevation bin | Average | STD DEV |
| --- | --- | --- |
| deg | dBm | dB |
| 0-5 | 27.25 | 3.85 |
| 5-10 | 22.6 | 1.79 |
| 10-15 | 19.99 | 2.68 |
| 15-20 | 18.23 | 6.14 |
| 20-30 | 15.75 | 4.5 |
| 30-60 | 10.01 | 3.12 |
| 60-90 | -3.73 | 6.73 |

The smallest standard deviations are for the data sets that ted to be more normal in shape, however the best of these the 5-10deg angular bin range would still have a 95% CI of 3.5dB which is large. The accuracy of OTA measurements and being discussed for EEIRP is in the order of ~2dB. If 95% CI were taken from these distributions then we are exceeding that requirement significantly.

Observation 4: If statistical analysis is done assuming individual beam directions are a sample of EEIRP the standard distribution and associated CI is larger than the measurement accuracy we typically target.

# 2 Summary

This paper covers a number of open items that were identified in the WF [1], a brief summary of each issue is as follows:

For the test beams direction sets we find that the sets with a larger number of beams do not provides any additional accuracy and that 9 test directions is sufficient. The 3x3A pattern and the 9 point diamond are bothe acceptable but ou preference is the 3x3A is it is superior in Ellipse reference case performance.

Proposal 1: The 3x3A test pattern is selected.

For the azimuth measurement grid step size the performance is directly related to beam width, this is the same case as TRP so we propose using the same methodology as for TRP.

Proposal 2: For azimuth step size use the same approach as TRP (effectively half the beam width)

When considering the worst case measurement conditions we have shown that minimum tilt and maximum electrical steering range are the worst case.

Observation 1: Worst case margin is when mechanical tilt in minimum

Observation 2: Worst case margin is when electrical steering range is maximum.

Proposal 3: The minimum mechanical down tilt and the maximum electrical steering range shall be declared.

Proposal 4: If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt

For the RF test channel, there is no clear relationship between EEIRP performance and frequency, as such there is no clear worst case channel. The middle channel therefore is a reasonable channel to measure.

Proposal 5: Use Option 2: M

A draft proposal has been provided for MU analysis, the budget is based on the in-band emissions TRP uncertainty analysis, 2 new uncertainties have been introduced which are EEIRP specific including an error term to cover EEIRP sampling error. A definition of EEIRP sampling error has been suggested.

The resolution requirement on accuracy has been discussed and our view of the example 95% confidence interval has been provided. Histograms and average/standard deviation data have been collected for 10k random beam directions to demonstrate that this approach provides an accuracy requirement which is considerably larger than the measurement uncertainty we are targeting.

# References

[1]	R4-2416572 WF for [112bis][302] NR_BS_RF_Part1_E_EIRP – ZTE

[2] R4-2416277 - Further discussion on Expected EIRP test - Huawei

[2] R4-2413276 Expected EIRP conformance – Test Vectors, MU budget and Conformance test method - Huawei