3GPP TSG-RAN WG4 Meeting #113 	R4-2419241

Orlando, USA, Nov 18 – 22, 2024

Source: 	Huawei, HiSilicon

Title: 	TP to TR 38.908 – section 6.5

Agenda Item:	7.11.1

Document for:	Approval

# 1 Introduction

In RAN4#112bis the drafting of sub-clause 6.5 of the TR was allocated to Huawei.

This TP is based on the current draft content in [1] which contains a draft version of the conformance TS text, as this is for the technical report just repeating the TS text add no information hence the background for this text has been added. The background is based on submission on the subject and discussion in off line meetings.

Some of the text in the test procedure has bene modified to remove potential confusion between variable names however the steps remain the same. Step 1 stating the calibration needs to be performed has been discussed offline and has been removed in this version.

# 2 TP to TR 38.908

## 6.5	Method of test [Huawei]

### 6.5.1	General

The EEIRP requirement contains 2 averaging processes, an average over the measurement points on the measurement grid (see sub-clause 6.2) and an average of beam steering directions. Mathematically these averages can be carried out in any order, as such the method of test may also carry out the averages in any order.

As it is likely that moving the positioner so a new measurement grid point can be measured will be a slower process than changing the beam steering direction, as such the method is written assuming beam steering averaging is done first.

### 6.5.1	Initial conditions

The initial conditions section contains the test set up initial conditions including:

- the test environment,

- the channels to be tested

- the beams to be tested

- etc

Test environment

EEIRP is to be measured under normal conditions

As EEIRP is based on average performance of a large number of BS over a network it is reasonable to assume normal conditions would represent the average network condition.

Currently only a few parameters are measured under extreme conditions which is extremely challenging as the OTA chamber cannot be environmentally controlled. Controlling the temperature of the DUT locally is done for limited tests but this could not be applied to EEIRP measurements.

RF channels

EEIRP measurement is a very long measurement, the number of channels tested will directly multiple the test time.

The frequency variation across the EEIRP band is approximately 10%, over this range the gain of the main beam will change. However EEIRP is dominated by the sidelobe behaviour of the antenna pattern rather than the main beam, looking at the EEIRP performance of an antenna over the frequency range there is no clear worst case, the performance is no worse at B or T than it is at the middle frequency.

As no frequency is worse then any other it could be argued any channel could be used, as such the middle channel is the obvious choice.

Test signal and test model

EEIRP requirement is based on PSD, testing should be carried out with the highest PSD the BS is capable of.

The performance is dependent on the radiation beam shape, non-linearities or distortions potentially generated by multi-carrier signals will not significantly alter the beam shape of the wanted signal.

As such as single carrier test signal is used with test model NR-FR1-TM1.1.

As the requirement is PSD if the BS output power is constant then this will be achieved at the smallest channel band width. Potentially in order to meet EEIRP small channel bandwidth may have a limited output power capability. As specific channel BW is hence no specified only that we test with the highest PSD and corresponding channel BW.

Test beam direction set

The test beam direction set has been discussed in sub-clause 6.3. The test beam directions are described with respect to the declared beam direction sets. In it the initial conditions the exact beam direction set is defined.

The steering range is defined by the following directions:

OTA peak directions set reference beam direction pair (D8)

OTA peak directions set maximum steering direction(s) (D10)

D10-1)	The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction.

D10-2)	The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction.

D10-3)	The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction.

D10-4)	The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction.

These 5 directions are defined for each of the beam identifiers required for the EEIRP conformance.

The worst-case beam identifiers with respect to mechanical tilt and electrical steering range are FFS.

The steering directions can be represented in a table {ed: test direction set not agreed yet – table shown as an example of how to represent the directions with respect to the declared values}

[

| Point No (k) | θ | ϕ |
| --- | --- | --- |
| 1 | θD8-0.66(θD8-θD10_4) | φD8-0.66(φD8-φD102) |
| 2 | θD8-0.66(θD8-θD10_4) | φD8 |
| 3 | θD8-0.66(θD8-θD10_4) | φD8-0.66(φD8-φD10_1) |
| 4 | θD8 | φD8-0.66(φD8-φD10_2) |
| 5 | θD8 | φD8 |
| 6 | θD8 | φD8-0.66(φD8-φD10_1) |
| 7 | θD8-0.66(θD8-θD10_3) | φD8-0.66(φD8-φD10_2) |
| 8 | θD8-0.66(θD8-θD10_3) | φD8 |
| 9 | θD8-0.66(θD8-θD10_3) | φD8-0.66(φD8-φD10_1) |

]

The test beam direction set is applicable to a specific mechanical tilt. Potentially different electrical steering ranges will be associated with different mechanical tilts. The worst-case conditions are:

[The minimum mechanical down tilt and the maximum electrical steering range shall be declared.

If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt]

The mechanical tilts and associated electrical steering ranges are declared.

In addition as this requirement is related to the horizon where as all other OTA requirements are related to the BS enclosure an additional statement is made

The test object shall be placed on the positioner with mechanical tilt set to 0 degrees.

Since this requirement is targeting the power transmitted above the horizon for a specific site installation, it is required that the BS is mounted on the positioner such that blocking effects due to the positioner are minimized.

### 6.5.2	Test procedure

The test procedure is based on a CATR/IAC where the BS is placed on a positioner and rotated so the required measurement direction is in line with the measurement antenna. The procedure has similarities to existing EIRP and TRP measurement procedures.

The information on the EEIRP measurement grid is discussed in sub-clause 6.4, the resulting grid requirements will be placed in an annex of TS 38.141-2 and referenced in the measurement procedure a similar approach as the TRP measurement grids.

The test procedure is as follows:

- Place the test object on the positioner, such that blocking effect in the vertical domain shall be minimized.

- Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

- The measurement device characteristics shall be:

-	Detection mode: True RMS.

-	The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1 for FR1.

- Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

- Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine EEIRP can be performed (see TS 38.141-2, Annex [M]).).

- Enable test beam  k for k=1..K from sub-clause [6.8.1] {ed: – initial conditions clause}

- Measure EIRP=EIRPp1+EIRPp2 at the specified frequencies with specified measurement bandwidth, where p1 and p2 denotes two orthogonal polarizations.

- Repeat step 6 - 7 for all K test beams.

- Repeat step 6 - 9 for all positioner locations in the appropriated EEIRP measurement grid (see annex M)

- Calculate EEIRPi profile from measured EIRP in step 7 for each elevation angular range (i=1..7 as described in TS 38.141-2, Annex [M]).

# References

[1]	R4-2416573	TP to TR 38.908 clause 6 – Conformance testing – ZTE