3GPP TSG-RAN WG4 Meeting #113	R4-2419249

Orlando, US, 18th – 22nd November 2024

Source:	Ericsson

Title:	Discussion on Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	7.16.3

Document for:	Discussion

# Introduction

In previous RAN4#112bis meeting, companies discussed the test scenarios, interference modelling and simulation assumptions for Rel-19 NR MMSE-IRC BS requirement. The agreements on general issues are captured in our other contribution [1].

In this contribution, the open issues on interference modeling and test parameters will be furtherly discussed as per the WF [2].

# Discussion

## 2.1	Interference Modelling

Agreements:

Interference modelling criteria

- On interference modelling criteria:

  - Use INR-based modelling and transfer the DIP value into INR value

- Agreement on DIP -> INR transformation:

- For 1 interferer only scenario, INR1=DIP11-DIP1 - For 2 interferers scenario, INR1=DIP11-DIP1-DIP2, INR2=DIP21-DIP1-DIP2

- Cases for link level evaluations based on the legacy LTE profile

Scenario; (DIP1, DIP2) dB; Transferred (INR1, INR2) dB. HetNet with 1 interferer; (-0,43, N/A); (9.82, N/A). HetNet with 2 interferers; (-0,43, -13,78); (12.38, -0.97). HomNet with 1 interferer; (-1,11, N/A); (5.35, N/A). HomNet with 2 interferers; (-1,11, -10,91); (7.28, -2.52).

Agreements:

Interference profile

- On interference modelling:

  - Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH

Open Issue:

- On how to account for HPUE in existing LTE profiles:

  - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE.

  - It is not precluded for companies to bring system level simulation on other profiles.

  - Proposals on how to account for HPUE in existing LTE profiles.

- Proposal 1: Perform new system-level simulations with the inclusion of HPUEs

- Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB

For HPUE, the ideal way would be to conduct system-level simulations to derive the DIP/INR values and analyze the performance gain of MMSE-IRC receiver vs MMSE. Although, system-level simulations would provide better insights in inter-cell interference scenario in NR, considering the study effort and no difference between the baseband processing of MMSE-IRC and MMSE receiver, LTE interference profile can be reused.

Further, even if the deployment of NR is much different from LTE, not many UEs would be HPUE and even a smaller number of HPUEs are expected to transmit at high power during a limited time (for e.g. at cell edge). On the other hand, the HPUE would have higher SINR when it is the wanted UE comparing to normal handheld UE. The percentage of HPUE would be dominant the trend of DIP curve, for example, the DIP could be similar when 1% and 99% of total UE are HPUE. Thus, It is hard to judge that the DIP curve would be very different from LTE profile when HPUE is deployed.

Observation 1: It is hard to judge that the DIP curve would be very different from LTE profile when HPUE is deployed which would be highly impacted by the HPUE percentage.

However, if the companies are willing to conduct system-level simulations, we recommend an alternate approach. It would be easier to extract the statistics from the DIP1 distributions of HomNet and HetNet scenarios, respectively in LTE from TS 36.884. These statistics can be used to generate random strongest interferers and introduce some percentage of HPUE interferers. Finally, new statistics and DIP1 distributions can be generated and used for link-level simulations.

Observation 2: An alternate approach to system-level studies can be discussed to derive DIP/INR in NR due to HPUE.

Further, DIP the ratio of the power of a given dominant interferer over the total power of all interferers along with the white noise. While INR is the ratio of average received power spectral density from the strongest interfering cell and the average power spectral density of a white noise source. This implies, INR is a measure of the relative strength of the strongest interferer compared to the noise power only (not compared to other interferers). This independence from other interferers makes it feasible to increase the INR by an artificial value for HPUE. Same is not true for DIP.

Proposal 1: Use INR-based modeling for requirement definition.

As we discussed in our previous contribution [3], it would be very low possibility to have 3dB or 7dB higher value for the strongest interferer (DIP1) over LTE profile. Further, in our previous simulations [4], a 7dB higher INR1 value is used for HetNet deployment to check the PC1 UE impact under an extremely worse condition. We have observed a degradation of ~3dB for 2Rx, ~ 2dB for 4Rx and ~1dB for 8Rx, in IRC performance. Therefore, HPUE will have much higher gain of MMSE-IRC over MMSE performance, which is enough to justify the requirements. To consider the worst case, we would prefer to consider only 7dB higher INR value.

Proposal 2: Use artificial increasing INR values with additional 7dB (for PC1 UE) in existing LTE profile.

Proposal 3: If INR value with additional 7dB is agreed for HPUE, then it may not be needed to test based on PC3 UEs.

Agreements:

Number of interferers

  - For 4Rx and 8Rx, consider 2 interferers.

Open Issue:

- Check the performance gain of 1 or 2 interferers for 2Rx and make decision based on simulation result

## From our simulations [5], we do not observe any significant gain of MMSE-IRC to the baseline MMSE receiver, for both HomNet and HetNet scenarios when number of interferers are increased from 1 to 2.

## Proposal 4: Consider 1 interferer for 2Rx case if HPUE not considered.

However, for HPUE case with 7dB higher INR value, the gain of MMSE-IRC to MMSE is expected to increase (~2-3 dB) as seen in our previous simulations [4]. This is sufficient to create a requirement.

Proposal 5: Consider 2 interferers for 2Rx in HPUE case.

## 2.2	Test parameters and simulation assumptions

Open Issue:

Test scope

- Proposals:

  - Option 1:

- 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

  - Option 2:

- 15kHz SCS FDD

- 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

  - Option 3:

- 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS FDD and TDD with pattern 7D1S2U, S=6D:4G:4U

- same requirements are applicable to both FDD and TDD with different UL-DL pattern.

Agreements:

  - Option 3 with adding note in the spec that ‘same requirements are applicable to both FDD and TDD with different UL-DL pattern’.

Regarding test scope, considering normal PUSCH requirements from TS 38.104 seems most reasonable to define requirements for Rel-19 BS MMSE-IRC. Aligning with TS 38.104 would be beneficial to maintain the specification and have the same test coverage.

Proposal 6: Use PUSCH requirements with transform precoding disabled from TS 38.104 to define the SCS and TDD patterns for BS MMSE-IRC.

- 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS FDD and TDD with pattern 7D1S2U, S=6D:4G:4U

- same requirements are applicable to both FDD and TDD with different UL-DL pattern.

Open Issue:

Channel bandwidth

- Proposals for CP-OFDM:

  - Option 1:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

  - Option 2: 10MHz/15kHz and 40MHz/30kHz for initial simulation purpose, additional channel bandwidth can be considered for requirement definition

  - Option 3: Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz

  - Option 4: Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz

  - Option 5: 5/20MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz

- Candidate options for DFT-s-OFDM:

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

  - Other options are not precluded

Agreements:

  - For CP-OFDM:

- For performance evaluation purpose, Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz.

- Further discuss the CHBW for requirement definition

As pointed by some companies and our previous contribution [3], the operating SNR for different channel bandwidths (CBW) do not have significant difference in performance of MRC vs IRC for CP-OFDM. Therefore, minimum CBW per SCS can be prioritized as done in many Rel-18 requirements.

Proposal 7: Prioritize minimum CBW for CP-OFDM, i.e 5 MHz for 15KHz SCS and 10 MHz for 30 KHz SCS to define BS MMSE-IRC requirements.

Open Issue:

Propagation condition

- Candidate options:

  - For serving cell:

- Option 1: TDLA30-10 and TDLC300-100 for HetNet and HomNet respectively

- Option 2: TDLA30-10 and TDLB100-100 for HetNet and HomNet respectively

- Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100

- Option 4: Use TDLB100-400

  - For interference cell:

- Option 1: Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000

- Option 2: Same propagation condition for serving and interference PUSCH

- Option 3: Use TDLC300-100

Following normal PUSCH requirement in TS 38.104, QPSK, 16QAM and 64QAM modulation schemes should be covered for the serving cell. Therefore, propagation conditions, i.e. TDLB100-400 Low, TDLC300-100 Low, and TDLA30-10 Low should be considered for both HetNet and HomNet, respectively. Most companies agree to follow legacy channel models for serving cell, but more discussion is required for interference cell.

Proposal 8: Consider TDLB100-400 Low, TDLC300-100 Low, and TDLA30-10 Low channel models for service cell in HetNet and HomNet, respectively.

Regarding interference cell, the interfering UE should have a higher delay spread to create interference to the serving UE even if interfering UE is at far cell-edge from the serving UE. But it is not necessary to create a new channel model, i.e. TDLC300-100 to achieve this from demodulation perspective.

Proposal 9: Do not consider TDLC1000 channel model for the interference cell.

From our simulation results [4][5], we provide a brief overview for 15 KHz SCS 5 MHz CBW case in Table 1 and 2. Similar observations were captured for 30 KHz SCS 10 MHz CBW case and with two interferers.

Table 1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS | FRC | SNR [dB] |  | Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  |  | IRC | MRC |  |
| 1 | 2 | TDLB100-100 |  | 9.83 | 2 | G_FR1_A3-8 | -0.47 | 3.33 | 3.8 |
|  |  |  |  | 5.36 | 2 | G_FR1_A3-8 | -1.78 | 0.10 | 1.8 |
|  | 4 | TDLC300-100 |  | 9.83 | 16 | G_FR1_A4-8 | 7.65 | 12.53 | 4.8 |
|  |  |  |  | 5.36 | 16 | G_FR1_A4-8 | 6.69 | 9.39 | 2.7 |
|  | 8 | TDLA30-10 |  | 9.83 | 20 | G_FR1_A5-8 | 4.92 | 9.69 | 4.7 |
|  |  |  |  | 5.36 | 20 | G_FR1_A5-8 | 4.79 | 7.04 | 2.2 |

Table 2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS | SNR [dB] |  | Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC |  |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 2 | -1.19 | 2.53 | 3.7 |
|  |  | TDLB100-100 |  | 5.36 | 2 | -1.77 | 0.02 | 1.7 |
|  | 4 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 5.73 | 11.13 | 5.4 |
|  |  | TDLB100-100 |  | 5.36 | 16 | 6.39 | 9.02 | 2.6 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.56 | 10.6 | 5.0 |
|  |  | TDLB100-100 |  | 5.36 | 20 | 6.08 | 8.43 | 2.3 |

Observation 3: The gain of IRC over MRC is almost similar while considering only TDLC300-100 channel for interference cell and choosing same propagation conditions for serving and interference cell.

Therefore, both option 2 and 3 can considered for interference cell propagation condition but it would be better to choose option 3 since we agree to introduce two sets of requirements for HomNet and HetNet separately, so different channel model for two deployments might be more suitable.  As for serving and interference cell, different delay spread would be more suitable.

Observation 4: It would be more suitable to use different channel model for different BS deployment scenarios.

Proposal 10: Consider different propagation condition for HetNet and HomNet deployments. As for serving and interference cell, consider different propagation channel.

Open Issue:

MCS

- Candidate options:

  - Option 1: Cover QPSK/16QAM/64QAM

- Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024)

- Option 1B: MCS index 2, 12, 20 in MCS table 1

- Option 1C: Cover MCS 2, 16, 20

  - Option 2: Choose only one MCS configuration

  - Option 3: Consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) and FFS 64QAM (ZTE)

As mentioned previously, QPSK, 16QAM and 64QAM modulations with corresponding channel models can be considered to keep the same test coverage as TS 38.104 normal PUSCH requirements. Therefore, option 2 is not feasible. Further, considering option 1A and option 3 would require defining new FRC tables which would incur extra test and simulation effort.

Observation 5: MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) would require defining new FRC tables in TS 38.104.

Observation 6: From our simulations [4][5], it is seen that target SNR for IRC (in table 1) for each MCS is feasible and in range with Rel-15 Normal PUSCH requirement (in table 3).

The results from IRC performance will be expectedly worse than Rel-15 normal PUSCH due to presence of interferers, but since the results in TS 38.104 are averaged based on the impairment value from companies, alignment is first required between companies.

For TDLC300-100, both MCS 12 and MCS 16 can be feasible. But MCS 16 would be a better choice to define the requirement since the results are aligned with Rel-15 normal PUSCH and the higher modulation can be tested.

Proposal 11: Take MCS index 2, 16, 20 in MCS table 1 for QPSK, 16QAM and 64QAM requirements.

Open Issue:

Waveform

  - Cover CP-OFDM and FFS DFT-s-OFDM

DFT-s-OFDM would have higher transmission power than CP-OFDM due to lower PAPR. It would lead to better SINR. From our simulations [5], we do not observe significant performance difference between CP-OFDM and DFT-s-OFDM. Therefore, only CP-OFDM would be enough to define the requirement.

Proposal 12: Only consider CP-OFDM for NR MMSE-IRC PUSCH requirements.

Open Issue:

DMRS configuration

- Candidate options on DMRS type:

  - Option 1: Cover DMRS configuration type 1 and type 2

  - Option 2: DMRS configuration type 1

- Candidate options on DMRS sequence generation:

  - Option 1:

- NID0=0, nSCID =0 for CP-OFDM

- NID0=0, group hopping and sequence hopping are disabled for DFT-s-OFDM

  - Option 2:

- nSCID =0 for all cells

- Different NID0=0 for serving and interference cells

DMRS Configuration Type define different patterns for placing DMRS within a Physical Resource Block (PRB). For BS MMSE-IRC, we can follow legacy normal PUSCH requirements in TS 38.104.

Proposal 13: Only consider DMRS configuration type 1 for NR MMSE-IRC PUSCH requirements.

Regarding DMRS sequence generation, nSCID (scrambling ID) is used to define variations in the DMRS sequence generation and allows different sequences on the same physical resources, which is useful in handling multi-layer transmissions in MIMO systems. From IRC receiver algorithm at BS demodulation branch, nSCID=0 for all cells is a suitable choice and aligned with current legacy requirements.

For NID0 (cell-specific ID), different values are reasonable to distinguish and ensure orthogonality of DMRS sequences between serving and interfering cells.

Proposal 14: Choose  nSCID=0 and different NID0 values for serving and interfering cells.

Open Issue:

Test requirement definition

- Proposals:

  - Option 1: Using SINR based requirement definition

  - Option 2: SNR based requirement definition

- Proposals on SINR definition:

  - Option 1:

- For BS type 1-C and BS type 1-H:

The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in the slot on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value.

- For BS type 1-O

The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in a slot on a RIB. N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a RIB. The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value.

As given in Sec 8.1 of TS 36.104 for LTE, both SNR and SINR based test requirements are defined because DIP-based interference modeling is used in LTE. For NR, we can follow INR-based interference modeling approach. Therefore, we consider SNR based requirement as currently existing in Sec 8.1 of TS 38.104.

Proposal 15: Use SNR based requirement as in TS 38.104 (Sec. 8.1).

# Conclusion

Based on the discussion, we made the following observations:

Observation 1: It is hard to judge that the DIP curve would be very different from LTE profile when HPUE is deployed which would be highly impacted by the HPUE percentage.

Observation 2: An alternate approach to system-level studies can be discussed to derive DIP/INR in NR due to HPUE.

Observation 3: The gain of IRC over MRC is almost similar while considering only TDLC300-100 channel for interference cell and choosing same propagation conditions for serving and interference cell.

Observation 4: It would be more suitable to use different channel model for different BS deployment scenarios.

Observation 5: MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) would require defining new FRC tables in TS 38.104.

Observation 6: From our simulations [4][5], it is seen that target SNR for IRC (in table 1) for each MCS is feasible and in range with Rel-15 Normal PUSCH requirement (in table 3).

Based on the discussion, we made the following proposals:

Proposal 1: Use INR-based modeling for HPUE case.

Proposal 2: Use artificial increasing INR values with additional 7dB (for PC1 UE) in existing LTE profile.

Proposal 3: If INR value with additional 7dB is agreed for HPUE, then it may not be needed to test based on PC3 UEs.

Proposal 4: Consider 1 interferer for 2Rx case if HPUE not considered.

Proposal 5: Consider 2 interferers for 2Rx in HPUE case.

Proposal 6: Use PUSCH requirements with transform precoding disabled from TS 38.104 to define the SCS and TDD patterns for BS MMSE-IRC.

o	15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

o	30 kHz SCS FDD and TDD with pattern 7D1S2U, S=6D:4G:4U

o	same requirements are applicable to both FDD and TDD with different UL-DL pattern.

Proposal 7: Prioritize minimum CBW for CP-OFDM, i.e 5 MHz for 15KHz SCS and 10 MHz for 30 KHz SCS to define BS MMSE-IRC requirements.

Proposal 8: Consider TDLB100-400 Low, TDLC300-100 Low, and TDLA30-10 Low channel models for service cell in HetNet and HomNet, respectively.

Proposal 9: Do not consider TDLC1000 channel model for the interference cell.

Proposal 10: Consider different propagation condition for HetNet and HomNet deployments. As for serving and interference cell, consider different propagation channel.

Proposal 11: Take MCS index 2, 16, 20 in MCS table 1 for QPSK, 16QAM and 64QAM requirements.

Proposal 12: Only consider CP-OFDM for NR MMSE-IRC PUSCH requirements.

Proposal 13: Only consider DMRS configuration type 1 for NR MMSE-IRC PUSCH requirements.

Proposal 14: Choose  nSCID=0 and different NID0 values for serving and interfering cells.

Proposal 15: Use SNR based requirement as in TS 38.104 (Sec. 8.1).

# References

[1] R4-2419329, “On general issues for MMSE-IRC receiver under interference”, Ericsson, RAN4#113, Orlando, Florida, USA.

[2] R4-2416548, “WF on [112bis][319] NR_demod_Ph5_Part1_General_BS”, China Telecom, RAN4#112bis, Hefei, Anhui, China.

[3] R4-2416091, “Discussion on Rel-19 MMSE-IRC BS demodulation requirements”, Ericsson, RAN4#112bis, Hefei, Anhui, China.

[4] R4-2416092, “Simulation results for Rel-19 MMSE-IRC BS demodulation requirements”, Ericsson, RAN4#112bis, Hefei, Anhui, China.

[5] R4-2419250, “Simulation results for Rel-19 MMSE-IRC BS demodulation requirements”, Ericsson, RAN4#113, Orlando, Florida, USA.

[6] RP-241297, “Revised WID on NR demodulation performance: Phase 5”, China Telecom, NTT DOCOMO, RAN Meeting #104, Shanghai, China,