3GPP TSG-RAN WG4 Meeting #113	R4-2419329

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.16.1

Source:	Ericsson

Title:	On general issues for MMSE-IRC receiver under interference

Document for:	Discussion

# 1	Introduction

In RAN#104 meeting, the revised WID on NR demodulation performance: Phase 5 was approved [1]. According to the WID, the following are objectives for UE and BS parts:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we provided our view on general issues for UE and BS demodulation requirement with interference.

# 2	Discussion

In this section, we discuss the general issues related to the UE performance requirements for 8Rx with interference and BS performance requirements with IRC.

## 2.1	UE performance requirements for 8Rx

During the last meeting of RAN4 #112bis in Hefei, companies have reached agreements on the issue of 8Rx UE MMSE-IRC receiver coverage:

8Rx UE MMSE-IRC receiver coverage Agreement: - Cover both Baseline 8Rx UE MMSE-IRC receiver and simplified 8Rx UE MMSE-IRC Receiver for the initial simulation analysis. - TBD: Criteria used to decide if one set of requirements can be introduced covering both receiver types. - Note: Test coverage shall cover both receivers

According to the agreements, companies are encouraged to provide simulation results for both baseline receiver and simplified receiver to better understand the performance gap so that to determine whether it is feasible to introduce one set of requirements.

We shared our results for the following cases (2 interference cells):

| Test num | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS (Table 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value 70% max Tput |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| Baseline 8Rx receiver |  |  |  |  |  |  |  |  |
| 1 | 1 | 10 / 15 | MCS17 | TDLC300-100 | 2x8, ULA Low | 8.65 | 11.07 | 2.42 |
| 2 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 7.26 | 13.22 | 5.96 |
| 3 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 9.21 | 14.47 | 5.26 |
| Simplified 8Rx receiver |  |  |  |  |  |  |  |  |
| 1a | 1 | 10 / 15 | MCS17 | TDLC300-100 | 2x8, ULA Low | 8.17 | 11.10 | 2.93 |
| 2a | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 9.07 | 13.25 | 4.18 |
| 3a | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 11.24 | 14.46 | 3.22 |

From the results above we can observe 1.81dB and 2.03dB performance difference comparing the baseline 8Rx receiver with the simplified 8Rx receiver.

If we look at the performance for Rel-18 8Rx PDSCH demodulation, around 2dB gap between the baseline 8Rx receiver and the simplified 8Rx receiver has been observed among different companies results, and it’s agreed to defined two sets of requirements to cover both receivers.

Observation 1: Around 2dB gap has been observed between the performance of the baseline 8Rx receiver and the simplified 8Rx receiver

Proposal 1: Define two sets of requirements for the baseline receiver and the simplified receiver respectively

2.2	BS performance requirements

Test scenarios

- On HomNet and HetNet scenarios:

  - FFS whether to introduce additional applicability rule.

s

As per [2], both Homogenous network (HomNet) and Heterogenous network (HetNet) scenarios are agreed to define the BS requirements for Rel-19 MMSE-IRC. Therefore, we could introduce manufactory declaration followed by appropriate applicability in Table 4.6 of TS 38.141-1 for BS type 1-C and type 1-H.

As introduced in LTE in TS 36.141 (Sec. 8.2.6.5), note has been added in the requirements table to only consider Wide Area BS for the enhanced performance PUSCH requirements with synchronous interference. A similar approach can be followed with Sec. 8 of TS 38.141-1 in NR for different BS classes (Home BS, Local Area BS and Wide Area BS) for both BS types.

Proposal 2	 Introduce new manufactory declarations and applicability rules.

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

8.1.2.1.x.x	Applicability of PUSCH with synchronous interference

Unless otherwise stated, PUSCH with synchronous interference requirements shall apply only for a BS declare to support MMSE-IRC receiver.

Proposal 3	 Adding a note to PUSCH with synchronous interference requirements to indicate HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS based on manufactory declaration D.2 in Table 4.6-1.

# 3	Summary

In this contribution, we shared our views on the general issues related to the IRC receiver under interference scenario. Our proposals are summarized as follows:

Observation 1: Around 2dB gap has been observed between the performance of the baseline 8Rx receiver and the simplified 8Rx receiver.

Proposal 1: Define two sets of requirements for the baseline receiver and the simplified receiver respectively.

Proposal 2:  Introduce new manufactory declarations and applicability rules.

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

8.1.2.1.x.x	Applicability of PUSCH with synchronous interference

Unless otherwise stated, PUSCH with synchronous interference requirements shall apply only for a BS declare to support MMSE-IRC receiver.

Proposal 3:  Adding a note to PUSCH with synchronous interference requirements to indicate HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS based on manufactory declaration D.2 in Table 4.6-1.

# References

[1]	R4-2416555: WF for [112bis][320] NR_demod_Ph5_Part2_UE, Nokia, RAN4 #112bis, Hefei, Anhui province, China

[2]	R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4 #112bis, Hefei, Anhui province, China