3GPP TSG-RAN WG1 Meeting #113	R4-2419345

Orlando, USA, November 18th – 22nd 2024

Agenda Item:	7.17.2.1

Source:	Ericsson

Title:	CSI compression evaluation of option 3 and option 4

Document for:	Discussion

# 1	Introduction

During RAN4#112bis, agreements were reached on data sharing and data formats for sharing models and datasets. Companies are invited to share datasets and models during the time around RAN4#113.

In addition, companies were requested to provide some additional simulation CDFs, and to provide views on how to proceed with option 4 and the commonality with option 3.

# 2	Simulation results and model selection for option 3 evaluation

According to the WF from RAN4#112bis, companies are requested to provide the following information in Excel form:

- CDF of SGCS per subband

- CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format)

In the Excel file contained within the .zip file of this TDOC, we provide this information. We provide both (i) a single CDF taking into account SGCS or Eigenvalue over all of the subbands and (ii) CDFs for each individual sub-band.

During RAN4#113, there will be a need to select which company’s models to select as example(s) for a test decoder. Having selected an example test decoder, the model will be shared with other companies and each company should train an encoder using the example test decoder and check whether they can observe similar performance to that obtained using their own decoder.

One or more example test decoder(s) need to be selected.

In principle, the need to select a model as the test decoder can be avoided by simply requesting every company to check they can create an encoder with acceptable performance using every other company’s decoder. However, since there are 15 models, this would cause a significant amount of work.

The number of selected test decoder(s) should not be so many that an excessively large amount of simulation effort is needed to compare all of them.

On the other hand, if a single decoder from one company is selected as the test decoder, and it happens that other companies cannot create a reasonable encoder based on the test decoder, this may not on its own disprove that option 3 is feasible. An alternative could be that the selected test decoder was for some reason a “bad” decoder that is not useful for creating encoders. To add some robustness to the conclusion, in the eventuality that the first selected test decoder does not work well then other test decoders should be attempted.

If other companies cannot train an encoder to interoperate with one example test decoder, this may not entirely disprove option 3. It may be that for some reason the test decoder is a “bad” decoder.

To facilitate a robust investigation, we propose that [4] companies decoders are selected with a priority order. Interested companies should check the highest priority decoder; if this works well then in principle no need for further simulation. However, if the highest priority test decoder cannot be used to obtain a good encoder then the second highest priority decoder should be used, and so on.

List [4] example test decoders in priority order. Evaluation of option 3 can take place according to the flowchart below.

Even if a good encoder is obtained with the highest priority test decoder, if companies have time and resources, investigating the other decoders could be of interest, because it could add some insight as to whether the possibility of “good/bad” test decoders (i.e. some test decoders that cannot be linked to effective encoders) exist.

Companies are encouraged to consider several test decoders in order to increase knowledge on whether there is variation in performance depending on the selected test decoder.

In order to select/prioritize which decoders to take as test decoders, it is of interest to consider the CDFs of both SCGS and the largest Eigenvalues in the used training dataset. It may be desirable to determine the amount of convergence in the training dataset, and whether there are any “training dataset outliers”. This may be done by comparison of the CDFs, although it may be somewhat subjective. Selection of test decoders created using “training dataset outliers” should be avoided.

Based on largest Eigenvalue CDFs, identify if there are “training set outliers”. Aim to select test decoders based on training datasets that are not outliers.

For the SCGS, it may also be good to check for outliers. Outliers are not models with a significantly different mean SCS, but rather models for which there is significant deviation in the shape of the SCGS CDF. Again, determining which models, if any, are outliers may be a subjective process.

Based on SCGS CDFs, identify if there are unusual or “outlier” CDFs. Aim to select test decoders from SCGS CDFs that are not outliers.

Once outliers have been excluded, the prioritization could be on the basis of mean SCGS. The rationale for this is (i) alignment has been achieved to the best extent possible and then (ii) it may make sense to select the model with the apparently best performance. The “gain over type II” should not be used, since there is significant variation between companies reported type II performance, and anyhow what is of most importance is the absolute performance.

After excluding outliers, prioritize decoders based on highest absolute [mean] SCGS.

# 3	Option 4 evaluation

Option 4 involves the specification capturing sufficient information to ensure that TE vendors (and other interested parties) can train test decoders that can interoperate with any encoder that is able to meet RAN4 requirements. This implies that a compliant encoder can interact with and be tested using any TE vendors decoder.

Two sub-options exist for option 4; capturing of a dataset in the specification (option 4a) or capturing of a reference encoder in the specification (4b).

To achieve option 4, it is at least necessary to ensure that the latent space is the same for each TE vendor. For option 4a, this means that the dataset needs to have both encoder input and encoder output.

For option 4b, there may be a question whether an encoder alone is sufficient or also a dataset needs to be standardized. If a dataset would be standardized, then option 4a and 4b would be almost the same (the encoder would simply be a way of adding latent space feature into the dataset in option 4b). So, to evaluate 4b as a different option, no standardization of the dataset should be assumed. However, parameters of the dataset may be standardized (such as a scenario for generating channel data).

The results from option 3 can be used for evaluating option 4a as follows:

- Identify one of the models from option 3 that works well, and capture the dataset, supplemented with the latent space feature, relating to that company’s model.

- Each company receives this dataset and creates a decoder (together with an associated encoder).

- Each company takes every other company’s decoder from step 2 and verifies whether their own encoder can interact with and achieve acceptable performance with every other company’s decoder.

  - This is modelling testing an encoder against different TE vendors decoders.

The results from option 3 can be used for evaluating option 4b as follows:

- Identify one of the models from option 3 that works well and share the reference encoder from that company.

- Each company receives the reference encoder and trains a decoder based on their own dataset.

- Each company takes every other company’s decoder from step 2 and verifies whether their own original encoder can interact with and achieve acceptable performance with every other company’s decoder.

  - Note that the original encoder is not the encoder received in step 2. It is the encoder that the company created from their own dataset using their own decoder when creating the results prior to the November meeting.

  - This is modelling testing an encoder against different TE vendors decoders.

For both option 4a and option 4b, there is an issue to discuss for step 2. When a decoder is created based on the dataset supplemented with the latent space feature (option 4a) or reference encoder (option 4b), should the “test” decoder use the agreed structure and training as used as input to the option 3 process and November meeting If the structure is re-used, this implies assuming standardization of a model structure (in addition to e.g. dataset supplemented with the latent space feature) as part of option 2a. It may even be worth to check both same structure and different structure, however there may be a large variety of alternative structure options.

RAN4 to discuss whether the “test decoders” generated for option 4 should assume the same structure and training as already agreed, or could have a different structure.

# Conclusion

In the previous sections we made the following observations:

Observation 1	One or more example test decoder(s) need to be selected.

Observation 2	The number of selected test decoder(s) should not be so many that an excessively large amount of simulation effort is needed to compare all of them.

Observation 3	If other companies cannot train an encoder to interoperate with one example test decoder, this may not entirely disprove option 3. It may be that for some reason the test decoder is a “bad” decoder.

Based on the discussion in the previous sections we propose the following:

Proposal 1	List [4] example test decoders in priority order. Evaluation of option 3 can take place according to the flowchart below.

Proposal 2	Companies are encouraged to consider several test decoders in order to increase knowledge on whether there is variation in performance depending on the selected test decoder.

Proposal 3	Based on largest Eigenvalue CDFs, identify if there are “training set outliers”. Aim to select test decoders based on training datasets that are not outliers.

Proposal 4	Based on SCGS CDFs, identify if there are unusual or “outlier” CDFs. Aim to select test decoders from SCGS CDFs that are not outliers.

Proposal 5	After excluding outliers, prioritize decoders based on highest absolute [mean] SCGS.

Proposal 6	RAN4 to discuss whether the “test decoders” generated for option 4 should assume the same structure and training as already agreed, or could have a different structure.