3GPP TSG-RAN WG4 Meeting # 113							R4-2419368

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.21.3.3

Source: 	OPPO

Title: 	On intermediate UE RF requirements

Document for:	Approval

- Introduction

In RAN4#112-bis meeting, the intermediate UE RF requirements have been fully discussed. Many of the requirements have been left as TBD. It might be more appropriate to further discuss the requirements and if in the end no consensus is reached, then we can leave the requirement discussion further in the WI phase.

- Discussion

For the TX requirement, it has been captured in WF [1] as below table

| RF Requirement for A-IoT intermediate UE- TX part |  |  | Potentially needed or not |
| --- | --- | --- | --- |
| Maximum output power |  |  | YES, as RAN4#112 agreed |
| Output power dynamics | Minimum output power |  | NO |
|  | Transmit OFF power |  | TBD |
|  | Transmit ON/OFF time mask |  | TBD |
|  | Power control |  | NO |
| Transmitted signal quality | Frequency error |  | YES, as RAN4#112 agreed |
|  | Transmit modulation quality | EVM | YES |
|  |  | Carrier leakage | TBD |
|  |  | In-band emissions | TBD |
| Output RF spectrum emissions | Occupied bandwidth |  | YES, as RAN4#112 agreed |
|  | ACLR |  | Depends on the co-existence, as RAN4#112 agreed |
|  | Spurious emissions |  | YES, as RAN4#112 agreed |
|  | Transmit intermodulation |  | TBD |

For the transmit off power, it is to limit the power radiated out in the power-off state. Hence the requirement is needed.

For the transmit on/off time mask, we believe it is needed to guarantee the intermediate UE can transmit from on to off and off to on in certain time.

For carrier leakage and in-band emission, we believe it should be together considered with the co-existence simulation since we use the legacy NR UE in-band emission and this can be further discussed in the WI phase.

Proposal 1: To agree on the following table for TX requirements:

Table 1 Summary of other TX requirements’

| Transmit OF power | Needed |
| --- | --- |
| Transmit ON/OFF time mask | Needed |
| Carrier leakage and IBE | Needed to be further discussed together with co-existence |

3	Conclusions

In this paper, we give further discussion on intermediate UE RF requirement the observations and proposals are as below:

Proposal 1: To agree on the following table for TX requirements:

Table 1 Summary of other TX requirements’

| Transmit OF power | Needed |
| --- | --- |
| Transmit ON/OFF time mask | Needed |
| Carrier leakage and IBE | Needed to be further discussed together with co-existence |

4 References

[1] R4-2417088	WF on RF requirement impact of A-IoT, Huawei