3GPP TSG-RAN WG4 Meeting # 113												R4-2419369

Orlando, US, 18th – 22nd November, 2024

Source: 	OPPO

Title: 	TP on the device 2b RF requirement

Agenda Item:	7.21.3.2

Document for:	Approval

- Introduction

In the previous meeting, the RF requirements for the device 2b were discussed, and this contribution summarize the conclusion so far.

- Text proposal

## <<Start of Change>>

6.7.4.3	Device 2b

6.7.3.1 TX

Maximum output power

In the transmission direction (D2R), for device 2b, as it can generate the signal by itself, the device function can be more similar to legacy IoT devices than backscattering-type devices. Therefore, its maximum output power and the corresponding power class can also be specified similarly to legacy IoT devices,

However, due to the nature of energy harvesting, its transmission and reception duty cycle may be limited by the availability of the harvested energy. Moreover, the power constraint on the AIoT devices may further impact the choice of components, e.g., oscillator, amplifier, etc.; thus, a significantly lower performance/capability can be expected.

Output power dynamics

-Transmit OFF power

As device type 2b, assuming the device is fully charged, it can be foreseen that it will work similarly to legacy IoT devices. Therefore, the transmit OFF power should be specified for type 2b to ensure the power level when the RF part is turned off will not exceed a certain range.

- Transmit ON/OFF time mask

The time mask requirement is to test the time period between the RF part turned on and off. For A-IoT devices, this time period is not a crucial requirement and hence it is not needed.

- Minimum output power

As device type 2b, assuming the device is fully charged, it can be foreseen that it will work similarly to legacy IoT devices. Therefore, the minimum output power should be specified for type 2b to ensure the lower limit of the output power for coverage.

- Power control

As device type 2b, assuming the device is fully charged, it can be foreseen that it will work similarly to legacy IoT devices. Therefore, the power control should be specified for type 2b to ensure the output power range.

Transmit signal quality

- Frequency error

As device type 2b, assuming the device is fully charged, it can be foreseen that it will work similarly to legacy IoT devices. Hence the frequency error is required.

- Modulation quality requirements

TBD

- In band emissions

TBD

Output RF spectrum emissions

- Occupied bandwidth

The transmission bandwidth configuration defines the bandwidth of the R2D/D2R signal in baseband. For the R2D channel, one or multiple PRBs are under consideration in RAN1. For D2R channel, the transmission bandwidth of 15kHz or more is being discussed.

- SEM

TBD

- ACLR

TBD

- Spurious emissions

TBD

Unwanted emissions

TBD

Transmit intermodulation

TBD

The TX RF requirement applicability for device 2b is summarized as below:

Table 6.7.3.1-1 TX requirement summary

| A-IoT device TX (D2R) RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
|  |  | Device 2b |
| Transmit output power | Maximum output power /output power | YES |
| Output power dynamic | Transmit OFF power | YES, as RAN4#112 agreed |
|  | Transmit ON/OFF time mask | NO, as RAN4#112 agreed |
|  | Minimum output power | YES |
|  | Power control | YES, as RAN4#112 agreed |
| Transmit signal quality | Frequency error | YES, as RAN4#112 agreed |
|  | Modulation quality requirements | TBD |
|  | In band emissions | TBD. |
|  | Carrier leakage | YES, as RAN4#112 agreed |
| Output RF spectrum emissions | Occupied bandwidth | YES |
|  | SEM | TBD |
|  | ACLR | TBD |
|  | Spurious emissions | TBD |
| Unwanted emissions |  | TBD |
| Transmit intermodulation |  | TBD |

6.7.3.2 RX

Reference sensitivity

The receiver sensitivity is to test the receiver capability to certain performance metrics in a given lowest RX power level. For device 2b, this requirement will be left to be further defined in WI phase to consider the RF architecture and the exact RF component capability.

Maximum input power

TBD

ACS

The ACS requirements shall be derived based on co-existence simulation, and guard band/guard RB may also be defined. As the BB LPF is proposed, the ACS requirement is supposed to be essential to verify its RF performance

ASCS

The ASCS requirement is supposed to define the selectivity of the in-channel receiver. However as the channel bandwidth of the device is limited to only 1RB, the ASCS is not needed.

In-band blocking

TBD

Out-of-band blocking

TBD

Receiver intermodulation

TBD

Rx spurious emission

TBD

Spurious response

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained. In the A-IoT deployment, the CW signal is set at a certain frequency for energy harvesting and reflection. Hence the scenario for this test is not needed.

The RX RF requirement applicability for device 2b is summarized as below:

Table 6.7.3.2 -1 TX requirement summary

| A-IoT device- RX Requirement | Potentially needed or not |
| --- | --- |
|  | Device 2b |
| Reference sensitivity | YES, as RAN4#112 agreed |
| Maximum input power | TBD |
| ACS | Depend on co-existence study |
| ACSC | NO |
| In-band blocking | TBD |
| Out-of-band blocking | TBD |
| Receiver intermodulation | TBD |
| Rx spurious emission | TBD |
| Spurious response | NO, as RAN4#112 agreed |
| Interference rejection | TBD |

## <<End of Change>>

- References