# 3GPP TSG-RAN WG4 Meeting #113	R4-2419402

# Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on BS MMSE-IRC requirements with inter-cell interference

Source: 	Huawei, HiSilicon

Agenda item:	7.16.3

Document for:	Discussion

Background

In last meeting, WF R4-2413570 [1] was approved, one objective is related to BS MMSE-IRC requirements under interference scenario, this contribution provides our views.

- Discussions

Test scenarios

One issue is test scenarios, it’s FFS whether to introduce additional applicability rules:

- On HomNet and HetNet scenarios: - Cover HomNet and HetNet scenarios. - FFS whether to introduce additional applicability rule. - On scenario with unaligned TDD pattern: - For NR_demod_Ph5 WI, only consider interference model with aligned TDD pattern between cells.

We propose that RAN4 should reduce test numbers since difference between HomNet and HetNet scenarios is interference power level and MMSE-IRC performance gain can be observed for both scenarios. We don’t see any baseband processing difference when scenario is changed. Hence, we propose the following:

Proposal 1: RAN4 to introduce the applicability rules that only one of HomNet and HetNet scenarios is tested.

Interference profile

One issue is how to account for HPUE in existing LTE profiles:

Interference profile - On interference modelling: - Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH - On how to account for HPUE in existing LTE profiles: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - Proposals on how to account for HPUE in existing LTE profiles. - Proposal 1: Perform new system-level simulations with the inclusion of HPUEs - Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB

We don’t support introducing interference model for HPUE. As discussed before, the only change is interference power level. We don’t see it’s meaningful to discuss any other interference power level if existing values works for performance definition.

Proposal 2: RAN4 to reuse the same interference modelling As captured in B.6.2 in TS.36.104 and don’t introduce any other model.

Number of interferers

One issue is number of interference cells. Candidate options are:

Number of interferers - For 4Rx and 8Rx, consider 2 interferers. - Check the performance gain of 1 or 2 interferers for 2Rx and make decision based on simulation result.

In LTE, only 1 interferer was considered for 2Rx MMSE-IRC requirements definition, the reason is that 2 interferers make the 2Rx MMSE-IRC performance gain over MMSE unobvious.  We are think it’s reasonable to introduce 2 interferers for 2Rx.

Proposal 3: RAN4 to consider 2 interference cells for 2Rx/4Rx/8Rx

Channel bandwidth

One open issue is the channel bandwidth for performance definition:

- Agreement: - For CP-OFDM: - For performance evaluation purpose, Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz. - Further discuss the CHBW for requirement definition

For performance definition, we prefer to only consider minimum bandwidth for each SCS, i.e. 5MHz SCS for 15kHz SCS and 10MHz for 30kHz SCS. In Rel-15, performance with all bandwidth for each SCS have been verified, it’s expected to reduce the test cases of bandwidth for future Release, so, many Rel-18 requirements only cover minimum bandwidth such as ATG and enhanced DMRS, the approach of which should also be applied to Rel-19 MMSE-IRC requirements with interference cell considering bandwidth have negligible impact on performance.

Proposal 4: For performance definition, RAN4 to only consider minimum bandwidth for each SCS, i.e. 5MHz for 15kHz SCS and 10MHz for 30kHz.

Antenna configuration

We have following options for antenna configuration:

- Candidate options: - Option 1: Cover 2/4/8 Rx with 1Tx - Option 2: Consider only 4RX and 8RX gNB - Option 3: Cover 2/4/8 Rx with 1/2/4 Tx - Option 4: Prioritize 4Rx case especially in the case of 2 explicit interferers - Option 5: Prioritize 1Tx

For Tx, we prefer to only consider 1Tx since we already propose to reuse LTE profiles, otherwise, we have to run SLS to study the new interference profiles for 2Tx. For Rx, based on our simulation results, all performance gain is larger than 2dB even for 1T2R, which is enough, hence we prefer to cover 2Rx,4Rx and 8Rx.

Proposal 5: RAN4 to consider 1T2R, 1T4R, 1T8R for requirements definition.

Propagation condition and MCS

One issue is propagation conditions and MCS

Propagation condition - Candidate options: - For serving cell: - Option 1: TDLA30-10 and TDLC300-100 for HetNet and HomNet respectively - Option 2: TDLA30-10 and TDLB100-100 for HetNet and HomNet respectively - Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100 - Option 4: Use TDLB100-400 - For interference cell: - Option 1: Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000 - Option 2: Same propagation condition for serving and interference PUSCH - Option 3: Use TDLC300-100 MCS - Candidate options: - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20 - Option 2: Choose only one MCS configuration - Option 3: Consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) and FFS 64QAM (ZTE)

We prefer to select one propagation condition and one MCS to reduce the test cost. Based on our simulation results in clause 3, MCS16 is a good choice to guarantee the reasonable SNR gain and SNR operating range. Hence, we slightly prefer MCS16, as for propagation conditions, we prefer TDLB100-400, which is usual configuration for MCS16. As for propagation conditions for interference cell, we slightly prefer TDLC300-100.

Proposal 6: Only choose one MCS and one propagation conditions for requirements definition.

Proposal 7:  RAN4 to use MCS16, TDLB100-400 for serving cell and TDLC300-100 for interference cells

Waveform

We have following options for waveform:

- Agreement - Cover CP-OFDM and FFS DFT-s-OFDM

Based on our understanding, DFT-s-OFDM is rarely used in the real deployment, considering the timeline, we prefer to only focus on CP-OFDM.

Proposal 8: RAN4 to only consider CP-OFDM for requirements definition.

DMRS configuration

We have following options for DMRS configuration:

- Candidate options on DMRS type: - Option 1: Cover DMRS configuration type 1 and type 2 - Option 2: DMRS configuration type 1 - Candidate options on DMRS sequence generation: - Option 1: - NID0=0, nSCID =0 for CP-OFDM - NID0=0, group hopping and sequence hopping are disabled for DFT-s-OFDM - Option 2: - nSCID =0 for all cells - Different NID0=0 for serving and interference cells

For DMRS configuration, RAN4 always use DMRS configuration type 1 for performance requirements, we don’t see any reason to include DMRS type 2 in this topic. Furthermore, we only introduce Rank1, which means no performance difference between DMRS type1 and type2. For the DMRS sequence generation, serving cells and interference cells shall have different DMRS initial seed to guarantee the orthogonality of DMRS sequence. Hence, we propose the following: nSCID=0 for all cells, NID0=0 for serving cell, NID0=1 for interference cell1 and NID0=2 for interference cell2.

Proposal 9: RAN4 to only introduce DMRS type 1 and  configure nSCID=0 for all cells, NID0=0 for serving cell, NID0=1 for interference cell1 and NID0=2 for interference cell2.

- Simulation results

In this clause we provide our initial simulation results to check the MMSE-IRC performance gain over MMSE-MRC under following assumptions:

Table 3-1: Simulation assumptions

| Assumptions |  | Serving cell | Interference cell 1 | Interference cell 2 |
| --- | --- | --- | --- | --- |
| Bandwidth/SCS |  | 5MHz/15kHz | 5MHz/15kHz | 5MHz/15kHz |
| Duplex mode |  | FDD | FDD | FDD |
| MCS |  | 2,16,20 | 16QAM random symbols | 16QAM random symbols |
| Propagation conditions: |  | TDLC300-100 Low for MCS2 TDLB100-400 Low for MCS16 TDLA30-10 Low for MCS20 | TDLC300-100 Low | TDLC300-100 Low |
| Antenna configuration |  | 1T2R, 1T4R,1T8R | 1T2R, 1T4R,1T8R | 1T2R, 1T4R,1T8R |
| Interference model |  |  | DIP set1: DIP1=-1.11dB DIP set2: DIP1=-0.43dB | DIP set1: DIP2=-10.91dB DIP set2: DIP2=-13.78dB |
| DM-RS | DM-RS configuration type | 1 | 1 | 1 |
|  | DM-RS duration | single-symbol DM-RS | single-symbol DM-RS | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 | pos1 | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 | 2 | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB | -3 dB | -3 dB |
|  | DM-RS port | {0} | {0} | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=1, nSCID =0 | NID0=2, nSCID =0 |
| Receiver assumption |  | MMSE-MRC, MMSE-IRC | N/A | N/A |

The simulation results are captured in Table 3-2:

Table 3-2: Simulation results

| DIP Set | MCS | Antenna configuration | MMSE-IRC (SNR(dB)@70% of maxTP) | MMSE (SNR(dB)@70% of maxTP) | Gain (dB) |
| --- | --- | --- | --- | --- | --- |
| DIP Set1 | MCS2 | 1T2R | 0.5 | 2.8 | 2.3 |
|  |  | 1T4R | -3.3 | 0.0 | 3.3 |
|  |  | 1T8R | -5.3 | -1.9 | 3.4 |
|  | MCS16 | 1T2R | 15.0 | 16.9 | 1.9 |
|  |  | 1T4R | 8.8 | 12.3 | 3.5 |
|  |  | 1T8R | 5.4 | 8.9 | 3.5 |
|  | MCS20 | 1T2R | 15.3 | 17.3 | 2.0 |
|  |  | 1T4R | 10.2 | 13.5 | 3.3 |
|  |  | 1T8R | 7.3 | 10.5 | 3.2 |
| DIP Set2 | MCS2 | 1T2R | 3.0 | 6.9 | 3.9 |
|  |  | 1T4R | 0.2 | 4.2 | 4.0 |
|  |  | 1T8R | -0.8 | 2.4 | 3.2 |
|  | MCS16 | 1T2R | 17.4 | 21.1 | 3.7 |
|  |  | 1T4R | 10.0 | 16.3 | 6.2 |
|  |  | 1T8R | 6.1 | 12.9 | 6.8 |
|  | MCS20 | 1T2R | 17.4 | 21.2 | 3.8 |
|  |  | 1T4R | 11.5 | 17.3 | 5.8 |
|  |  | 1T8R | 7.9 | 14.0 | 6.1 |

- Conclusion

This contribution provides our views on BS MMSE-IRC requirements. The proposals and observations are:

Proposal 1: RAN4 to introduce the applicability rules that only one of HomNet and HetNet scenarios is tested.

Proposal 2: RAN4 to reuse the same interference modelling As captured in B.6.2 in TS.36.104 and don’t introduce any other model.

Proposal 3: RAN4 to consider 2 interference cells for 2Rx/4Rx/8Rx

Proposal 4: For performance definition, RAN4 to only consider minimum bandwidth for each SCS, i.e. 5MHz for 15kHz SCS and 10MHz for 30kHz.

Proposal 5: RAN4 to consider 1T2R, 1T4R, 1T8R for requirements definition.

Proposal 6: Only choose one MCS and one propagation conditions for requirements definition.

Proposal 7:  RAN4 to use MCS16, TDLB100-400 for serving cell and TDLC300-100 for interference cells

Proposal 8: RAN4 to only consider CP-OFDM for requirements definition.

Proposal 9: RAN4 to only introduce DMRS type 1 and  configure nSCID=0 for all cells, NID0=0 for serving cell, NID0=1 for interference cell1 and NID0=2 for interference cell2.

- Reference

[1]   R4-2416548 WF on [112bis][319] NR_demod_Ph5_Part1_General_BS