3GPP TSG-WG4 Meeting #113	R4-2419423

# Orlando, US, 18th - 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-2 | CR | 0029 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.115-2: Rated TRP output power limit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-11-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Table title for table 6.2.2.1-1 says it is for both repeater type 2-O and NCR type 2-O. It is wrong. EIRP limits for NCR-Fwd type 2-O are shown in table 6.2.2.1-2. Both rated TRP output power limits and rated beam EIRP output power limits for NCR- type 2-O for MT UL transmission are shown in table 6.2.2.1-2. However, subclause 6.2.2 is for OTA outpu power (EIRP). TRP output limits are specified in subclause 6.2.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the table title for table 6.2.2.1-1. Remove the TRP limit from table 6.2.2.1-2 and correct the corresponding text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EIRP limits and TRP limits for NCR-Fwd type 2-O UL transmission are specified in multiple places. It is confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2.2	OTA output power (EIRP)

6.2.2.1	Definition and applicability

Radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction.

For each declared beam, the requirement is based on declarations captured in clause 4.6 for a beam identifier (D.3), reference beam direction pair (D.6), rated beam EIRP (D.9) at the beam's reference direction pair, OTA peak directions set (D.7), the beam direction pairs at the maximum steering directions (D.8) and their associated rated beam EIRP and beamwidth(s) for reference beam direction pair and maximum steering directions (D.10).

For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the repeater is declared to radiate at the associated beam peak direction.

For each beam peak direction associated with a beam direction pair within the OTA peak directions set, a specific rated beam EIRP level may be claimed. Any claimed value shall be met within the accuracy requirement as described below. Rated beam EIRP is only required to be declared for the beam direction pairs subject to conformance testing as detailed in clause 6.2.4.

NOTE 1:	OTA peak directions set is set of beam peak directions for which the EIRP accuracy requirement is intended to be met. The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

NOTE 2:	A beam direction pair is data set consisting of the beam centre direction and the related beam peak direction.

NOTE 3:	A declared EIRP value is a value provided by the manufacturer for verification according to the conformance specification declaration requirements, whereas a claimed EIRP value is provided by the manufacturer to the equipment user for normal operation of the equipment and is not subject to formal conformance testing.

For passbands where the supported fractional bandwidth (FBW) is larger than 6%, two rated beam EIRP may be declared by manufacturer:

-	Prated,out,FBWlow for lower supported frequency range, and

-	Prated,out,FBWhigh for higher supported frequency range.

For frequencies in between FFBWlow and FFBWhigh the rated beam EIRP is:

-	Prated,out,FBWlow, for the output whose frequency is within frequency range FFBWlow ≤ f < (FFBWlow +FFBWhigh) / 2,

-	Prated,out,FBWhigh, for the output whose frequency is within frequency range (FFBWlow +FFBWhigh) / 2 ≤ f ≤FFBWhigh.

The repeater radiated transmit power requirements are specified at single-band RIB.

If beams have been declared equivalent and parallel (D.11, D.12), only a representative beam is necessary to be tested to demonstrate conformance.

The repeater rated beam EIRP output power for repeater type 2-O UL transmission shall be within limits as specified in table 6.2.2.1-1.

Table 6.2.2.1-1: Repeater rated beam EIRP output power limits for repeater type 2-O UL transmission

| Repeater class | Prated,p,EIRP |
| --- | --- |
| Wide Area | (note 1) |
| Local Area | ≤ + 55 + X dBm, Note 2 |
| NOTE1: There is no upper limit for the Prated,p,EIRP of the repeater type 2-O UL transmission. NOTE2: X = 10*log (ceil (passband bandwidth/100MHz)) |  |

There is no upper limit for the rated beam EIRP output power of NCR-Fwd type 2-O DL transmission.

The rated beam EIRP output power for NCR-Fwd type 2-O UL transmission shall be within limits as specified in table 6.2.2.1-2.

Table 6.2.2.1-2: Rated beam EIRP output power limits for NCR-Fwd type 2-O for UL transmission

| NCR-Fwd class | Prated,p,EIRP |
| --- | --- |
| Wide Area | (note 1) |
| Local Area | ≤ + 55 + X dBm (Note 2) |
| NOTE1: There is no upper limit for Prated,p,EIRP of the NCR type 2-O UL transmission. NOTE2: X = 10*log (ceil (passband bandwidth/100MHz)) |  |