3GPP TSG-RAN WG4 Meeting #113                                                                R4-2419433

Orlando, Florida, USA, 18th – 22nd November, 2024

Agenda item:          7.17.1

Title:	Discussion on general aspects for AI/ML air

WI/SI:	NR_AIML_air-Core

Release:	Rel-19

Source:	Korea Testing Laboratory

Document for:	Discussion

# 1. Introduction

This document based on WF[3] offers a comprehensive technical perspective on the additional conditions and associated IDs, as well as the stochastic performance variation of AI/ML where addresses the necessary signaling procedures and the critical aspects of post-deployment testing.

2. Discussion

2.1 Additional conditions and associated ID

The NW-side additional condition was discussed at the beam management, the positioning accuracy enhancement and CSI prediction. In the beam management discussion and the positioning accuracy enhancement, the content of NW-side additional condition was not reached the agreement. In response to   RAN2 Q2 of LS[1], it was discussed and not agreed whether or not NW-side additional condition can be represented by associated ID in beam management discussion. However, it was agreed to study the identification of a possible NW-side additional condition in CSI prediction for UE-sided model [Table 1]. The most important thing, still under discussion in RAN1 and well recognized in RAN4, is the key challenge of ensuring consistency between training and inference, which have been taking into account the associated ID, NW side additional condition and the existing assistance data IE.  It is our understanding that not all of these may be feasible for conformance testing. For example, there is Vendor-specific IE in TS 29.444 [Table 2], [1] where states “Vendor-specific IEs are defined to cover requirements and features not specified by 3GPP.” and “In a network with Vendor specific enhancements, unrecognized vendor specific IEs shall be handled as unknown optional IEs.” The associated ID for UE sided model can be directly linked to the NW-side additional conditions, which may have too many combinations for each AI/ML use case. Not all possible combinations are required to ensure a negligible degradation of the model generalization performance, although it is quite important to maintain consistency between training and inference.

Here is the concept of associated ID. AI/ML has the characteristics of overfitting and generalization. When the performance is excellent only on a specific data set and the performance deteriorates on other datasets, it may be overfitting. The characteristics of the generalization have the expected performance presented on various data sets while the generalization has limitations. In other words, it can be defined that its function can work well under a similar property environment between the training and the inference. This means that there is a range of the data set where the expected performance is guaranteed. Therefore, the set (range) of datasets including the generalization performance can be the requirement for the operation of the AI/ML use case, which can be called the associated ID (or Dataset ID). Similarly, it is possible to distinguish whether a particular dataset can provide the expected performance if it is related to NW-sided additional conditions called ID-A for example, which is the identification between gNB and UE, directly correlated with the generation of a particular dataset by configured and indicated under the name of ID-A that is the additional condition such as the same downlink spatial domain transmission filter, specific mobile environments, the gNB configuration, etc. It should be noted that it was agreed that additional conditions are not part of the UE capability for the AI/ML-enabled feature/FG and does not imply that additional conditions are necessarily specified. We can develop multiple AI/ML models based on the additional conditions or data sets and identify them according to the additional conditions or data sets that are independently separated.

In summary, it is said that “it does not imply that additional conditions are necessarily specified” and “information and/or indication on NW-side additional conditions is provided to UE” in Table 3. For AI/ML beam management conformance testing, the association ID is provided between a test setup(TE) and DUT, which means that the test setup can be configured based on the additional condition of the conformance specification. Our understanding it that there could be many combinations of both, and it is unclear how the scope of RAN4 conformance test could be adapted.

Observation 1: There are numerous combinations of NW-side additional conditions and UE-side additional conditions.

Observation 2: The additional conditions must be defined at least for the conformance testing when the associated ID is used between a test setup and a DUT.

Proposal 1:  It is necessary for RAN4 to study and discuss the NW-side additional conditions and the UE-side additional conditions at least for the conformance testing.

Table 1 : Capture the output of RAN1 #118bis

9.1.1 Specification support for beam management Agreement Answer to Q2 in R1-2407604 as below: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

9.1.2 Specification support for positioning accuracy enhancement Agreement For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data.

9.1.3 Specification support for CSI prediction Agreement For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered.

Table 2 : TS 29.244, Interface between the Control Plane and the User Plane Nodes

### 5.9 Usage of Vendor-specific IE Vendor-specific IEs are defined to cover requirements and features not specified by 3GPP. NOTE 1: When an IE is intended to be used by more than one vendor, the definition of the IE is encouraged to be specified by 3GPP to ease implementation and interoperability. NOTE 2: The PFCP entities can use Vendor-specific IE(s) in the PFCP message relevant to the PFCP Association Setup procedure to learn which vendor specific enhancements are supported by the peer. Vendor-specific IE may be sent with any PFCP message. Vendor-specific IE may be added directly to a PFCP message, or by embedding it into a grouped IE. In a network with Vendor specific enhancements, unrecognized vendor specific IEs shall be handled as unknown optional IEs.

Table 3 : TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

### 4.2.3 Additional conditions For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. It does not imply that additional conditions are necessarily specified. Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions. Note: whether specification impact is needed is a separate discussion.

2.2 AI/ML Performance

From RAN4#112bis, R4-2417212[4] Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

At the last RAN4 #112bis meeting, it was discussed how to compare the legacy performance and AI/ML based performance when the overall performance of AI/ML has a stochastic variation, i.e. a certain range when there is a negligible degradation in AI/ML generalization. Therefore, the AI/ML based performance requirement should have a certain range of requirements that allows a negligible degradation, those relevant procedures can be feasible on the LCM operation. It also requires to consider or discuss what a negligible degradation is. Another issue is that presumably there could be multiple AI/ML models, we should evaluate the AI/ML performance on each AI/ML model, which have different performance depending on CPU, power consumption, a mobile environment, UE speed, etc.

Observation   3:  There could be multiple AI/ML models, we should evaluate the AI/ML performance on each AI/ML model, which have different performance depending on CPU, power consumption, a mobile environment, UE speed, etc.

Observation   4: The AI/ML performance provides the stochastic performance variation.

Proposal 2: Discuss how to evaluate the AI/ML performance when there are multiple models with different performance depending on CPU, power consumption, a mobile environment, UE speed, etc.

From our perspective, we would like to highlight the significance of the LCM function for conformance testing. It is presently understood to test the minimum performance of AI/ML functionality/feature, for example, the association ID #1 representing AI/ML beam management while together operating the LCM function such as the performance monitoring.

For gNB sided decision for the LCM operation, based on the results of the model monitoring from UE, gNB can decide on model updating/switching or fallback to legacy schemes. There are implementation areas such as the fallback performance thresholds that determine when and how gNB commands the fallback to the UE depending on the performance monitoring.  When configuring the conformance test environment, the test setup may decide that since gNB manages the instructions for the LCM operation, the test results are pass or fail based on the performance monitoring reports. In this case, the DUT would not fall back to the legacy scheme operation even if the performance is poor.

For UE sided inference, the LCM function, provided the performance monitoring and also selection, switch, activation, deactivation and fallback, can fallback to non-AI/ML function or a legacy algorithm during the conformance testing and re-activate the AI/ML functionality/feature, the association ID #1, or switch to another AI/ML physical model UE implemented under the scope of the identical AI/ML functionality, the association ID #1. Of course, this circumstance means that UE sided inference and LCM are running. As we all know, the AI/ML performance provides the stochastic performance variation such as “AI/ML can achieve about 70%~80% beam prediction accuracy” in TR 38.843. This means that the AI/ML performance results can be defined within a certain interval the minimum requirements. In other words, it is a general way to make a decision by observing the average monitoring results whether the minimum performance of the AI/ML functionality/feature would be pass or fail. It could be an efficient way to consider separating the conformance testing processes for AI/ML functionality/feature as follows.

Firstly, during conformance testing when the LCM management is operating on the UE sided, performance monitoring and reporting is required but the transition to deactivation and fallback shall be avoided. Our understanding is that this can be configured by setting the threshold or the management of the fallback/deactivation function is set to a very low performance requirement (AI/ML test mode),in order to avoid the deactivation and the fallback.

Secondly, it is the normal operation state that the conformance test performs AI/ML functionality/feature where the UE’s LCM management can report to gNB such as “Management Request” or “Management Decision Request” [38.843]. In this signalling proceudure, we can expect(inference) various outcomes. It simply passes if there is not a single “Management Request” or “Management Decision Request” such as fallback request during the conformance test. It might be acceptable if there is an occasional “Management Request” or “Management Decision Request” such as fallback.  It is inadequate if  “Management Request” or “Management Decision Request” is sent to gNB(Test setup) most of the time during the test.  It is difficult to decide whether the minimum requirement has been met as the AI/ML performance provides the stochastic performance variation in case of almost full of signalling procedures. However, there is the monitoring function that we can utilize as shown in the first scenario. Namely, the first setup is more efficient process without wasting test time and cost.

Observation 5: It has different behavior to verify the minimum performance of AI/ML functionality/feature depending on “Signalling procedures for model and functionality LCM” in TR 38.843.

Thirdly, to verify the function of LCM management, a separate or a fake AI/ML inference mimic environment.

In summary, RAN4 can do the conformance test in a way of efficiency way by using them if RAN1/RAN2 define the detail fallback and the deactivation mechanism considering on the view or opinion of RAN4’s. It should be discussed within RAN4 group for the way of efficiency conformance test by using the mechanism of the fallback / the deactivation if it might not be possible to define the mechanism in detail since the reason of the implementation area.

Proposal 3: It should be discussed within RAN4 group for the way of efficiency conformance test using the mechanism of the fallback / the deactivation if it might not be possible to define the mechanism in detail since the reason of the implementation area.

Proposal 4: It is efficiency way for the conformance test such as the performance test, except the protocol test, to decide one of between “Network decision, network-initiated AI/ML management (replace Network to TE)” and “UE decision, event-triggered as configured by the network (replace Network to TE)”

Table 4 Signalling procedures for model and functionality life cycle management [38.843]

| For example: Network decision, network-initiated AI/ML management (replace Network to TE) | For example: Network decision, UE-initiated AI/ML management(replace Network to TE) |
| --- | --- |
| For example: UE decision, event-triggered as configured by the network(replace Network to TE) | For example: UE autonomous, decision reported to the network(replace Network to TE) |

2.3 Post deployment testing

It is captured “Post deployment testing” from the WF on requirements for AI/ML air interface in Table 5.

Table 5 : [4]	R4-2417212, “WF on requirements for AI/ML air interface”

From RAN4#112bis, R4-2417212[4] Issue 1-2: Post deployment testing When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive: - Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model) - FFS whether validation can be done at the device along with inference for another active functionality(ies) - FFS on the feasibility - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models. - Other aspects not precluded. - Option 2: Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device. For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed. - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

As we understand, some of the gNB side additional conditions can indeed be specified, for sure there are often similar environments across different gNBs, but not all of the gNB side additional conditions are specified. From this perspective, it is essential to allow “Update or fine-tuning” based on field environments to ensure performance. The previous meeting clearly stated that both options should be included in RAN4 discussions, emphasizing not mutually excluding one in favour of the other.  Thus, the core specification work is needed to support those processes as depicted “Post-deployment validation due to model change/drift” in [RP-242399] . In our understanding, other WGs have been working on specifications for the LCM procedure where there is no discussion about the validation process when needing “Update or fine-tuning” based on field environments.

Regarding Option 1 ,   “FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model)”, the validation that occurs in the field is more appropriate due to the real field data for “Update or fine-tuning”.  This approach is preferable to laboratory conditions due to the limitations inherent in laboratory environments. It is therefore necessary to support an efficient process to aid validation, taking into account signalling overheads and time-consuming processes.

The third bullet for option 1: it could be possible but it is linked to the complexity and the capability for UE. It can be further discussed.

The fourth bullet for option 1 : It is possible but it needs further specification work to ensure the reliable and efficient validation process  for “Update or fine-tuning” in “Post deployment testing”.

The fifth bullet for option 1: capturing model inputs and, if necessary, other test data such as ground truth during conformance testing. This stored data can later be used to validate new or updated models.” This way seems somewhat reasonable because conformance requirements have provided reliability on legacy UEs. Even though, in our view, that approach can’t validate all AI/ML use cases. For example, while the Update or fine-tuning model can perform well against stored data, it may struggle in a field environment. This could be due to potential overfitting or issues with the dataset itself. Conversely, the AI/ML model which indicates “the Update or fine-tuning”, might excel in a field environment even if it does not perform as expected against the stored data. It could be possible that this situation occurs in the two-side model if compatible models can be allowed at NW and UE, namely there are not fully specified reference models.

Therefore, it is important to consider the validation process against field environments if discrepancies exist between the AI/ML model for the field environment and the one used for conformance testing.  By using the method in section 2.2 again, replacing TE to network, it can efficiently perform the "Update or fine-tuning" validation.

In our understanding, option 2 suggests that the validation process could be conducted through the LCM procedure in field/lab environments. If this is the case and also reducing the signalling overhead and time-consuming process, we believe this would be a desirable and efficient approach.  A more efficient procedure can be achieved by monitoring the function of the LCM in field environments, as controlling the threshold for the range of performance could reduce the time-consuming aspects of the process mentioned in section 2.2.

In summary, validating the process in the field/lab environments for checking whether or not changed functionalities through “Update or fine-tuning” can be made more efficient by managing the performance thresholds through the LCM.

Observation 6: Conducting the validation process through the LCM procedure is a desirable and efficient approach because “Update or fine-tuning” is for optimizing the performance against field environments.

In the following, it provides the overfitting problem and the necessity of the validation process.

Overfitting is a common problem in machine learning (ML) models, where the model performs well on training data but poorly on new, unseen data. To prevent overfitting, various methods can be used, such as early stopping, data augmentation, regularization, and modifying the ML model with techniques like adding dropout layers or batch normalization. From the RAN4 perspective, it is emphasized that data augmentation is crucial for controlling test environments, while other approaches should be considered by UE vendors.  Required efforts are made to configure models to reflect real-field environments for AI/ML use cases, however, it's important to note that in the conformance testing, RAN4 sets minimum requirements for each test case. While data augmentation is a powerful tool for mitigating overfitting, it may not completely prevent the issue in all cases when solely relied upon. Consequently, it is necessary to validate whether test configurations, especially those considering data augmentation, can effectively prevent overfitting. If there is uncertainty, other approaches should be taken into account.

It's important to recognize that an AI/ML model could exhibit poor performance in a field deployment condition due to overfitting and/or data drift, among other factors. While the test environment may have been designed to mitigate overfitting, it can still be challenging to completely avoid overfitting (local minima) in field deployment conditions. It is necessary to validate whether a DUT has the overfitting problem under the test configuration(setup) may not completely prevent overfitting. For example, if a DUT can be trained and validated under a test setup to check the overfitting problem, a test setup (a TE) can send a signal to a DUT to prohibit the LCM management of a DUT such as fallback/deactivation/switch(another AI/ML model) or set a very low threshold for fallback/deactivation/switch(another AI/ML model). In this configuration, while the validation process is running, a DUT can simultaneously report the performance monitoring result to a test setup(TE) and through this procedure(step) we can check if the overfitting problem exists.

Observation 7: Required efforts are made to configure models to reflect real-field environments for AI/ML use cases, however, it's important to note that in the conformance testing, RAN4 sets minimum requirements for each test case.

Observation 8: While data augmentation is a powerful tool for mitigating overfitting, it may not completely prevent the issue in all cases when solely relied upon.

Proposal 5: It is necessary to validate whether test configurations can effectively prevent overfitting. If there is uncertainty, other approaches should be taken into account.

Proposal 6: It is necessary to validate whether a DUT has the overfitting problem when test configurations may not completely prevent overfitting.

3. Conclusion

In this contribution, we provided our viewpoints on WF[3]

Observation 1: There are numerous combinations of NW-side additional conditions and UE-side additional conditions.

Observation 2: The additional conditions must be defined at least for the conformance testing when the associated ID is used between a test setup and a DUT.

Proposal 1:  It is necessary for RAN4 to study and discuss the NW-side additional conditions and the UE-side additional conditions at least for the conformance testing.

Observation   3:  There could be multiple AI/ML models, we should evaluate the AI/ML performance on each AI/ML model, which have different performance depending on CPU, power consumption, a mobile environment, UE speed, etc.

Observation   4: The AI/ML performance provides the stochastic performance variation.

Proposal 2: Discuss how to evaluate the AI/ML performance when there are multiple models with different performance depending on CPU, power consumption, a mobile environment, UE speed, etc.

Observation 5: It has different behavior to verify the minimum performance of AI/ML functionality/feature depending on “Signalling procedures for model and functionality LCM” in TR 38.843.

Proposal 3: It should be discussed within RAN4 group for the way of efficiency conformance test using the mechanism of the fallback / the deactivation if it might not be possible to define the mechanism in detail since the reason of the implementation area.

Proposal 4: It is efficiency way for the conformance test such as the performance test, except the protocol test, to decide one of between “Network decision, network-initiated AI/ML management (replace Network to TE)” and “UE decision, event-triggered as configured by the network (replace Network to TE)”

Observation 6: Conducting the validation process through the LCM procedure is a desirable and efficient approach because “Update or fine-tuning” is for optimizing the performance against field environments.

Observation 7: Required efforts are made to configure models to reflect real-field environments for AI/ML use cases, however, it's important to note that in the conformance testing, RAN4 sets minimum requirements for each test case.

Observation 8: While data augmentation is a powerful tool for mitigating overfitting, it may not completely prevent the issue in all cases when solely relied upon.

Proposal 5: It is necessary to validate whether test configurations can effectively prevent overfitting. If there is uncertainty, other approaches should be taken into account.

Proposal 6: It is necessary to validate whether a DUT has the overfitting problem when test configurations may not completely prevent overfitting.

4. Reference

- R1-2407640, “LS on applicable functionality reporting for beam management UE-sided model,” RAN1#118bis

- “Interface between the Control Plane and the User Plane Nodes,” 3GPP TS 29.244, V18.7.0(2024-09)

- R4-2417212, “WF on requirements for AI/ML air interface”