3GPP TSG-RAN4 Meeting # 113	R4-2419435

# Orlando, US,  18th August - 22nd Novemeber

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5204 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_enh-Core) Formal CR on Interruption time for hard satellite switch |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Erisson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - ssb-TimeOffset doesn’t need to be configured in hard switch. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove ssb-TimeOffset. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - May cause misunderstanding of applicability of requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1C.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of Change 1

##### 6.1C.3.2.2	Interruption time for hard satellite switch with re-sync

The interruption time is the time between t-service and the time when the UE is ready to receive any DL channel/signal and transmit any UL channel/signal from/to the target satellite .

When intra-frequency hard switch to NR SAN cell is commanded,

the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + Tprocessing  + T∆ + Tmargin ms

Otherwise, no interruption time requirement is applied.

Where:

-	Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when UE starts synchronizing with target satellite. If the target cell Es/Iot ≥ -2 dB, then Tsearch = [Tfirst_SSB] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	T∆ is same as the one defined in section 6.1C.1.2.2.1.

-	Tprocessing is time for UE processing. Tprocessing can be up to 10 ms.

-	Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

-	Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite.

UE is allowed to skip measurements for other cells and satellites than the target satellite and source satellite from T-service until the satellite switch completion.

End of Change 1