3GPP TSG-RAN WG4 Meeting #113	R4-2419444

Orlando, USA, 18th – 22nd, November 2024

Agenda Item:	7.20.3.2

Source: 	Ericsson

Title:	Discussion on RRM impacts for SBFD operations (SBFD)

Document for:	Discussion

# Introduction

The agreements on the SBFD CLI handling and SBFD operation during RAN4#112 meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

## 2.1 Other RRM core requirements for SBFD operation

### Impact on SSB based measurements

#### Issue 1-2-1: Requirements for SSB based measurement - Down-select one of the two options: - Option 1: - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. - Option 2: - For inter-frequency measurement without gap, UL transmission is prioritized. - Follow option 1 in other cases.

Based on the RAN1#117 agreements, SSB reception from the serving cell will be prioritized over any UL transmission when there is a collision.

Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission. - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs

RAN1 118 bis agreement

For SSB symbols configured with SBFD subbands, Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. Note: The SSB block is assumed to be within DL subband

When serving and neighbor SSB in time domain is fully overlapped in time domain, our understanding is the legacy TDD principle will be applied. The SSB is prioritized over UL transmission.

When serving and neighbor SSB are not fully overlapped in time domain. Due to the half-duplex UE cannot transmit and receive SSB at the same time. The collision between neighbor SSB and UL transmission within UL subband needs to be resolved.

For intra-frequency and inter-frequency without gap case, if the serving SSB symbol is configured within SBFD subband, the neighbor SSB will be in same frequency or very close in frequency with the serving SSB symbols. Even though the UL transmission within the UL subband can serving good UL traffic, based on the RAN1 latest agreement, only DL receptions within DL usable PRBs are allowed.

- Proposal 1: For SSB based L3 measurement on neighbor cell collision with UL transmission, existing requirements apply for SSB based neighbor cell measurement.

### Impact on CSI-RS based measurements

#### Issue 1-2-2: Requirements for CSI-RS based measurement #### Agreements (from online session) #### RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations. #### CSI-RS colliding with dynamically scheduled UL transmission #### CSI-RS puncturing in frequency domain in SBFD symbols #### CSI-RS distributed over SBFD and non-SBFD symbols #### Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI. #### RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

Firstly, to our understanding the RAN1 agreement for CSI-RS in regarding configuration is applicable for CSI-RS configured as NZP-CSI-RS specified within TS38.214 clause 5.2.2.3.1. This include the L1-RSRP measurement and CSI-RS based RRM measurements, RLM/BFD measurement which specified in RAN4 specification and normal CSI which typically specify in RAN1 specifications.

RAN1#117 several agreements were reached for CSI-RS:

For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. • Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only • Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols • FFS granularity of the configuration, e.g. per UE, per channel/signal etc. • FFS whether support of configuration 2 is subject to UE capability

For configuration 2, it remains for further study whether it is a separate UE capability, considering that within one measurement occasion, the CSI-RS may across the normal DL symbol and a SBFD symbol.

It is uncertain whether the UE can collect enough measurement samples due to the punctured CSI-RS. Also, it is unclear whether UE will be able to combine the CSI-RS results across two distinct symbols into one measurement.

- Observation 1: The configuration 2 which will causing CSI-RS distributed over both SBFD symbol and non-SBFD symbol. And it is unclear whether it is up to potential UE capability.

- Proposal 2 : RAN4 can initiate CSI-RS measurement impact discussion while wait RAN1 for further conclusion on whether configuration 2 will subject to UE capability.

However, we are noticing there is a similarity here for CSI-RS based measurement and UE CLI RSSI measurement. The only difference is the CLI RSSI measurement will only be performed within SBFD symbols.

For measurement type 2 (One CLI-RSSI measurement resource across two DL subbands) performed within SBFD symbols, the current agreement for UE-to-UE CLI measurement, the RSSI measurement can combine the 2 DL subbands into one wideband measurement report for at least method #1.

The primary intent of Type 2 measurement is to enable the UE to leverage all measurement resources simultaneously, thereby reducing measurement delay. We see the benefit of this type of measurement and would like to further discuss whether this can be supported for CSI-RS based measurement.

- Proposal 3: For CSI-RS based L1-RSRP and L3, RLM /BFD measurement, RAN4 can first agree on whether measurement across two DL subbands can be supported.

From measurement behavior perspective, the UE shall measure according to the configuration.

- For legacy normal DL symbols, the CSI-RS measurement shall be performed as the legacy behaviour.

- For SBFD symbols, RAN4 can further discuss whether the CSI-RS measurement can be performed in a similar way as the type 2 RSSI measurement.

The issue remains how UE can combine the measurement performed across different symbols. Since multiple samples are typically involved for L3 and L1-RSRP measurement, and 2 symbols are required for RLM/BFD measurement.

- Proposal 4:  RAN4 can further discuss how to update the measurement requirement once the measurement method is being agreed.

For CSI-RS colliding with the UL transmission (case 2), RAN1 reached agreement with the priority rules at RAN1#116.

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

As the RAN1 rules are currently based on serving cell conditions. To our understanding, when dynamic UL transmission is scheduled to be colliding with the semi static configured CSI-RS resources, the measurement occasion should be dropped.  This has potential impact on the existing measurement delay and accuracy requirement for CSI-RS based measurement.

- Proposal 5: For semi-static CSI-RS based measurements, in cases where collisions with uplink transmissions occur, UL transmission shall be prioritized.

- Proposal 6: When semi-static CSI-RS based measurements being dropped, RAN4 can further discuss how to relax the measurement requirement accordingly.

For the measurement being performed on the SBFD symbol, the CSI-RS may be punctured in frequency domain if it falls outside the usable DL PRBs, which may impact the CSI-RS-based measurement requirements.

For CSI-RS based RSRP measurement the requirement typically applicable when the bandwidth of CSI-RS is 48 PRBs and the density is 3.

Even if some PRBs containing the CSI-RS are punctured, if the total configured CSI-RS resources exceed the bandwidth threshold after punctuation, measurement accuracy should be maintained. This can be ensured through appropriate network configuration.

- Observation 2: For CSI-RS based L1 RSRP and L3 measurement, the transmitted bandwidth in frequency domain can be guaranteed by network configuration to avoid impact on the measurement accuracy.

Even with guarantee from the CSI-RS bandwidth through NW configuration, UE needs to combine and averaging different measurement across different symbols, take L1-RSRP measurement which using 3 samples as example, the scenario can become:

- 3 samples are all performed within non-SBFD symbols

- 3 samples are all performed within only SBFD symbols

- 3 samples are performed within both SBFD and non-SBFD symbols

To set requirements applicable for all 3 scenarios, one alternative is to consider the 3 samples are always performed with both SBFD and non-SBFD symbols and this is the worst measurement performance due to the complication in averaging.

RAN4 can introduce a new requirement as the minimum requirement for UE perform CSI-RS measurement when SBFD symbol is involved.

Alternative is to investigate based on all 3 possible scenarios and set requirements accordingly.

- Proposal 7: For CSI-RS based L1-RSRP and L3 measurement, RAN4 first agree on whether combine measurement results across different symbols can be supported.

- Proposal 8: For CSI-RS based L1-RSRP and L3 measurement, RAN4 further discuss how to set the requirement based on the results combination.

For RLM measurement, UE constantly uses BLER as a metric to indicate out-of-sync/in-sync status to higher layers. This threshold can be either from default configure#0 BLER 10% Qout and BLER 2% Qin or specific network configured thresholds for Qin and Qout with rlmInSyncOutOfSyncThreshold.

For RLM/BFD measurement, the CSI-RS bandwidth is typically at least 24PRBs and the density is 3. Also to notice the RLM evaluation period is typically longer in compare with BFD measurement.

For UE to qualify the In-Sync and Out-of-Sync, different evaluation metric is used.

- For Out-of-Sync, all the configured RLM-RS resources bellow Qout.

- For In-Sync, only one of the configured RLM-RS resources above Qin

By counting the number of successful and unsuccessful decoding attempts for both Qout and Qin, the UE estimates the radio link quality by determining the BLERin and BLERout and signals an out-of-sync and in-sync indication from L1 to the higher layers.

The BFD measurement works slightly different, since it only has the Qout threshold being compared, there are two parameters being configured from RRC layer. One is to decide how many times of Qout can be count as one beam failure instance. Another is the maximum BFI events UE can trigger beam failure recovery.

The legacy requirement also considers a BFD measurement restriction when the CSI-RS configured for BFD is on the same symbol which configured for as SSB for RLM, BFD, CBD or L1-RSRP measurement.

RAN4 specify evaluation period for RLM/BFD measurement have quite tight relationship with how the RLM/BFD measurement is shared with different frequency and Scells being configured. This evaluation period is specified on TS38.133 clause 8.1.3 and clause 8.5.3 respectively. The radio link monitoring and beam failure recover procedure is tightly based on how each UE evaluation is being performed.

If the RLM-RS or CSI-RS configured for BFD colliding with the UL transmission, this evaluation occasion will be dropped, this will both impact the beam level indication and the cell level evaluation.

If the RLM-RS or CSI-RS configured for BFD is across the SBFD symbols, if the CSI-RS resource bandwidth can be guaranteed, the evaluation of the occasion will be kept and whether this will impact evaluation accuracy still needs RAN4 further discussion.

- Observation 3: Measurement performed with SBFD symbols have different impact to RLM/BFD evaluation compare with L1-RSRP and L3 measurement.

- Proposal 9: RAN4 further discuss how to address the CSI-RS based RLM/BFD measurement based on whether colliding with UL transmission and whether across SBFD and non-SBFD symbols.

### Impact on other RRM requirements

#### Issue 2-3-5: Requirements for RACH requirements Recommended WF - RAN4 to discuss whether RACH requirements are impacted due to SBFD operation based on RAN1 agreements. #### Issue 2-3-6: Requirements for UL PC and or spatial relation update Recommended WF - RAN4 to discuss whether requirements for UL PC and or spatial relation switch are impacted due to SBFD operation based on RAN1 agreements. #### Issue 2-3-7: Requirements for MG and BWP switch Recommended WF - RAN4 to discuss whether to clarify the term “UL slot” in MG and BWP switch requirements when SBFD operation is enabled. #### Issue 2-3-8: Requirements for UL resource muting Recommended WF - RAN4 further to discuss whether additional RRM requirements would be defined for UL resource muting for PUSCH feature. #### Issue 2-3-9: Requirements for generic SBFD operation Recommended WF - RAN4 to discuss whether there is any RRM impact e.g. to scheduling restrictions/interruptions requirements due to SBFD operation.

Many of the listing issues are still waiting for RAN1 progress for example the RACH RO validation, the UL resources muting for PUSCH etc. We suggest RAN4 to wait for the further conclusion from RAN1 and decide whether there will further RRM requirement impacts.

For the UL slot definition, we do see there is a need to clarify the UL slot and the UL transmission within the SBFD slot. However, this wording update can wait until the CR phase.

- Proposal 10:For RACH impact, UL resources muting and UL spatial relation update, RAN4 confirm there is no impact on requirements.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: For SSB based L3 measurement on neighbor cell collision with UL transmission, existing requirements apply for SSB based neighbor cell measurement.

	Proposal 2 : RAN4 can initiate CSI-RS measurement impact discussion while wait RAN1 for further conclusion on whether configuration 2 will subject to UE capability

	Proposal 3: For CSI-RS based L1-RSRP and L3, RLM /BFD measurement, RAN4 can first agree on whether measurement across two DL subbands can be supported.

	Proposal 4:  RAN4 can further discuss how to update the measurement requirement once the measurement method is being agreed.

	Proposal 5: For semi-static CSI-RS based measurements, in cases where collisions with uplink transmissions occur, UL transmission shall be prioritized.

	Proposal 6: When semi-static CSI-RS based measurements being dropped, RAN4 can further discuss how to relax the measurement requirement accordingly.

	Proposal 7: For CSI-RS based L1-RSRP and L3 measurement, RAN4 first agree on whether combine measurement results across different symbols can be supported.

	Proposal 8: For CSI-RS based L1-RSRP and L3 measurement, RAN4 further discuss how to set the requirement based on the results combination.

	Proposal 9: RAN4 further discuss how to address the CSI-RS based RLM/BFD measurement based on whether colliding with UL transmission and whether across SBFD and non-SBFD symbols.

	Proposal 10:For RACH impact, UL resources muting and UL spatial relation update, RAN4 confirm there is no impact on requirements.

# References

- R4-2416858 “WF on SBFD RRM”, Huawei, October ,2024

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024