3GPP TSG-RAN WG4 Meeting #113 	R4-2419447

# Orlando, USA, 18th – 22nd , November 2024

Agenda Item:	7.24.2.3

Source: 	Ericsson

Title:	On general RRM aspects for Rel-19 conditional Intra-CU LTM

Document for:	Discussion

# Introduction

After RAN plenary in September , the following objective has been agreed in the WID [1] for Conditional LTM:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

For RAN4 112 bis meeting, many open issues are listed in the WF[2] In this contribution, we will address several aspects which are related to RRM requirements for conditional LTM.

# Discussion

## Scope of the RRM requirement for conditional LTM

Certain agreements have been reached during the RAN4 112bis meeting regarding the Rel-19 CLTM scope. One remaining issue is regarding the subsequent CLTM and how to specify RAN 4 RRM requirement for the subsequent CLTM.

Issue 1-1-4: Subsequent conditional LTM < Way Forward > FFS the following proposals: - Proposal 1: FFS whether there is any difference in the delay for initial and subsequent CLTM cell switch. - Proposal 2: RAN4 should further clarify the scenario for conditional LTM including subsequent LTM. - Proposal 3: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell. - Proposal 4: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Regarding potential differences in delays between the initial and subsequent CLTM, RAN2 are still discussing how the execution conditions for initial LTM and subsequent CLTM will be.

Our understanding is that the execution conditions will affect when the UE performs the subsequent CLTM. Also, if these conditions differ from the initial CLTM, this will impact the delay timeline.

As the use of L3 measurement for execution conditions is still under discussion, we cannot exclude this scenario from the cell switch delay requirements for subsequent CLTM. This will also influence the measurement and evaluation time related to the delay requirement.

Based on current agreements, the RAN2 direction is to specify subsequent CLTM conditions. However, details on execution conditions and whether L3 measurement will be included remain pending further progress in RAN2.

- Proposal 1: RAN4 specify subsequent C-LTM requirement.

- Proposal 2: Detail of starting point of the subsequent C-LTM can wait for RAN2 further progress.

- For general issue, we also would like to mention based on the RAN2 127bis meeting agreements:

- 5. RACH-less Conditional intra-CU LTM is supported. - 6. RACH based conditional intra-CU LTM is supported.

- This also reflected on one of the open issues within the WF [1].

Issue 1-4-1: Whether cover both RACH-based and RACH-less conditional LTM < Way Forward > FFS the following options: - Option 1: Cover both RACH-based and RACH-less conditional LTM. - Option 2: FFS whether both RACH-less and RACH-based CLTM are supported by RAN1/2.

- Proposal 3: RAN4 to specify requirement for both RACH based and RACH-less Conditional LTM.

## Conditional LTM delay

For Rel-18 LTM, the processing time has relation to the number of maximum candidates that can be supported for early ASN.1 decoding based on UE capability.

Issue 1-2-1: General aspects on Conditional LTM delay < Way Forward > FFS the following options: - Option 1: CHO requirements framework can be used as baseline. - Option 1a: Rel-18 LTM requirements framework can be used as references. - Option 2: Revisit conditional LTM delay when other WGs achieve further conclusions. - Option 3: Wait for RAN2 progress for defining cell switch delay requirements for conditional LTM. - Option 4: RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario. - Option 5: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM.

In Rel-18, RAN4 introduced a new UE capability for early ASN.1 decoding or fast RRC processing. If UE supports this capability, upon TCI state activation or PDCCH order based RACH trigger, UE can decode the ASN.1 for those cells and the RRC processing time for early ASN.1 is 0ms for these cells. The early decoding capability shall be supported for Rel-19 conditional LTM, the detail numbers can be further discussed upon RAN1/RAN2 agreements.

- Proposal 4: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM.

Another issue we would like to discuss is regarding the starting point of the conditional LTM delay.

Issue 1-2-2: The starting point of conditional LTM delay < Way Forward > FFS the following options: - Option 1: The starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM. - Option 2: The starting point of conditional LTM delay is the time when UE transmits an RRCReconfigurationComplete message. - Option 3: The starting point of CLTM cell switch delay depends on when the condition evaluation begins, which is up to RAN1/RAN2.

Base on the latest RAN2 127 bis meeting agreement:

1. Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

RAN2 has agreed that the starting point for the CLTM will be based on the RRC message. From the RAN4 requirements perspective, the starting point—at least for the initial CLTM—will occur when the UE receives an RRC_Reconfiguration message containing both the LTM candidate configuration and the corresponding execution conditions.

However, for subsequent CLTM, the triggering conditions and execution details are still unclear and under further discussion.

- Proposal 5: Rel-19 conditional LTM 1st time cell switch delay starting point shall be upon UE receiving the RRC_Reconfiguration message.

- Proposal 6: The starting point of the subsequent C-LTM shall wait for RAN2 further progress.

For the ending point of the conditional LTM, based on the RAN2 127 bis agreement

11. The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

This also clarifies the related issue for RAN4 discussion.

Issue 1-2-3: The ending point of conditional LTM delay < Way Forward > FFS the following options: - Option 1: - The ending point of conditional LTM delay is the time when the UE transmits the first UL message on the target cell. - FFS the support of RACH-based and/or RACH-less, which is up to RAN1/2 agreement. - It can be revisited if any technical issue will be identified considering the progress in other WGs. - Other option is not precluded.

To our view, the ending point of the conditional LTM shall follow the same definition of the Rel-18 definition which specified in TS 38.133 clause 6.3.1.2.

- Proposal 7: The ending point of the Rel-19 C-LTM both for the 1st time and the subsequent C-LTM shall be the time the UE transmits the first UL message on the target cell.

## Conditional LTM early synchronization and TCI state activation

Several agreements reached in regarding the early synchronization and TCI state activation.

7. UE based TA measurement mechanism is supported for conditional intra-CU LTM. 8. PDCCH ordered early TA acquisition is supported for conditional LTM. 9. Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

The current RAN2 agreement point out the baseline direction for early UL sync and early TCI state indication.

Issue 1-3-2: Early UL sync < Way Forward > FFS the following options: - Option 1: For UL-sync, we can consider some situations as follows, - Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0. - RAR-based TA acquisition. - UE based TA measurement. - Option 2: How/whether to support early RACH on candidate cells during conditional LTM. - Option 3: Whether updates are needed to the Rel-18 PDCCH ordered RACH delay requirements. - Option 4: If DL TODA-based UE autonomous TA estimation is found beneficial for the conditional LTM cell switch, RAN4 to define the requirements for DL TODA-based UE autonomous TA estimation. - Option 5: Wait for more progress from other WGs.

- Proposal 8: RAN 4 requirement assumption for the early UL sync shall start with PDCCH ordered early TA acquisition.

Another issue can be discussed is regarding the TCI state activation.

Issue 1-3-3: Early TCI state activation < Way Forward > FFS the following options: - Option 1: How/whether to support early TCI activation on candidate cells during conditional LTM. - Option 2: FFS whether updates are needed to the Rel-18 early TCI state activation delay requirements. - Option 3: R18 LTM early TCI state activation can be as baseline. - Option 4: Wait for more progress from other WGs.

- Proposal 9:  RAN 4 requirement assumption for the early TCI activation shall based on the Rel-18 TCI activation framework.

## Conditional LTM cell switch delay requirements components

The Rel-18 LTM, L1-RSRP measurements requirements on neighbouring cell was introduced. Further, L1 measurement-based event-triggered reporting is currently under discussion for another objective within the Rel-19 Mobility enhancements. As the conditional LTM is expected to execute the cell switch upon an event entering condition, RAN4 needs to define the measurement delay for the event triggered measurements which trigger execution of the conditional LTM.

Based on the RAN2 127bis meeting agreement

2.Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

At least LTM3 and LTM5 will be used for the C-LTM execution condition, however whether CHO conditions will be considered is still within RAN2 further discussion.

Though RAN2 did not conclude the discussion of the framework for CLTM configuration and execution, based on the existing agreements, we expect that CLTM procedure might include:

- UE receive of the conditional LTM configuration:

- UE measure and evaluate the conditions.

- UE receive TCI state activation.

- UE establishes DL/UL early sync with the Network.

- UE execute LTM and cell switch.

The delay component and definition have very tight relationship with RAN2 agreed procedure for example how subsequent CLTM will be triggered and executed, How the UE will measure and evaluate the conditions, and how UE will receive the TCI activation and establish DL/UL early sync. We think still good to wait for RAN2 further agreement for the delay component definition and the cell switch delay equation.

- Proposal 10: RAN4 to wait for RAN2 progress for defining cell switch delay requirements for conditional LTM.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: RAN4 specify subsequent C-LTM requirement.

	Proposal 2: Detail of starting point of the subsequent C-LTM can wait for RAN2 further progress.

	Proposal 3: RAN4 to specify requirement for both RACH based and RACH-less Conditional LTM.

	Proposal 4: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM.

	Proposal 5: Rel-19 conditional LTM 1st time cell switch delay starting point shall be upon UE receiving the RRC_Reconfiguration message.

	Proposal 6: The starting point of the subsequent C-LTM shall wait for RAN2 further agreement.

	Proposal 7: The ending point of the Rel-19 C-LTM both for the 1st time and the subsequent C-LTM shall be the time the UE transmits the first UL message on the target cell.

	Proposal 8: RAN 4 requirement assumption for the early UL sync shall start with supporting both UE based TA measurement and PDCCH ordered early TA acquisition.

	Proposal 9:  RAN 4 requirement assumption for the early TCI activation shall based on the Rel-18 TCI activation framework.

	Proposal 10: RAN4 to wait for RAN2 progress for defining cell switch delay requirements for conditional LTM.

# References

- [1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.

- [2] R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2