3GPP TSG-RAN WG4 Meeting #113	R4-2419453

Orlando, US, 18 – 22 November, 2024

Source:	Nokia

Title:	Discussion on Fragmented Carrier Study Item Description

Agenda item:	7.6.1

Document for:	Discussion

Introduction

At RAN#104 a new study item was agreed on addressing the issues raised by operators to optimize UE DL intra-band CA with fragmented carriers. The latest version of the SID is found in [1].

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements for study purpose are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

This contribution discusses the aspects of methods for reducing the number of UE Rx RF chains for Fragmented Carriers.

Agreements at previous meetings

At RAN4#112-bis the following were captured in the WF [2].

Agreement:

The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter

  - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers

  - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

Discussion

Following the agreements of the architecture and definition in RAN4#112-bis, we would like to revisit the study item description and have it use the terms of the agreements. In previous discussions and contributions, multiple terms/expressions have been used such as ‘UE Rx chain’/‘RF chain’/‘Rx chain’/‘Rx RF chain’. It was agreed that we use the term of the architecture as One Rx RF chain, which is operated in the state as Fully shared RF chain [2]. We therefore need to update occurrences of one RF chain (Rx omitted) or a Fully shared Rx chain (Rx used instead of RF) and similar terms, which no longer reflect the agreements and confuse the discussion on the reference architecture and the reference terminology.

TP with changes to the study item description

The proposed changes to the study item description are found in the following, that use the agreed definitions.

---  ---  ---  --- Start of TP ---  ---  ---  ---

# 3 Justification RAN4 has introduced numerous NR Carrier Aggregation (CA) configurations in past releases. Fragmented carriers in the same band consuming multiple Rx RF chains may limit the UE CA capabilities as the maximum supported number of component carriers (CCs) in a CA combination is limited by the number and capabilities of the Rx RF chains as well as the baseband processor capabilities in the UEs. This limitation prevents operators from fully utilizing their fragmented spectrum holdings in a single CA combination. A multi-company proposal was submitted to RAN#102 [RP-233374] and RAN4#110 [R4-2402309] indicating a potential solution that could have global impact. This study will investigate solutions to address the aforementioned issue, focusing on the use of fewer (shared) Rx RF chains than the number of carrier fragments in a band. # 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The objective of this study is as follows: - Identify methods for reducing the number of UE Rx RF chains (e.g. from separate Rx RF chains per CC to shared Rx RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering: - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - Other requirements are not precluded, if identified - The ability to semi-statically switch hardware resources (i.e. Rx RF chains) [RAN4, RAN2 – See note 2]; - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4]; - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]; - Impacts on DL performance [RAN4]; - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2]. NOTE 1: No RAN1 impact is foreseen NOTE 2: RAN2 work, if necessary, will be triggered by RAN4 LS NOTE 3: This study starts from single DL band. Sharing Rx RF chain is not considered among inter-band DL carriers. - Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

---  ---  ---  --- End of TP ---  ---  ---  ---

Agree the presented TP and update the SID accordingly to consistently use the term Rx RF chain.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following proposal was made:

Proposal 1: Agree the presented TP and update the SID accordingly to consistently use the term Rx RF chain.

- References

- RP-242404, Revised SID: Study on NR FR1 DL Fragmented Carriers, MediaTek Inc.

- R4-2417201, WF on NR FR1 DL fragmented carriers study, MediaTek Inc.