3GPP TSG-RAN WG4 Meeting # 113						R4-2419481

Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			Coexisting study simulation assumptions and result for A-IoT

Agenda item:		7.21.2.1

Document for:		Approval

# Introduction

In this paper, we discuss additional simulation assumptions and present simulation results for some of cases of coexisting for A-IoT.

# Discussion

The simulation cases are agreed in and for discussion purpose, it is quoted as below.

Issue 3-4-1: Evaluation cases for co-existence evaluation

| No. | deployment scenario and topology | spectrum | (Aggressor -> Victim) | note |
| --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR DL | R2D |
|  |  |  | NR DL->device | R2D |
| 3 (O) | D1T1-A2-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 4 (O) | D1T1-B-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 6-1 (O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 6-2 (O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |

## Simulation methodology

Issue 4-1-3: Rx for device 1 and 2a

Agreement in RAN4#111: Assume no frequency selectivity for co-existence evaluation for calibration purposes for device 1 and 2a. Agreement in RAN4#112: - Alt-1: Assume no frequency selectivity - Alt-2: SINR; R2D with LPF. Interference from NR; Frequency selectivity: [7.5 dB]. Noise bandwidth; [180kHz]. SINRR2D_with_LPF=Wanted_SR2D_180kHzNoiseFloor 180kHz+(InterferenceNR10MHz/LPF_Selectivitylinear). - Alt-3: For R2D receiver with LPF. RF BW; 10MHz. BB BW; [180kHz]. SINRi (linear, not dB); Input SINR at the antenna connector with noise/interferencde BW=RF BW. SINRo (linear, not dB); Output SINR after BB LPF assuming downconverted noise/interference BW=BB BW. SINRo=SINRi28*SINRi+4*BWRFBWBB. - Alt-4: other LLS assumptions are not excluded

Agreement:

- For R2D, SINR at antenna connector is used as the starting point, i.e interference and noise bandwidth is 10MHz.

RAN4 assume a 10 MHz noise bandwidth for R2D and in simulations as starting point, the SINR is calculated by integrating the NR interferer within the 10MHz. This is too pessimistic for RF ED receiver. Using this model will result in unrealistic coexisting simulation conclusion.

The NR signal interference with A-IoT signal is simulated using the simulation assumption in Table 1. in Figure 5, when there is no NR signal but the total transmitted bandwidth is 10MHz, the SNR threshold is 17.6 dB. And when a NR signal with x dB lower PSD than A-IoT signal is present, the SNR threshold will increase. Such SNR increase can be deemed as the noise increase from the NR signal interferer in receiver.

Assuming noise band of 10 MHz for R2D is not correct and not in line with the device 1/2a performance.

To factor in the noise increase in the SINR calculation based on the LLS results, the noise increase is listed in Table 1 for different PSD difference between A-IoT RB and NR RB. Such noise raise value can be added in the SINR calculated using the 180kHz noise bandwidth. With this, we suggest the procedure below for R2D SINR calculation:

Step 1: Calculate the SINR within the A-IoT transmission bandwidth (1 RB)

Step 2: Calculate the A-IoT signal power in linear scale and calculate the A-IoT signal PSD level within one RB (should be the same with A-IoT signal power as only one RB is transmitted in this case).

Step 3: Calculate the NR interference power P_NR in linear scale within the 10 MHz

Step 4: Calculate the PSD of NR signal within 1 RB:  P_NR /51

Step 5: Compare the PSD level difference in dB between PSD of NR signal in Step 4 and A-IoT signal power in Step 2.

Step 6: Look up for the noise raise value in dB in Table 1 referencing to the PSD difference in step 5

Step 7:  Calculate the new SINR value by subtracting the noise raise value in dB (Step 6) into the SINR in step 2

Step 8: Compare the SINR in step 7 to SNR threshold without the NR interference, if lower than SNR threshold, mark the device as outage.

In this approach, the RF ED noise caused by NR interferer is considered when PSD difference is lower than 11 dB, when the PSD difference between A-IoT signal and NR signal is higher than 11 dB, the signal is not decodable. Therefore, in such case, either a constant high noise factor is added in the SINR or just mark this device as outage. The device distribution with different PSD difference interfering situation is simulated and illustrated in Figure 6. It can be observed that most of devices (e.g 83%) are with PSD difference below 11 dB and decodable, but still around 17% of devices have higher interference situation and cannot decode the A-IoT signal.

Around 17% of devices will be outage due to the too high NR signal interference

RAN4 consider to use the above procedure to estimate the R2D SINR.

Table 1: noise increase due to NR signal interferer for a noise bandwidth of 180kHz RF ED receiver

| Case#1 | PSD difference | SNR threshold | Noise raise factor (dB) |
| --- | --- | --- | --- |
| 1 | -22 | 17,8 | 0 |
| 2 | -21 | 18 | 0,2 |
| 3 | -20 | 18,15 | 0,35 |
| 4 | -19 | 18,3 | 0,5 |
| 5 | -18 | 18,5 | 0,7 |
| 6 | -17 | 18,8 | 1 |
| 7 | -16 | 19,2 | 1,4 |
| 8 | -15 | 19,5 | 1,7 |
| 9 | -14 | 20,1 | 2,3 |
| 10 | -13 | 21,2 | 3,4 |
| 11 | -12 | 22,8 | 5 |
| 12 | -11 | 26,7 | 8,9 |

Figure 4: R2D detection in presence of the NR interfering signal

Table 2:Simulation parameters

BW=52 PRBs, only 1 PRB for A-IoT and 1 PRB guard each side

TBS= 20

OOk-1

No CFO/No SFO

ADC=8 bits

LPF after RF ED

Channel: TDL_A, 30ns delay spread

Detect_sync= false

Figure 5: BLER performance with 49 RB NR signal

Figure 6: The CDF of PSD difference between NR signal and A-IoT signal

# Conclusions

In this contribution, we present our view on the coexisting simulation with below proposal and observation:

Observation 1 Assuming noise band of 10 MHz for R2D is not correct and not in line with the device 1/2a performance.

Observation 2 Around 17% of devices will be outage due to the too high NR signal interference

Proposal-1: RAN4 consider to use the above procedure to estimate the R2D SINR.

# References

- R4-2417196, WF on Co-existence study for ambient IoT, CMCC