# 3GPP TSG-RAN WG4 Meeting # 113						R4-2419485

# Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			On WUR RF requirement testability

Agenda item:		7.23.2.4

Document for:		Approval

# Introduction

In this paper, we present our overview on the WUR RF requirement testability issue.

# Discussion

In WF[1], there are several issues relating to the testing aspects.

Issue 4-1-1: General framework on LP-WUS testing

Way forward: FFS

Issue 4-1-2: Separate RF test case for idle and connection mode

Way forward: FFS

Issue 4-1-3: How to get feedback from LP-WUR during the test

Way forward: FFS

Issue 4-1-4: detailed Test procedure

Way forward: FFS

Issue 4-1-5: UE test mode for LP-WUR

Agreements:

FFS A UE test mode for LP-WUR is needed

For RF testing, if one mode testing can fully test the noise figure, linearity, selectivity, there is no need to test is again in another mode. One company mentioned the AGC function may be trigged using different sync signal in idle and connected mode and more may need to understand on UE behaviour with latest RAN1 agreements.

According to latest Ran1 agreement below:

Agreement

- For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

  - LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

  - UE is configured with legacy C-DRX configurations as Rel-18

  - UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal

  - No impact on RRM/RLM/BFD measurement requirements is assumed

  - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP)

    - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

      - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification

And

Agreement

Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective,

- Opt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions

- Introducing monitoring window is not precluded

It can be observed that in connected mode, the LP-WUS periodicity is configured no larger than C-DRX cycle, meaning that MR may be waken up for other configured activities (RRM, RLM etc) and therefore, MR will be in sync and OOK WUR no need to have a separate sync signal during connected mode.  This also imply the OOK/OFDM WUR may not rely on the LP-SS signal to trigger the AGC, both LP-WUS or LP-SS could be used for AGC monitoring.

OOK/OFDM WUR at connected mode does not need additional sync signal given the current RAN1 agreement that LP-WUS periodicity is no larger than long C-DRX cycle.

Either connected or idle mode should be tested.

Some companies have concern on the testing time, mainly because the it may take long time from transmitting the LP-WUS till MR waking up and monitorring PDCCH. Though it is no agreed how to get feedback in issue 4-1-3, a generic testing framework should solve this issue, e.g a testing mode is designed by RAN5.  RAN4 may suggest RAN5 a new test mode but we think this can be decided by RAN5 and RAN4 can send LS to RAN5 to trigger the discussion.

Send a LS to RAN5 with text below:

In case a LS to RAN5, below LS text is proposed:

RAN4 are discussing the test metric for wake up receiver RF performance test. As the WUR only detect LP-WUS/LP-SS and no other signals, legacy throughout monitoring for PDSCH is not possible anymore. Therefore, RAN4 propose the test metric of Miss Detection Rate of LP-WUS (target 1%). To test MDR of LP-WUS, there are two options to do it and RAN4 agree that it will be up to RAN5 to decide which options is suitable for WUR receiver test.

Two options below to test

- Using the legacy paging procedure to detect successfully LP-WUS reception by WUR

- New test mode for testing the successfully LP-WUS reception by WUR

# Conclusions

In this contribution, we present our view on WUR RF testability with below proposal:

Observation 1 OOK/OFDM WUR at connected mode does not need additional sync signal given the current RAN1 agreement that LP-WUS periodicity is no larger than long C-DRX cycle.

Proposal-1: Either connected or idle mode should be tested.

Proposal-2: Send a LS to RAN5 with text below:

RAN4 are discussing the test metric for wake up receiver RF performance test. As the WUR only detect LP-WUS/LP-SS and no other signals, legacy throughout monitoring for PDSCH is not possible anymore. Therefore, RAN4 propose the test metric of Miss Detection Rate of LP-WUS (target 1%). To test MDR of LP-WUS, there are two options to do it and RAN4 agree that it will be up to RAN5 to decide which options is suitable for WUR receiver test.

Two options below to test

- Using the legacy paging procedure to detect successfully LP-WUS reception by WUR

- New test mode for testing the successfully LP-WUS reception by WUR

# References

- R4-2417112, WF on LP-WUS UE RF requirements, vivo

# Annex 1

3GPP TSG-RAN WG4 Meeting # 113						R4-241xyz

Orlando, US,18th–22nd November 2024

Title:	LS on WUR testibility

Response to:

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	RAN WG4

To:	RAN WG5

Cc:

Contact Person:

Name:	Chunhui Zhang

E-mail Address:	Chunhui (dot) Zhang (at) ericsson (dot) com

Attachments:

1. Overall Description:

RAN4 are discussing the test metric for wake up receiver RF performance test. As the WUR only detect LP-WUS/LP-SS and no other signals, legacy throughout monitoring for PDSCH is not possible anymore. Therefore, RAN4 propose the test metric of Miss Detection Rate of LP-WUS (target 1%). To test MDR of LP-WUS, there are two options to do it and RAN4 agree that it will be up to RAN5 to decide which options is suitable for WUR receiver test.

Two options below to test

- Using the legacy paging procedure to detect successfully LP-WUS reception by WUR

- New test mode for testing the successfully LP-WUS reception by WUR

2. Actions:

To RAN WG1/WG2 group.

ACTION: 	RAN4 would like RAN5 consider the information above.

3. References:

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #114		2025- 02‑17 to 205- 02- 21			Athens

TSG RAN WG4 Meeting #114-bis 	2025‑04‑07 to 2025‑04‑11				China