3GPP TSG RAN WG4 Meeting #114		 R4-2419501

# Orlando, Florida, USA, 18 – 22 November, 2024

Source:	Boost Mobile Network (former DISH Network)

Title:	Clarification for PC2/PC1.5 FDD+FDD request

Agenda item:			6.8.4

Document for:	Discussion

# 1. Introduction

This paper proposes for RAN4 to clarify how to address some configurations of PC2/PC1.5 2UL FDD+FDD requested by Boost Mobile Network.

# 2. Discussion

In RAN4#111, Boost Mobile Network (formerly known as DISH Network) requested PC2/PC1.5 2UL FDD+FDD band combinations [1] as described in table 1 below.

Table 1: PC2/PC1.5 FDD + FDD Requested Combinations from Boost Mobile Network

| Band combination | UL configuration | Power class |
| --- | --- | --- |
| CA_n66A-n71A | CA_n66A-n71A | 2Tx: PC3@n66 1Tx, PC3@n71 1Tx; CA power class PC2 2Tx: PC2@n66 1Tx, PC2@n71 1Tx; CA power class PC1.5 3Tx: PC3@n66 (UL MIMO 2Tx 20+20 dBm), PC3 n71 1Tx ; CA power class PC2 3Tx: PC2@n66 (UL MIMO 2Tx 23+23 dBm), PC2 n71 1Tx ; CA power class PC1.5 |
| CA_n70A-n71A | CA_n70A-n71A | 2Tx: PC3@n70 1Tx, PC3@n71 1Tx; CA power class PC2 2Tx: PC2@n70 1Tx, PC2@n71 1Tx; CA power class PC1.5 3Tx: PC3@n70 (UL MIMO 2Tx 20+20 dBm, PC3 n71 1Tx ; CA power class PC2 3Tx: PC2@n70 (UL MIMO 2Tx 23+23 dBm, PC2 n71 1Tx ; CA power class PC1.5 |

The requested combination cases are as follows:

- Total power PC2 2Tx with FDD PC3 1Tx + FDD PC3 1Tx

- Total power PC2 3Tx with FDD PC3 2Tx + FDD PC3 1 Tx

- Total power PC1.5  2Tx with FDD PC2 1Tx + FDD PC2 1Tx

- Total power PC1.5  3Tx with FDD PC2 2Tx + FDD PC2 1Tx

The following table 2 lists the cases of combinations that can be addressed in the current HPUE WID [2].

Table 2: Objectives addressed in the current HPUE WID

| Rel-19 High power UE (power class 1,5 and 2) for NR CA/DC combinations with/without NR SUL |  |  |
| --- | --- | --- |
| Obj. | Band combination list | Power class cases for uplink |
| 1 | High power UE (power class 1.5 or 2) for NR Intra-band Carrier Aggregation (CA) with high power on FDD or TDD band | 1UL(FDD): PC2 on FDD band 1UL(TDD): PC1.5 or PC2 on TDD band Note: single UL carrier for FDD, Single UL carrier or intra-band UL CA for TDD |
| 2 | High power UE (power class 1.5 or 2) for NR Inter-band Carrier Aggregation (CA)/Dual connectivity (DC) with/without Supplementary uplink (SUL) with high power on TDD band(s) Note: Including PC3 FDD/TDD+ PC3 TDD case | PC2 inter-band CA/DC: 1UL(TDD): PC2 on TDD band 2UL (TDD+TDD): PC3 or PC2 on TDD band 2UL (FDD+TDD): PC3 on FDD band, PC3 or PC2 on TDD band |
|  |  | PC1.5 inter-band CA/DC: 1UL(TDD): PC 1.5 on TDD band 2UL (TDD+TDD): PC3, PC2 or PC1.5 on TDD band 2UL (FDD+TDD): PC3 on FDD band, PC1.5 on TDD band |
|  |  | PC2 SUL band combinations with or without CA: SUL: PC3 or PC2 on SUL band NUL(TDD): PC2 on TDD band PC1.5 SUL band combinations with or without CA: SUL: PC3 or PC2 on SUL band NUL(TDD): PC1.5 on TDD band NUL = Normal Uplink in contrast to SUL. |
| 3 | High power UE (power class 2) for NR Inter-band Carrier Aggregation (CA)/Dual connectivity (DC) with high power on FDD band(s) Note: Including PC3 FDD+ PC3 FDD | 1UL(FDD): PC2 on FDD band 2UL (FDD+FDD): PC3 on FDD band |
| 4 | High power UE (power class 1.5) for NR Inter-band Carrier Aggregation (CA)/Dual connectivity (DC) with high power on both FDD and TDD bands | 2UL (FDD+TDD): PC2 on FDD band, PC2 or PC1.5 on TDD band |

From the table 2, it is understood that the requested case a and case b (2Tx/3Tx PC2 FDD+FDD) can be address in Objective 3. This request can be done using normal excel sheet.

Proposal 1: 	RAN4 to confirm that the following PC2 band combination can be requested as part of Objective 3 in the current HPUE WID.

| Band combination | UL configuration | Power class – PC2 |
| --- | --- | --- |
| CA_n66A-n71A | CA_n66A-n71A | 2Tx: PC3@n66 1Tx, PC3@n71 1Tx; CA power class PC2 3Tx: PC3@n66 (UL MIMO 2Tx 20+20 dBm), PC3 n71 1Tx ; CA power class PC2 |
| CA_n70A-n71A | CA_n70A-n71A | 2Tx: PC3@n70 1Tx, PC3@n71 1Tx; CA power class PC2 3Tx: PC3@n70 (UL MIMO 2Tx 20+20 dBm, PC3 n71 1Tx ; CA power class PC2 |

With regards to case c and case d, PC2/PC1.5 with FDD PC2 in one or both UL bands was never addressed in RAN4 in the past. Furthermore there might be concern that PC1.5 FDD+FDD would require 4Tx architecture. The requested case c and d are 2Tx/3Tx PC1.5 – FDD PC2+FDD PC2. But it is not clear whether these can be address as part of Objective 4 of this HPUE WID. We propose for RAN4 to discuss how to address this request and agree to include these combos to the WID as example combos.

Proposal 2: 	RAN4 to discuss and agree to include the following 2Tx/3Tx PC1.5 FDD+FDD band combinations in the current HPUE WID as example bands.

| Band combination | UL configuration | Power class – PC1.5 |
| --- | --- | --- |
| CA_n66A-n71A | CA_n66A-n71A | 2Tx: PC2@n66 1Tx, PC2@n71 1Tx; CA power class PC1.5 3Tx: PC2@n66 (UL MIMO 2Tx 23+23 dBm), PC2 n71 1Tx ; CA power class PC1.5 |
| CA_n70A-n71A | CA_n70A-n71A | 2Tx: PC2@n70 1Tx, PC2@n71 1Tx; CA power class PC1.5 3Tx: PC2@n70 (UL MIMO 2Tx 23+23 dBm, PC2 n71 1Tx ; CA power class PC1.5 |

# 3. Conclusion

With regards to the PC2/PC1.5 FDD+FDD band combinations requested in RAN4#111, the following are proposed:

Proposal 1: 	RAN4 to confirm that the following PC2 band combination can be requested as part of Objective 3 in the current HPUE WID.

| Band combination | UL configuration | Power class – PC2 |
| --- | --- | --- |
| CA_n66A-n71A | CA_n66A-n71A | 2Tx: PC3@n66 1Tx, PC3@n71 1Tx; CA power class PC2 3Tx: PC3@n66 (UL MIMO 2Tx 20+20 dBm), PC3 n71 1Tx ; CA power class PC2 |
| CA_n70A-n71A | CA_n70A-n71A | 2Tx: PC3@n70 1Tx, PC3@n71 1Tx; CA power class PC2 3Tx: PC3@n70 (UL MIMO 2Tx 20+20 dBm, PC3 n71 1Tx ; CA power class PC2 |

Proposal 2: 	RAN4 to discuss and agree to include the following 2Tx/3Tx PC1.5 FDD+FDD band combinations in the current HPUE WID as example bands.

| Band combination | UL configuration | Power class – PC1.5 |
| --- | --- | --- |
| CA_n66A-n71A | CA_n66A-n71A | 2Tx: PC2@n66 1Tx, PC2@n71 1Tx; CA power class PC1.5 3Tx: PC2@n66 (UL MIMO 2Tx 23+23 dBm), PC2 n71 1Tx ; CA power class PC1.5 |
| CA_n70A-n71A | CA_n70A-n71A | 2Tx: PC2@n70 1Tx, PC2@n71 1Tx; CA power class PC1.5 3Tx: PC2@n70 (UL MIMO 2Tx 23+23 dBm, PC2 n71 1Tx ; CA power class PC1.5 |

# Reference

- R4-2407575, Request for PC2/PC1.5 inter-band FDD-FDD UL CA configuration, DISH Network, RAN4#111, Fukuoka, Japan, May 20th – 24th, 2024

- R4-2412266, Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink, China Telecom, RAN4#112, Maastricht, Netherlands, 19- 23 August 2024