3GPP TSG-RAN WG4 Meeting #113	R4-2419509

# Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0050 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (IoT_NTN_FDD_LS_band) CR to TS 36.102 adding missing regulatory EIRP density requirements in clauses 6.2A.1 and 6.2B.1 for B254 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_LS_band |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The ETSI EN 301 441 regulatory EIRP-density requirements for B254 are missing in TS 36.102. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 6.2A.1, add the NS_04N/NS_05N EIRP-density requirements. - In clause 6.2B.1, add the NS_04N/NS_05N EIRP-density requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ETSI EN 301 441 regulatory EIRP-density requirements for B254 are not specified in TS 36.102. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1, 6.2B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TR 36.764... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================Start of the change 1==========================

6.2A.1	UE maximum output power for category M1

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.2A.1-1: UE Power Class

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | +/-2 | 20 | +/-2 |
| 255 |  |  | 23 | +/-2 | 20 | +/-2 |
| 254 |  |  | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2A.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2A.1-2: Additional requirements for transmit EIRP density

| EUTRABand | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| 254 | NS_04N | 1.4 | 1610 - 1618.25 | 27dBm/4kHz (mean EIRP limit) 15dBm/4kHz (peak EIRP limit) |
|  | NS_05N | 1.4 | 1618.25 - 1626.5 |  |

====================End of the change 1==========================

====================Start of the change 2==========================

6.2B.1	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |
| 254 | 23 | +/-2 | 20 | +/-2 |
| 253 | 23 | +/-2 | 20 | +/-2 |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2A.1-2 where channel bandwidth is replaced with 0.2MHz, and when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

====================End of the change 2==========================