3GPP TSG-RAN WG4 Meeting #113 	R4-2419514

Orlando, USA, November 18th – 22nd, 2024

Agenda Item: 7.20.3.1

Source: MediaTek inc.

Title: Discussion on SBFD simulation performance

Document for: Discussion

# Introduction

In the previous meeting RAN4 agreed to run simulation [1] to provide performance results based on the agreed simulation assumptions [2]. In this document, we provide our simulation performance.

# Simulation performance

Following the agreed simulation assumptions, we have performance based on the mandatory simulation assumptions:

- AWGN Channel

- 30kHz SCS

- 1dB SNR

- one L1 sample

- 48 PRBs

- 1 OFDM symbol

The simulation performance for SR-RSRP measurement accuracy (delta RSRP) is given below:

| Timing offset | 5% | 50% | 95% | Abs accuracy [dB] |
| --- | --- | --- | --- | --- |
| 1.67 us | - 0.68 | - 0.2 | 0.29 | 0.68 |
| 4.67 us | - 1.43 | - 0.89 | - 0.34 | 1.43 |

# Summary

Simulation performance is provided based on the agreed simulation assumptions.

# References

- R4-2416858, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Hefei, China, Oct 2024.

- R4-2416910, “Simulation assumptions for L1 SRS-RSRP measurement”,