3GPP TSG-RAN WG4 Meeting #113	R4-2419525

# Orlando, United States, 18 – 22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2593 | rev | - | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1: DL CA HPUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, AT&T, T-mobile, Samsung, Nokia, Ericsson, Boost Mobile Network, Vivo, Inmarsat, Viasat, Rakuten, UIC, Anterix |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Power_Limit_CA_DC-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 specifications include notes for band combinations where HPUE operation is applicable. This results in lower output power even if UE has the necessary HW to support higher output power, but specification process is not fully complete yet. This CR enables using HPUE for intra- and inter-band CA with single CC UL configured also for cases when HPUE applicability note is not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new general clause to CA to specify HPUE for intra- and inter-band CA with single UL CC configured. Add an exception to refsens to test exceptions with default power class for single CC UL with DL CA when MSD for higher power classes is not defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HPUE is not possible by UE indication for 1UL CC with DL CA for cases where HPUE note is not present in clause 5.5A. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.0 (new clause), 7.3A.2.1, 7.3A.2.2, 7.3A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

## 6.2A	Transmitter power for CA

### 6.2A.0	General

Maximum output power for intra-band and inter-band CA with single configured UL CC is not bounded by the power class specified in clause 6.2A.1. For intra-band and inter-band CA with single configured UL CC, PC2 and PC1.5 applicability note in clause 5.5A is not required to be in place and UE shall meet the requirements for the power class it indicates for that CA configuration. UE can indicate power class for the single configured UL CC with DL CA as specified in Table 6.2.1-1. If UE supports UL MIMO in this carrier, it can indicate power class for the CA configuration as specified in Table 6.2D.1-1.

NOTE:	The non-CA and CA configurations for which minimum requirements have been evaluated for Power Class 2 or Power Class 1.5 operation are indicated in clause 5.5A.

### 6.2A.1	UE maximum output power for CA

<End of change 1>

<Start of change 2>

## 7.3A	Reference sensitivity for CA

### 7.3A.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. For operations with 4 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,4R in Table 7.3.2-2 when MSD > 0.

For reference sensitivity exception test points where the specified carrier frequency does not correspond to a valid NR-ARFCN, the closest NR-ARFCN as specified in clause 5.4.2 applies.

For reference sensitivity level tests or reference sensitivity exception tests specified in clause 7.3A, SCS=15kHz based UL test configuration can be replaced by SCS=30kHz based UL test configuration. The equivalent substitution relationship between different SCS UL test configuration is shown in table 7.3A.1-1 for the operating bands above 2.2GHz.

Table 7.3A.1-1: Equivalent substitution relationship between different SCS UL test configuration

| SCS (kHz) | (BW[MHz], Lcrb) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | (5, 25) | (10, 50) | (15, 75) | (20, 100) | (25, 128) | (30, 160) | (35, 180) | (40, 216) | (45, 240) | (50, 270) |
| 30 | (10,12) | (10, 24) | (30, 36) | (20, 50) | (25, 64) | (30, 75) | (35, 90) | (40, 100) | (45, 108) | (50, 128) |

### 7.3A.2	Reference sensitivity power level for CA

#### 7.3A.2.1	Reference sensitivity power level for Intra-band contiguous CA

For intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, and Table 7.3.2-3.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. When UE supports other than default power class for a band combination of a single UL CC and DL CA configuration, and this higher power class is not applicable according to clause 5.5A.1, reference sensitivity and exceptions for reference sensitivity in accordance with clause 7.3A.4 and 7.3A.6 shall be tested with output power limited to default power class. If the higher power class supported by the UE is applicable according to clause 5.5A.1, the reference sensitivity and exceptions for reference sensitivity shall be tested with the higher power class.

For aggregation of two or more downlink FDD carriers with two uplink carriers, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.1-1 and the reference sensitivity power level increased by ΔRIBC. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.1-1: Intra-band contiguous CA uplink configuration for reference sensitivity

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBstart = 0) | 10 (RBstart = 42) | 30.8 | 26.1 | FDD |
| CA_n7B | 15/15 | 10MHz + 40MHz | 9 (RBstart = 26) | 36 (RBstart = 180) | 34 | 25 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |

#### 7.3A.2.2	Reference sensitivity power level for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, and Table 7.3A.2.2-1 with the reference sensitivity power level increased by ΔRIBNC  given in Table 7.3A.2.2-1 for the SCC(s).

For aggregation of two or more downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

When UE supports other than default power class for a band combination of a single UL CC and DL CA configuration, and this higher power class is not applicable according to clause 5.5A.2, reference sensitivity and exceptions for reference sensitivity in accordance with clause 7.3A.4 and 7.3A.6 shall be tested with output power limited to default power class. If the higher power class supported by the UE is applicable according to clause 5.5A.2, the reference sensitivity and exceptions for reference sensitivity shall be tested with the higher power class.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 50.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |
| CA_n66(2A) CA_n66(3A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 |  |  |  |  |  |  |

#### 7.3A.2.3	Reference sensitivity power level for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in  Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6.

When UE supports other than default power class for a band combination of a single UL CC and DL CA configuration, and this higher power class is not applicable according to clause 5.5A.3, reference sensitivity and exceptions for reference sensitivity in accordance with clause 7.3A.4 and 7.3A.6 shall be tested with output power limited to default power class. If the higher power class supported by the UE is applicable according to clause 5.5A.3, the reference sensitivity and exceptions for reference sensitivity shall be tested with the higher power class.

For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2.

#### 7.3A.2.4	Void

<End of change 2>