# 3GPP TSG-RAN4 Meeting #113	R4-2419526

# Orlando, United States, November 18 – 22, 2024

Agenda item:		9.2

Source: 		Qualcomm Incorporated, AT&T, T-mobile, Samsung, Nokia, Boost Mobile Network, Vivo, Inmarsat, Viasat, Rakuten, UIC, Anterix

Title: 		Support for HPUE with DL CA with single UL CC configured

Document for:		Approval

## 1	Introduction

As of today, RAN4 specifications explicitly indicate when HPUE is applicable for a band, as well as when HPUE is applicable to be used in CA configuration.

UEs having HPUE(=>26dBm) hardware support need to reduce power back to PC3(23dBm) in CA operation in case the HPUE basket work has not been completed. This may happen in DL CA with single UL carrier when simply configuring a second DL carrier can result in an output power drop of at least 3dB. This is illustrated in Figure 1.

Figure 1: In case HPUE is not explicitly specified for CA, UE needs to reduce output power and leave it’s HW capabilities unused.

## 2	Enabling higher output power with DL CA

Given that the issue exists already in closed releases, the best approach is to give an allowance for UE to exceed CA power class, and allow them to indicate higher output power up to the level supported for single carrier operation. When formulated as an allowance UE can take full advantage of the higher output power capability, but also, the divergence from existing RAN4 HPUE introduction as well as regular UE power class indication is minimized this way.

Network needs to be made aware of UE power capabilities. Existing UE capabilities PowerClass and ue-PowerClassPerBandPerBC can be used to indicate maximum power for each individual band in a CA configuration. Therefore, UE can use either of these capabilities to unambiguously inform network of the used maximum output power without impact to single carrier (capability ue-PowerClass).

Proposal 1: Allow UE to maintain HPUE maximum transmit power capability in inter-band and intra-band DL CA with single configured UL carrier even when HPUE requirements are not yet completed for the CA configuration.

The pre-condition for indicating support for higher power class is that single carrier requirements are specified. The same applies in case UL-MIMO is supported. This avoids situations where e.g. A-MPR or single carrier Refsens requirements would be missing.

Proposal 2: For inter-band CA and intra-band CA with single configured UL carrier

- UE can indicate any power class specified for single carrier for the UL band.

- If UL MIMO is supported, UE can indicate any power class specified for MIMO with single carrier.

## 3	Handling of MSD requirements

One of the debated aspects in RAN4 have been the relationship of this allowance and MSD specifications, i.e. potential UE receiver desensitization due to higher output power. MSD requirements are based on an artificial worst case reference point to set minimum UE RF performance based on a specific channel bandwidth, center frequency and RB allocation, while in field operation different parameters can be used. It is rare that such worst case spectrum allocation is held by an operator and on top of that worst case scheduling occurs. The specifications should not penalize the performance of all the CA combinations due to some corner cases.

If there is still a concern on DL performance, network has means to limit the maximum output power used by configuring Pmax on the CC transmitting UL

Therefore, it is sufficient to maintain the existing MSD requirements as a reference point and specify that if HPUE requirements are not yet defined for the higher power classes, MSD shall be verified at default power class power level (e.g PC3).

Proposal 3: If HPUE requirements for single UL with DL CA are not specified, MSD is verified at default power class output power level.

There are also on-going discussion on how to handle the HPUE MSD framework overall, the options in most recent WF [1] lists options as follows

  - RAN4 specifies equations/simplified guidelines/lookup tables “LUT” that define the relationship between the default power class MSD and the HPUE MSD requirements to replace the existing HPUE MSD tables

  - Unified Excel tool of calculating HPUE MSD are being developed, to be published in 3GPP in excel format if agreement is reached

  - PC2/PC1.5 MSD are not specified, UE is verified only under default power class transmission

  - PC2/PC1.5 MSD is included only in TR, UE is verified only under default power class transmission

Some concerns were raised in previous meeting on allowing HPUE operation when there is no certainty on MSD levels for the higher power classes, resulting in potential issue when requirement is introduced. All of the options being discussed resolve this concern, as there either would be no PC2 or PC1.5 MSD at all, or there would be means to derive the MSD directly from PC3 MSD-value either via a table or publicly available excel.

Observation 1: All options under discussion in Rel-19 RF enhancements resolve the issue of not knowing what HPUE MSD level will be.

To reach a consistent treatment of CA combinations and simultaneously avoid undue delays in enabling to support for HPUE DL CA configurations, the right approach is to allow the UE support of HPUE DL CA configurations now. If Rel-19 WI agrees to not specify PC2/PC1.5 MSD at all, also HPUE DL CA configurations can be limited to be verified only under default power class. In case Rel-19 WI concludes on excel- or table-based approach, no further update is needed.

Proposal 4: If necessary, further adjust MSD requirements based on rel-19 WI conclusion:

- If Rel-19 WI concludes that no PC2/PC1.5 MSD is specified, also single UL with DL CA combinations can be limited to only be verified at default power class output power.

- If Rel-19 WI concludes that normative MSD for PC2/PC1.5 continues to be specified, then RAN4 shall keep testing said normative MSD.

Proposal 5: Endorse the companion draft CR in R4-2419525

## 4	Conclusion

Based on current RAN4 specifications UEs are not allowed to utilize their real maximum output power capability but instead need to artificially reduce output power back to default power class even when only single UL carrier with DL CA is used. This results in reduced maximum output power and therefore lost coverage and UL performance. Following proposals are made to allow UEs to take advantage of their higher output power capability even if the 3GPP process for the CA configuration is not yet fully completed.

On enabling higher UE output power with DL CA

Proposal 1: Allow UE to maintain HPUE maximum transmit power capability in inter-band and intra-band DL CA with single configured UL carrier even when HPUE requirements are not yet completed for the CA configuration.

Proposal 2: For inter-band CA and intra-band CA with single configured UL carrier

- UE can indicate any power class specified for single carrier for the UL band.

- If UL MIMO is supported, UE can indicate any power class specified for MIMO with single carrier.

On handling of MSD

Proposal 3: If HPUE requirements for single UL with DL CA are not specified, MSD is verified at default power class output power level.

Observation 1: All options under discussion in Rel-19 RF enhancements resolve the issue of not knowing what HPUE MSD level will be.

Proposal 4: If necessary, further adjust MSD requirements based on rel-19 WI conclusion:

- If Rel-19 WI concludes that no PC2/PC1.5 MSD is specified, also single UL with DL CA combinations can be limited to only be verified at default power class output power.

- If Rel-19 WI concludes that normative MSD for PC2/PC1.5 continues to be specified, then RAN4 shall keep testing said normative MSD.

Concluding RAN task

Proposal 5: Endorse the companion draft CR in R4-2419525

## 4	References

[1]	R4-2417098, “WF on HPUE RF requirements”, Samsung, Skyworks, Nokia