3GPP TSG-RAN WG4 Meeting #113									                        R4- 2419552

Orlando, USA, Nov. 18 – 22, 2024

Agenda item:			7.6.3

Source:	Ericsson

Title:	Discussion on the impact on UE RF requirements of fragmented carriers

Document for:	Approval

# Introduction

Fragmented carriers in the same band requiring multiple Rx chains may limit the UE CA capabilities if part of complex band combinations with many DL CCs combined with DL MIMO in fragmented operator spectrum holdings. A new RAN4 study item was approved in the RAN #104 meeting to investigate solutions to address this.

The objectives of the SI are as follows [1]:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In the last meeting, several agreements were captured in the following WF [2].

The following sections will discuss the impact on UE RF requirements of fragmented carriers.

# Discussions

## ACS and In-band blocking

To study the requirements for fragmented carriers, we will begin by reviewing the existing ACS and IBB requirements for non-contiguous CA. The current ACS and IBB have been defined based on the worst-case scenario for each carrier, and therefore, they should be used for various deployment situations.

- Existing ACS and in-band blocking requirements are based on worst-case scenarios for each carrier and apply to various deployment scenarios.

Figure 1 provides a visual representation of the previously described equation. ACS and In-band blocking (IBB) are located within the gap between two non-contiguous CCs, in which -56 dBm and -44 dBm are the interference powers of IBB in test cases 1 and 2, respectively.

Figure 1. Visual representation of the equation.

Consider n25 in Toronto as an example of an asymmetric fragmented spectrum [5]. For a single operator or two operators coordinated via RAN sharing or spectrum pooling. CC1: 1945-1970 MHz (25 MHz bandwidth), Gap: 1970-1980 MHz (10 MHz gap, potentially used by another operator or legacy system), CC2: 1980-1995 MHz (15 MHz bandwidth).

Figure 2. An example band combination of n25 in Toronto.

For the case in Figure 2, the ACS requirements apply but not the in-band blocking requirements, as according to the equations above, the ACS and in-gap IBB requirements are only defined and applicable if the gap width is large enough. A UE vendor can implement this configuration with a single Rx chain if the existing requirement is met: 30 dB for the 15 MHz carrier and 26 dB for the 25 MHz carrier. Should relaxed ACS conditions be specified for the gap to facilitate a single Rx configuration further, then this would require additional BC indication to distinguish this from a “standard” BC for which the existing requirements are met. The relaxed selectivity might not be accepted for all deployments. Configuration of the BC is then up to the network (i.e. the operator).

We note that the existing non-contiguous intra-band CA ACS requirements can be met with a good margin.

Relaxing the ACS and in/band blocking requirements could lead to increasing interference between adjacent fragments, potentially degrading overall network performance and user experience. Furthermore, existing UEs are designed to meet current requirements. Relaxing standards could create compatibility issues with the existing device ecosystem.

Rather than relaxing the RF minimum requirements for deploying the fragmented carriers, the focus should be on innovative receiver designs based on the existing requirements, leaving the implementation up to the UE vendor.

- Relaxing ACS and in-band blocking requirements could create compatibility issues with the existing device ecosystem.

- The existing non-contiguous intra-band CA ACS requirements can be met with a good margin.

- Maintaining the current ACS and IBB requirements is important to ensure UE selectivity. Implementing non-contiguous intra-band carriers with a single Rx chain is up to UE implementation.

## REFSENS

The reference sensitivity requirements of non-contiguous intra-band CA are defined as a function of the sub-block gap, which is the separation between two carrier components, independent of where these carrier components are located within the frequency band [7]. The existing REFSENS requirement was derived based on the worst-case scenario where the Tx-Rx frequency separation/ UL-DL gap was minimum, corresponding to minimal duplexer distance and the largest sub-block gap between 2 CCs. For the case of band n25, the minimum separation is 15 MHz, as shown in Figure 3.

Figure 3. Tx-Rx separation for band n25.

It was pointed out in [8] that several factors may impact the PCC and SCC DL sensitivity performance. i.e. PCC Tx nonlinearity, DL image falling into other DL CC, LPF attenuation, etc. Among these, LPF attenuation has been identified as the most crucial factor that may cause REFSENS degradation. Some companies are concerned that due to the wider bandwidth of LPF with the fully shared architecture, the attenuation might be poorer than that of partially shared architecture, which in turn increases the noise floor and eventually might saturate the ADC. It should be pointed out that the blocking performance is a trade-off between the level of LPF suppression and the ADC’s dynamic range, referred to as ENOB of the ADC. Furthermore, as stated in [9], mapping the agreed example bands for fragmented carriers to some intra-band contiguous CA bands, the maximum aggregated bandwidth is already as wide as the max DL band, as shown in Table 1.

Table 1. Example intra-band contiguous CA bands have maximum DL BW

| Intra-band contiguous CA bands | Maximum aggregated bandwidth (MHz) | BCS | Example bands for fragmented carriers |
| --- | --- | --- | --- |
| / | / | / | n2/n25, n26, n66 |
| CA_n3B | 75 | 4 and 5 | n3(FDD) |
| CA_n7B | 70 | 4 and 5 | n7(FDD) |
| CA_n41C | 190 | 4 and 5 | n41(TDD) |

Understandably, the LPF attenuation is determined by its filter order and roll-off frequency, and increasing the ENOB of the ADC might be challenging. However, they are implementation-dependent and might vary with different chipset vendors. The link budget analysis in [7] showed that with a reasonable LPF attenuation, the existing REFSENS could be maintained.

- The existing REFSENS could be maintained with a reasonable LPF attenuation.

Another question to consider is: If relaxed REFSENSE and other requirements are to be specified to facilitate a fully shared Rx chain configuration, an additional “fragmented” BC indication would be necessary to differentiate it from a “standard” BC, which complies with the existing requirements. For a UE supporting a fragmented carrier BC, the network would not be able to determine whether interference on the non-contiguous wanted carriers is due to a blocker in the gap or any other interference. The question is, how should the network handle this scenario?

- Signaling aspects should be discussed in conjunction with the minimum requirements for non-contiguous BC. It is uncertain how the UE would indicate the supported fragmented carrier using a single Rx chain architecture and the expected network actions in case of interference. It is not possible to distinguish interference from an in-gap blocker and other interference.

Relaxed requirements for the single-RX receiver may, therefore, lead to relaxed requirements in general for non-contiguous band combinations.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	Existing ACS and in-band blocking requirements are based on worst-case scenarios for each carrier and apply to various deployment scenarios.

Observation 2	Relaxing ACS and in-band blocking requirements could create compatibility issues with the existing device ecosystem.

Observation 3	The existing non-contiguous intra-band CA ACS requirements can be met with a good margin.

Observation 4	The existing REFSENS could be maintained with a reasonable LPF attenuation.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Maintaining the current ACS and IBB requirements is important to ensure UE selectivity. Implementing non-contiguous intra-band carriers with a single Rx chain is up to UE implementation.

Proposal 2	Signaling aspects should be discussed in conjunction with the minimum requirements for non-contiguous BC. It is uncertain how the UE would indicate the supported fragmented carrier using a single Rx chain architecture and the expected network actions in case of interference. It is not possible to distinguish interference from an in-gap blocker and other interference.

# References

- RP-241669, Study on NR FR1 DL Fragmented Carriers

- R4-2417201, WF on fragmented DL carriers study, Mediatek Inc.

- RP-180557, LS response on work to support IMT-2020/5G in the Transport Network

- TS. 38.133 v18.5.0

- TS. 38.101-1 v18.5.0

- RP-240503, Fragmented carriers in the DL

- R4-126713, Reference sensitivity for 1UL non-contiguous intra-band CA in Band 25, Qualcomm Inc.

- R4-2415360, Discussion on UE RF requirement impact for DL fragmented carriers, Murata

- R4-2416062, Views on FC RF requirement, CHTTL