3GPP TSG-RAN WG4 Meeting # 113												R4-2419554

Orlando, US, 18th – 22nd November, 2024

Source: 	Qualcomm Incorporated

Title: 	TP to 38.769 on co-existence simulation assumptions

Agenda Item:	7.21.2.2

Document for:	Approval

- Introduction

In previous meetings, the co-existence simulation assumptions were discussed and the agreements were captured in TR 37.769 v1.0.0. In this meeting we provide text proposal on co-existence simulation assumptions based on the latest agreements.

- Text proposal

## <<Start of Change>>

### 6.6.3	Co-existence evaluation assumptions

#### 6.6.3.1	Deployment

Simulation assumptions related to network layout is captured for D1T1 and D2T2 in Table 6.6.3.1-1.

Table 6.6.3.1-1. Deployment parameters for D1T1 and D2T2

| Parameter | D1T1 | D2T2 |  |
| --- | --- | --- | --- |
| Carrier frequency | 900 MHz (Band n8) |  |  |
| BW for NR | 10MHz with 15KHz SCS |  |  |
| BW for A-IOT system | 180kHz, for 15kHz SCS |  |  |
| Waveform (R2D) | OOK waveform generated by OFDM modulator |  |  |
| Waveform (CW) | Unmodulated single tone |  |  |
| A-IoT DL power control | No |  |  |
| A-IoT UL power control | No |  |  |
| Traffic model | Full buffer |  |  |
| Frequency reuse | 1 |  |  |
| NR BS deployment (outdoor), i.e. scenario 1-1 and 1-2 | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 A-IOT indoor scenario per sector the minimum 2D distance between macro BS and indoor factory center is set as 100m. Inter-NR BS distance: 750m NR BS height: 25m |  |  |
| NR BS deployment (indoor), i.e. scenario 2-1 and 2-2 | NR indoor BS deployed co-site with A-IoT indoor reader; NR indoor BS ISD as 20m; NR indoor Min BS-UE distance: 0m; NR indoor UE uniformly distributed. NR indoor BS Tx power assumed [24] dBm Antenna pattern of A-IoT reader is reused. The self-interference cancellation of R2D in UL for co-located scenario can be reported by interested company. |  |  |
| A-IoT reader deployment / Intermediate UE dropping | For D1T1-A2 and D1T1-B: 18 A-IoT readers on a square lattice with spacing D, located D/2 from the walls. L=120m x W=60m; D=20m Reader height = 8 m Room height = 10m 2 A-IoT readers are activated in one drop as baseline. Minimum distance between active readers: 60m as baseline | For D2T2-A2 and D2T2-B: The intermediate UEs selected from the fixed positions. L=120m x W=50m; D=20m Intermediate UE height = 1.5 m Room height = 3m Number of intermediate UE for simulation: 2 UE at one drop. Minimum distance between intermediate UEs: 60m as baseline |  |
| CW deployment | For D1T1-A2 and D2T2-A2, CW and A-IoT reader are collocated. For D1T1-B and D2T2-B, CW topology layout is the same as A-IoT reader. For each device, the nearest CW node will be activated during the simulation, and CW node is not co-located with any activated reader |  |  |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop Minimum distance between reader and device is 1m |  |  |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. For NR UE indoor, uniformly distributed, Option1: 10% indoor, 90% outdoor, Option2: 100% indoor UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |  |  |
| NR BS Inter-site distance | MCL of 70 dB |  |  |
| Pathloss model | PL= PLb+PLtw+PLin+N(0, σP2) - PLin = 0.5 * d2D-in where d2D-in is the distance to nearest factory/office boundary on the line between Tx and Rx point. Set maximum value of d2D-in as [25m] as optional |  |  |
|  | PLb : NLOS and LOS in TR38.901 NR BS – NR UE: Uma Outdoor NR UE – A-IoT reader/ device: Umi Device – A-IoT reader: InF-DH Indoor NR UE – device: InH-Office Indoor NR UE –A-IoT reader: InF-DH |  | PLb : NLOS and LOS in TR38.901 NR BS – NR UE: Uma Outdoor NR UE – intermediate UE/ device: Umi Device – Intermediate UE: InH-Office Indoor NR UE – device: InH-Office Indoor NR UE – intermediate UE: InH-Office |
| O2I penetration loss | High penetration loss as in TR 38.901 |  |  |

#### 6.6.3.2	NR BS/ A-IoT reader/ intermediate UE/ CW RF characteristics

The NR BS and A-IoT reader are defined for two different antenna configurations as illustrated in Table 6.6.3.2-1 and Table 6.6.3.2-2. Assumptions related to CW RF characteristics relevant for different deployment scenarios are captured in Table 6.6.3.2-4.

Table 6.6.3.2-1: NR BS RF parameters

| NR BS Parameter | Values for evaluation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss:15 |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |
| ACLR | Option 1: 30dBc, Option 2: 17dBc |

Table 6.6.3.2-2: A-IoT reader RF parameters

| A-IoT reader parameters | Values for evaluation |
| --- | --- |
| A-IoT reader total Tx power | 33 dBm |
| A-IoT reader receiver Noise Figure (dB) | 10 |
| A-IoT reader antenna gain (dBi) | 6 |
| ACLR | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB) |
| ACS | Same as legacy NR BS |
| Antenna configuration | Antenna Array Geometry：equals to omni-directional antenna pattern in GCG in horizontal Antenna pattern (horizontal): AHϕ=-min12ϕϕ3dB2,Am, φ3dB = 90°, Am = 15 dB Antenna pattern (horizontal): AVθ=-min12θ-90°θ3dB2,SLAv, θ3dB= 90°, SLAv = 15 dB Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss): 6 |

Table 6.6.3.2-3: Intermediate UE RF parameters

| intermediate UE parameters | Values for evaluation |
| --- | --- |
| Intermediate UE total Tx power (dBm) | Baseline: 23 Optional: Tx power control is allowed |
| Gain of antenna intermediate UE (dBi) | 0 |
| Intermediate UE receiver Noise Figure (dB) | 9 |
| Antenna configuration | Omni directional antenna |

Table 6.6.3.2-4: CW RF parameters

| CW parameters | D1T1 | D2T2 |
| --- | --- | --- |
| Tx power (dBm) | If UL spectrum is used, UE Tx power is assumed, i.e. 23dB If DL spectrum is used, A-IoT reader Tx power is assumed, i.e. 33 dBm | Intermediate UE Tx power is assumed. |
| Antenna gain | Same as A-IoT reader | Same as intermediate UE |

#### 6.6.3.3	NR UE/ A-IoT device RF characteristics

Assumptions relevant for modelling the NR UE and A-IoT device RF characteristics are captured in Table 6.6.3.3-1 and Table 6.6.3.3-2.

Table 6.6.3.3-1: NR UE RF parameters

| NR UE Parameter | Values for evaluation |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| Height of UE antenna (m) | 1.5 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

Table 6.6.3.3-2: A-IoT device RF parameters

| A-IoT device parameters | Device 1 | Device 2a |
| --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 | [0] |
| A-IoT device reflection （backscatter）loss (dB) | OOK: -6 dB | OOK: -6 dB |
| A-IoT device power gain of reflection amplifier (dB) | N/A | 10(M) |
| A-IoT Device receiver sensitivity (dBm) Use this value to determine whether device can camp on the cell. | -36 | [-45] |
| A-IoT device noise figure (dB) | 24 | [20] |
| Guard band | 0PRB | 0PRB |

## <<End of Change>>

- References

[1] 3GPP TR 36.942 v18.0.0