3GPP TSG RAN WG4 #113													            R4-2419555

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:		6.19.1

Source:				Inmarsat, Viasat

Title:	Input to UE RF requirements for NTN L-bands n253, n521, n250

Document for:	Discussion and Decision

# Background

The intent of this paper is to provide additional regulatory information pertaining to the new proposed bands n253, n251 and n250.

# Regulatory Information

This section describes the regulatory information associated with bands n253, n251 and n250. The bands are defined as follows:

Table 1 NR-NTN Proposed Bands

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n253 | 1668MHz – 1675MHz | 1518MHz – 1525MHz | FDD |
| n251 | 1626.5MHz – 1660.5MHz | 1518MHz – 1559MHz | FDD |
| n250 | 1668MHz – 1675MHz | 1518MHz – 1559MHz | FDD |

One important aspect to note is that the majority of the existing regulatory requirements for emissions for MSS bands, including in the L-band range, with the main exception being FCC Part 25, are specified as “radiated” requirements, in the form of EIRP limits.  We think that there is a need to define a general methodology for application of such radiated regulatory requirements to FR1 NTN specifications, in a way that does not create unnecessary burdens on the implementation, but ensures compliance to regulation in all of the operational scenarios.

Given that 3GPP is already in the process of defining radiated requirements, a possible approach would be the introduction of such requirements in the 3GPP TS 38.101-5 (and corresponding LTE 36.102) specs under a new section for OTA radiated requirements.  Whether there is a need for conversion of regulatory emission limits from radiated to conducted requirements should also be further discussed.

Observation 1: The majority of the existing regulatory requirements for emission limits in MSS bands, with main exception in the FCC Part 25, are specified as radiated requirements in the form of EIRP limits.

Proposal 1: 3GPP should identify the appropriate methodology to capture radiated regulatory requirements in the specification(s), in such a way that ensures compliance to regulations in all operational scenarios, but does not create unnecessary implementation burden.

Proposal 2: 3GPP is already in the process of defining FR1 OTA radiated requirements for UE.  A possible approach could be the introduction of these requirements under new sections on OTA radiated requirements in the corresponding UE RF specs.

Proposal 3: Whether there is a need for conversion of regulatory emission limits from radiated to conducted requirements should also be further discussed

## Protection of Radioastronomy Bands

A specific aspect of L-bands is the close proximity to spectrum allocated for radioastronomy purposes. The two radioastronomy bands of interest directly overlap with L-band and extended L-band respectively:

- 1660-1660.5 MHz

- 1668-1670 MHz

Observation 2: There are two radioastronomy bands in close proximity to the spectrum licensed for L-band which must be protected.

The locations at which these frequencies are utilised are recorded in the ITU master international frequency register.  The protection of these bands is stipulated in several documents.

### ITU-R M.1480

ITU-R Recommendation M.1480 [1] provides a general requirements framework for global mobile satellite services operation in bands between 1-3 GHz, including MSS L-bands.

These requirements include protections out-of-band for the Radio Astronomy frequencies in the 1660.5-1668 MHz.

The following tables (2a and 3a) show the protection limits outside the 1660.5 MHz range:

TABLE  2a (end)

| Frequency range (MHz) | Carrier-on |  | Carrier-off |  |
| --- | --- | --- | --- | --- |
|  | e.i.r.p. limit (dBW) | Measurement bandwidth (kHz) | e.i.r.p. limit (dBW)(1) | Measurement bandwidth (kHz) |
| 1 626-1 626.2 | – 47 to – 40(3), (4) | 30 | – 77 | 100 |
| 1 626.2-1 626.5 | – 40(4) | 30 | – 77 | 100 |
| 1 626.5-1 660.5 | (5) | (5) | (5) | (5) |
| 1 660.5-1 662.5 | (5) | (5) | (5) | (5) |
| 1 662.5-1 665.5 | – 60 | 30 | – 77 | 100 |
| 1 665.5-1 670.5 | – 60 | 100 | – 77 | 100 |
| 1 670.5-1 680.5 | – 60 | 300 | – 77 | 100 |
| 1 680.5-1 690.5 | – 60 | 1 000 | – 77 | 100 |
| 1 690.5-2 250 | – 60 | 3 000 | – 77 | 100 |
| 2 250-12 750 | – 60(6), (7), (8) | 3 000 | – 77 | 100 |
| (1) Peak-hold measurement techniques should be used in the bands 1 000-1 525 MHz and 1 559-1 626.5 MHz and from 1 662.5-12 750 MHz. These values should be at or below the values for the carrier-on state. (2) In the sub-band 1 573.42-1 580.42 MHz, the average measurement time is 20 ms. (3) Linearly interpolated (dBW) versus frequency. (4) The power limits specified in Table 2a in the 1 624.5-1 626.5 MHz band require further study. This study is important to determine whether less stringent limits may enhance spectrum efficiency and utilization immediately above 1 626.5 MHz. (5) The unwanted emissions in that frequency range are limited by § 3.2. (6) In the band 3 263.0-3 321.0 MHz the maximum e.i.r.p. in one, and only one, 300 kHz measurement bandwidth shall not exceed _ 38 dBW. Elsewhere in this band the power limit in Table 2a shall be applied. (7) In each of the bands 4 894.5-4 981.5 MHz, 6 526.0-6 642.0 MHz and 8 175.5-8 302.5 MHz the maximum e.i.r.p. in one, and only one, 300 kHz measurement bandwidth shall not exceed – 48 dBW. Elsewhere in this band the power limit in Table 2a shall be applied. (8) In the band 9 789.0-9 963.0 MHz the maximum e.i.r.p. in one, and only one, 300 kHz measurement bandwidth shall not exceed _ 59 dBW. Elsewhere in this band the power limit in Table 2a shall be applied. |  |  |  |  |

TABLE  3a

Limits for unwanted emissions within the 1 626.5-1 662.5 MHz band for MESs
with an e.i.r.p. less than or equal to 15 dBW

| Offset from the edge of the band of the nominated bandwidth (kHz) | Maximum e.i.r.p. (dBW) |
| --- | --- |
| 0-25 | 0 to – 15 |
| 25-125 | – 15 to – 50 |
| 125-425 | – 50 |
| 425-1 500 | – 50 to – 65 |
| 1 500-36 000 | – 65 |

### ETSI EN 301 681

ETSI EN 301 681 harmonized standard specifies requirements for the protection of radiastronomy bands. In section 4.2.6 of the document, the protection of radiastronomy services is mandated by disabling devices in the vicinity of radioastronomy stations recorded in the ITU master international frequency register.

Requirements are also set out for the activation of this disable function:

“An MES which is in the carrier-on state shall not continue transmissions for a period of time longer than 1s after receipt of a transmission disable command.”

The spectrum emissions when a transmission disable command is issued are dictated by the carrier-off state emissions mandated by the harmonized standard:

Table 2 EN 301 681 Carrier-off state requirements

| Frequency (MHz) | EIRP (dBW) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| 30 to 1 000 | -87 | 100 kHz | peak hold |
| 1 000 to 1 525 | -87 | 100 kHz | peak hold |
| 1 525 to 1 559 | -97 (see note 3) | 100 kHz | average |
| 1 559 to 1 610 | -70 | 1 MHz (see note 1) | average (see note 2) |
| 1 610 to 12 750 | -87 | 100 kHz | peak hold |
| NOTE 1: Measurement bandwidths less than 1 MHz are allowable provided the power in the narrower bandwidth is integrated over 1 MHz. NOTE 2: The average measurement method defined in clause 5.2.2.3 shall apply except that a measurement time of 20 ms shall be used in the sub-band 1 573,42 MHz to 1 580,42 MHz. NOTE 3: If MES antenna gain is higher than 8 dBi, the unwanted emission limit in the band 1 525 MHz to 1 559 MHz shall be considered to be -90 dBW in 100 kHz. |  |  |  |

The emissions limit for 1610-12750MHz captures the carrier-off requirements for protection of radioastronomy bands within 1660-1660.5MHz and 1668-1670MHz, mandating a limit of -87dBW/100kHz, measured in any time period as a peak hold measurement.

Observation 3: In ETSI EN 301 681, TX-off as a measure for protection of RA stations is considered sufficient assuming the carrier-off state requirement can be met.

The ITU M.1480 specifies a carrier off state level of -77dBW/100kHz

Observation 4: The ETSI EN 301 681 requirement for carrier-off state is less stringent than that of the ITU M.1480.

Comparing this carrier-off state with the 3GPP limit which for a 5MHz CHBW is -3dBW/Hz, the EN 301 681 limit is -27dBW/Hz and the ITU M.1480 limit is -17dBW/Hz.

Observation 5: The 3GPP carrier off state requirement is -3dBW/Hz, compared to ETSI EN 301 681 at -27dBW/Hz and ITU M.1480 at -17dBW/Hz.

### FCC and NTIA

Both the Federal Communications Commission (FCC) and the National Telecommunications and Information Administration (NTIA) recognize the importance of protecting Radio Astronomy services.

These focus on the following frequency range which is dedicated to Radio Astronomy and Radiosonde:

- 1660.5-1668 MHz

As well as on the following frequency ranges, which are shared between Radio Astronomy and the Mobile Satellite Services:

- 1660-1660.5 MHz

- 1668-1668.4 MHz

- 1668.4-1670 MHz

The FCC captures the following:

- 47 CFR 2.106(b)(376)(i):

  - 5.376A Mobile earth stations operating in the band 1660-1660.5 MHz shall not cause harmful interference to stations in the radio astronomy service.

- 47 CFR 2.106(b)(379)(i)

  - 5.379A Administrations are urged to give all practicable protection in the band 1660.5-1668.4 MHz for future research in radio astronomy, particularly by eliminating air-to-ground transmissions in the meteorological aids service in the band 1664.4-1668.4 MHz as soon as practicable.

- 47 CFR 2.106(b)(379)(ii)

  - In the band 1668-1668.4 MHz, Resolution 904 (WRC-07) shall apply.

- 47 CFR 2.106(b)(379)(iii)

  - 5.379C In order to protect the radio astronomy service in the band 1668-1670 MHz, the aggregate power flux-density values produced by mobile earth stations in a network of the mobile-satellite service operating in this band shall not exceed −181 dB(W/m2) in 10 MHz and −194 dB(W/m2) in any 20 kHz at any radio astronomy station recorded in the Master International Frequency Register, for more than 2% of integration periods of 2000 s.

- 47 CFR 2.106(c)(246)

  - US246 No station shall be authorized to transmit in the following bands: 73-74.6 MHz, 608-614 MHz, except for medical telemetry equipment and white space devices, 1400-1427 MHz, 1660.5-1668.4 MHz, 2690-2700 MHz, 4990-5000 MHz, 10.68-10.7 GHz, 15.35-15.4 GHz, 23.6-24 GHz, 31.3-31.8 GHz, 50.2-50.4 GHz, 52.6-54.25 GHz, 86-92 GHz, 100-102 GHz, 109.5-111.8 GHz, 114.25-116 GHz, 148.5-151.5 GHz, 164-167 GHz, 182-185 GHz, 190-191.8 GHz, 200-209 GHz, 226-231.5 GHz, 250-252 GHz.

As seen above, for the protection of the radioastronomy band operating at 1660-1660.5MHz, only the guarantee that no harmful interference is imposed upon the radioastronomy service is mandated.

For the protection of the radioastronomy band in 1668-1670MHz a power flux density limit is imposed, which is measured at the radioastronomy station. The MSS UE must not exceed a PFD level of -181dBW/m^2 in 10MHz and -194dBW/m^2 in any 20kHz. This cannot be exceeded for more than 2% of integration periods of 2000s.

Observation 6: Protection of the radioastronomy bands is mandated by the FCC. For the 1660-1660.5MHz RA band, interference must not be imposed upon the RA station by the UE. For the 1668-1670MHz band, PFD limits are imposed.

NTIA provides useful information about the areas where Radioastronomy stations operate in the Unites States:

Observation 7: The locations and radius of operation of Radio Astronomy stations are well-known and published via various sources, such as NTIA and the ITU frequency registry, therefore, protection measures can be restricted to the locations of proximity to RA stations.

### Proposed Approach

In order to improve the protection of Radio Astronomy stations, we propose a number of possible mechanisms.  Currently, no explicit mechanisms are specified, but implicit mechanisms, such as RB restrictions, can be implemented.

It is also necessary for the network to be aware of the coarse location of UE that are operating in the vicinity of Radio Astronomy stations.

Proposal 5: Specify a “TX disable” signal to be issued by the Network when a UE is close to a radioastronomy station which is registered in the ITU master international frequency register.

Proposal 6: Further study mechanisms to improve protection of Radio Astronomy stations, the following options could be considered:

- TX Disable

- RB restrictions

- NS

Proposal 7: Further investigation is required in order to understand what ‘close’ to a radioastronomy station means in practical distances and limits depending on device transmission characteristics.

Proposal 8: Derivation of distance from a RA station which yields this PFD performance should be calculated, and these distances / limits may become an additional network signalled value in the event a UE is within this distance from an RA station on the ITU master frequency international register.

## EN 301 681 General Regulatory Information

The ETSI EN 301 681 harmonized standard mandates the regulatory spectrum emissions concerned with L-band and Extended L-band. Band 253 is extended L-band, and bands 251 and 250 are combinations of L-band and extended L-band as seen in Table 1.

### Out of Band Masks

If a UE has only the capability of operating using the L-band spectrum (e.g. a UE which exclusively utilises band 251) the spectrum emissions mask which the UE must comply with is captured in Table 3 within ETSI EN 301 681.

The spectrum emissions mask is as follows for Table 3:

Figure 1 EN 301 681 Table 3 OOB SEM

If a UE has the capability of operating using any part of the extended L-band spectrum (e.g. a UE which exclusively utilises bands 253 and 250), the spectrum emissions mask which the UE must comply with is captured in Table 3a within EN 301 681.

The spectrum emissions mask is as follows for Table 3a:

Figure 2 EN 301 681 Table 3a OOB SEM

Observation 8: The ETSI EN 301 681 harmonized standard provides regulatory out of band spectrum emissions masks for ESD for L-band and extended L-band operation, which is applicable to bands 253, 251 and 250.

### In-band Mask

The ETSI EN 301 681 also provides an in-band spectrum emissions mask, captured in Table 4a. The limits specified in the table are taken from the edge of the nominated bandwidth of the transmission. The nominated bandwidth is declared by the UE manufacturer and is defined as follows:

Nominated bandwidth (Bn): bandwidth of the MES radio frequency transmission that is nominated by the applicant and that is wide enough to encompass all spectral elements of the transmission necessary for communication and which have a level greater than the specified unwanted emissions limits; also the nominated bandwidth is centred on the transmit frequency and does not exceed 180 % of the 3 dB bandwidth of the signal and is within the assigned part of the MSS transmit frequency band within which the MES operates. NOTE: the nominated bandwidth is wide enough to take into account the transmit carrier frequency stability.

Observation 9: ETSI EN 301 681 defines a nominated transmission bandwidth, which can be as wide as 180% of the 3dB bandwidth of the signal of interest. The edge of the nominated bandwidth is where the in-band ETSI EN 301 681 spectrum emissions mask is taken from.

Figure 3 EN 301 681 Table 4a In-band Spectrum Emissions Mask

Observation 10: ETSI EN 301 681 harmonized standard provides regulatory in-band band spectrum emissions masks for ESD for L-band and extended L-band operation, which is applicable to bands 253, 251 and 250.

### Protection MSS Big LEO bands

The frequency bands in the 1616-1626.5MHz are licensed to other MSS operators. The portion of this spectrum from 1626-1626.5MHz is licensed globally. The harmonized standard ETSI EN 301 681 mandates a spectrum emission mask to protect this band:

Figure 4 Spectrum Emissions Masks for EN 301 681

Comparing the in-band mask to the out of band mask for the lower band edge of the L-band spectrum (i.e. the edge which borders the other operator’s spectrum), we can see that the the emissions limits imposed on this lower edge are up to 40dB more stringent when compared to the in-band mask.

Observation 11: The ETSI EN 301 681 specification captures the emisisons requirements for the protection of the other operators transmitting at 1626-1626.5MHz, and is sufficient to guarantee the protection of systems operating in this band.

## FCC Part 25

The FCC mandates spectrum emissions requirements depending on the nominated transmission bandwidth of the UE and the mean transmitter power of the UE. The FCC specifies the attenuation with reference to the UE mean transmit power over a measurement bandwidth of 4kHz, the magnitude of which varies depending on the transmit power. In the following sections we assume that the UE has an EIRP of -7dBW (i.e. a PC3 device with a 0dBi antenna), and that the channel has been allocated directly adjacent to the edge of band.

### NR CHBW – 5MHz

Figure 5 FCC Part 25 SEM - 5MHz CHBW

### NR CHBW - 10MHz

Figure 6 FCC Part 25 SEM - 10MHz CHBW

### NR CHBW - 20MHz

Figure 7 FCC part 25 SEM - 20MHz CHBW

Observation 12: The FCC Part 25 regulations specify an SEM for attenuation from maximum transmit power. The stringency of this SEM depends on the channel bandwidth of the transmission and the transmit power of the UE.

## Application and Testing of Requirements

In previous meetings, the point has been raised as to how a radiated requirement as specified by ETSI. The ETSI EN 301 681 harmonized standard specifies a method by which a device can be tested for compliance to radiated requirements by means of conducted tests.

Section 5.2.1.4.6 defines a methodology by which ETSI mandates radiated compliance can be discerned from conducted tests. A device can be considered compliant to ETSI if the tests are carried out via a conducted interface using the framework specified in this section, using the same test parameters as specified in the relevant test section e.g. frequency and measurement bandwidth.

The results from these must be then added to the highest antenna gain in the antenna beam pattern at the frequency of interest. Therefore a full characterisation of the antenna must be obtained for the whole range of frequencies that are to be tested.

Observation 13: Compliance to the ETSI EN 301 681 requirements can be verified via a conducted interface, with the highest antenna gain at the frequency of interest being added to the PSD of each test point to assess compliance to the emissions masks specified in the EN 301 681 standard.

Proposal 9: The option to assess compliance to ETSI EN 301 681 radiated requirements by conducted means should be captured in a note within the specification, stating that antenna characteristics over the full frequency range are required in order to suitable assess the compliance to these radiated requirements.

Proposal 10: This approach to assess compliance to ETSI EN 301 681 should also be considered for FCC part 25.202. With the radiated requirements being ascertained by conducted means and the radiated requirements can be discerned for regulatory approval using an appropriate characterisation of the UE antenna over the required frequency response. This process should also be captures in a note within the specification.

# Conclusions

The following observations and proposals are summarized as follows:

Observation 1: The majority of the existing regulatory requirements for emission limits in MSS bands, with main exception in the FCC Part 25, are specified as radiated requirements in the form of EIRP limits.

Proposal 1: 3GPP should identify the appropriate methodology to capture radiated regulatory requirements in the specification(s), in such a way that ensures compliance to regulations in all operational scenarios, but does not create unnecessary implementation burden.

Proposal 2: 3GPP is already in the process of defining FR1 OTA radiated requirements for UE.  A possible approach could be the introduction of these requirements under new sections on OTA radiated requirements in the corresponding UE RF specs.

Proposal 3: Whether there is a need for conversion of regulatory emission limits from radiated to conducted requirements should also be further discussed

Observation 3: In ETSI EN 301 681, TX-off as a measure for protection of RA stations is considered sufficient assuming the carrier-off state requirement can be met.

Observation 4: The ETSI EN 301 681 requirement for carrier-off state is less stringent than that of the ITU M.1480.

Observation 5: The 3GPP carrier off state requirement is -3dBW/Hz, compared to ETSI EN 301 681 at -27dBW/Hz and ITU M.1480 at -17dBW/Hz.

Observation 6: Protection of the radioastronomy bands is mandated by the FCC. For the 1660-1660.5MHz RA band, interference must not be imposed upon the RA station by the UE. For the 1668-1670MHz band, PFD limits are imposed.

Observation 7: The locations and radius of operation of Radio Astronomy stations are well-known and published via various sources, such as NTIA and the ITU frequency registry, therefore, protection measures can be restricted to the locations of proximity to RA stations.

Proposal 5: Specify a “TX disable” signal to be issued by the Network when a UE is close to a radioastronomy station which is registered in the ITU master international frequency register.

Proposal 6: Further study mechanisms to improve protection of Radio Astronomy stations, the following options could be considered:

- TX Disable

- RB restrictions

- NS

Proposal 7: Further investigation is required in order to understand what ‘close’ to a radioastronomy station means in practical distances and limits depending on device transmission characteristics.

Proposal 8: Derivation of distance from a RA station which yields this PFD performance should be calculated, and these distances / limits may become an additional network signalled value in the event a UE is within this distance from an RA station on the ITU master frequency international register.

Observation 8: The ETSI EN 301 681 harmonized standard provides regulatory out of band spectrum emissions masks for ESD for L-band and extended L-band operation, which is applicable to bands 253, 251 and 250.

Observation 9: ETSI EN 301 681 defines a nominated transmission bandwidth, which can be as wide as 180% of the 3dB bandwidth of the signal of interest. The edge of the nominated bandwidth is where the in-band ETSI EN 301 681 spectrum emissions mask is taken from.

Observation 10: ETSI EN 301 681 harmonized standard provides regulatory in-band band spectrum emissions masks for ESD for L-band and extended L-band operation, which is applicable to bands 253, 251 and 250.

Observation 11: The ETSI EN 301 681 specification captures the emisisons requirements for the protection of the other operators transmitting at 1626-1626.5MHz, and is sufficient to guarantee the protection of systems operating in this band.

Observation 12: The FCC Part 25 regulations specify an SEM for attenuation from maximum transmit power. The stringency of this SEM depends on the channel bandwidth of the transmission and the transmit power of the UE.

Observation 13: Compliance to the ETSI EN 301 681 requirements can be verified via a conducted interface, with the highest antenna gain at the frequency of interest being added to the PSD of each test point to assess compliance to the emissions masks specified in the EN 301 681 standard.

Proposal 9: The option to assess compliance to ETSI EN 301 681 radiated requirements by conducted means should be captured in a note within the specification, stating that antenna characteristics over the full frequency range are required in order to suitable assess the compliance to these radiated requirements.

Proposal 10: This approach to assess compliance to ETSI EN 301 681 should also be considered for FCC part 25.202. With the radiated requirements being ascertained by conducted means and the radiated requirements can be discerned for regulatory approval using an appropriate characterisation of the UE antenna over the required frequency response. This process should also be captures in a note within the specification.

# References

- Recommendation ITU-R M.1480; (05/2000); Essential technical requirements of mobile earth stations of geostationary mobile-satellite systems that are implementing the global mobile personal communications by satellite (GMPCS) – Memorandum of understanding arrangements in parts of the frequency band 1-3 GHz - https://www.itu.int/rec/R-REC-M.1480-0-200005-I/en

- ETSI EN 301 681; V2.1.2 (2016-11) - https://www.etsi.org/deliver/etsi_en/301600_301699/301681/02.01.02_60/en_301681v020102p.pdf

- FCC Title 47 Part 2 Section 106 – Table of Frequency Allocations -  https://www.ecfr.gov/current/title-47/chapter-I/subchapter-A/part-2/subpart-B/section-2.106

- FCC Part 25.202 -  https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25#p-25.202

- NTIA 1660.5-1668.4 MHz (December 1, 2015) - https://www.ntia.gov/files/ntia/publications/compendium/1660.50-1668.40_01DEC15.pdf

- NTIA 1660-1660.5 MHz (May 1, 2015) - https://www.ntia.doc.gov/files/ntia/publications/compendium/1660.00-1660.50_01MAY15.pdf