3GPP TSG-RAN WG4 Meeting #113 			             R4-2419558
Orlando, USA, 18th – 22nd November, 2024
 
Agenda Item:	7.9.4
Source:			SKY Perfect JSAT
Title:			Ku-band Maximum TRP and EIRP Analysis

Document for:	Discussion and Information

# Introduction

At RAN4#112bis, in SKY Perfect JSAT’s contribution (R4-2415679[1]), we introduced the related recommendations and regulations published by the ITU for Antenna sidelobe pattern and maximum off-axis EIRP density and we proposed not to specify maximum EIRP and Power limit. As a result of discussions, the following points have been summarized in the WF(R4-2416563 [2]):

Issue 1-2-1: Maximum TRP and EIRP for the NTN Ku band - FFS on the following options - Option 1: No maximum TRP and EIRP defined - Option 2: Maximum TRP and EIRP need to be defined, same approach as the NTN Ka band in the current specification - Option 3: Define maximum TRP and EIRP based on declaration by manufacturer

In this contribution, SKY Perfect JSAT provides our views for Maximum TRP and EIRP for the NTN Ku band.

# Analysis and Discussions

## VSAT UE Max Power for Co-existence Study

In RAN4#112bis, there was a concern raised about how the co-existence study would be conducted if a maximum EIRP is not specified. The details are provided in the contribution R4-2419556[3], which discusses the parameters for the co-existence study, the UE's EIRP value will only affect to the co-existence study between the uplink NTN in the 14.0 (13.75)-14.5 GHz range and the TN in the 14.8-15.35 GHz range. There is a 300 MHz separation between the NTN and TN frequencies. Since there is sufficient separation between the TN and NTN frequencies, it is expected that even a high Maximum EIRP for the UE would remain at a feasible level.

we understood that the study is continued for defining ACLR and ACS for operating in adjacent channels. On the other hand, even if the maximum EIRP value used in the co-existence study is not set as a requirement for the UE, and no restrictions are specified on the maximum EIRP as a requirement, it is expected that practical co-existence would still be feasible due to the frequency separation between TN and NTN.

Proposal 1: RAN4 should consider that practical co-existence is likely still feasible due to the frequency separation between TN and NTN, even if the maximum EIRP value from the co-existence study is not enforced as a requirement for Ku-band VSAT UE and no specific restrictions on maximum EIRP are set.

## VSAT UE Max Power Impact for Power Control

In RAN4#112bis, there was a remark that the Maximum Power requirement is necessary for Power Control. The physical layer power control procedures are described in TS 38.213, and it should be applied to the Ku-band VSAT UE. On the other hand, due to the distance from the VSAT UE to the satellites (GSO and NGSO), there are many combinations of antenna diameter and transmit power. We believe that it is essential to support various type of Max TRP and EIRP from the viewpoint of throughput, which will be discussed later.

To solve this issue, we would like to discuss whether it is possible to configure Max TRP and EIRP for each VSAT UE as variable parameters instead of defining Max TRP and EIRP as specifications in 3GPP standard, or whether it is possible to operate VSAT UE without Max TRP nor EIRP.

Proposal 2: For power control function, to discuss whether it is possible to configure Max TRP and EIRP for each VSAT UE as variable parameters instead of defining Max TRP and EIRP as specifications in 3GPP standard, or whether it is possible to operate VSAT UE without Max TRP nor EIRP.

## Impact for Communication Link Performance

In the Ka-band, UE maximum EIRP limit was specified in the table below.

TS38.101-5 Table 9.2.1.1-2: UE maximum output power limits for Fixed VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPMAX (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 1 | 35 | 76.2 |
|  | 2, 3 | 43 | 76.2 |
| NOTE: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. |  |  |  |

TS38.101-5 Table 9.2.1.2-2: UE maximum output power limits for Mobile VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPMAX (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 4 | 35 | 76.2 |
|  | 5 | 43 | 76.2 |
| NOTE: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. |  |  |  |

SKY Perfect JSAT calculated link budget and estimate the link quality and the achievable throughput in case 5G NR waveform carrier is transmitted from Ku-band VSAT UE to GSO Software Defined Satellites(SDS) and Legacy GSO satellites. Ku-band VSAT UE Tx EIRP density for Fixed/Mobile VSAT is 76.2dBm/13RBs, that is same as Ka-band. The GW station controlling the VSAT is assumed to have a 5m diameter antenna, and the satellite’s parameters use typical values for commercially utilized Ku-band GSO. The analysis was performed for communication with a legacy GSO satellite as well as SDS satellite since it is expected that SDS satellites will become popular in addition to legacy GSO satellites and various satellite specifications (e.g., satellite EIRP, satellite G/T, etc.) is deployed. In the analysis, satellite EIRP and satellite G/T is assumed to be 58.0 dBW and 12.5 dB/K for SDS, 44.0 dBW and -2.0dB/K for legacy GSO satellite, respectively.

The following 6 patterns of Modulation & Coding Scheme(MCS) were used for the calculations;

Table 2.3-1: CQI & MCS list for link budget analysis

| CQI index | modulation | code rate x 1024 | Efficiency | SNR |
| --- | --- | --- | --- | --- |
| 1 | QPSK | 78 | 0.152 | -9.48 |
| 2 | QPSK | 120 | 0.234 | -6.68 |
| 4 | QPSK | 308 | 0.377 | -1.80 |
| 6 | QPSK | 602 | 1.176 | 2.42 |
| 8 | 16QAM | 490 | 1.914 | 6.37 |
| 10 | 64QAM | 466 | 2.731 | 10.27 |

The calculation results are as shown in the table below;

Table 2.3-2: Link Budget Analysis for GSO Software Defined Satellites (SDS)

(VSAT UE EIRPMAX=76.2dBm)

| Carrier | CQI |  | 1 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modulation |  | QPSK | QPSK | QPSK | QPSK | 16QAM | 64QAM |
|  | Code Rate x 1024 |  | 78 | 120 | 308 | 602 | 490 | 466 |
|  | Channel Bandwidth | MHz | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 |
|  | Bit Efficiency | bps/Hz | 0.15 | 0.23 | 0.60 | 1.18 | 1.91 | 2.73 |
|  | Throughput | Mbps | 2.85 | 4.39 | 11.26 | 22.01 | 35.83 | 51.11 |
| Uplink | Uplink EIRP | dBW | 46.20 | 46.20 | 46.20 | 46.20 | 46.20 | 46.20 |
|  | Uplink path loss | dB | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 |
|  | Satellite G/T | dB/K | 12.50 | 12.50 | 12.50 | 12.50 | 12.50 | 12.50 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Uplink C/N | dB | 7.49 | 7.49 | 7.49 | 7.49 | 7.49 | 7.49 |
| Downlink | Satellite EIRP | dBW | 58.00 | 58.00 | 58.00 | 58.00 | 58.00 | 58.00 |
|  | Output back-off | dB | 10.78 | 10.78 | 10.78 | 10.78 | 10.78 | 10.78 |
|  | Downlink path loss | dB | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 |
|  | Rx Antenna G/T | dB/K | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Downlink C/N | dB | 30.47 | 30.47 | 30.47 | 30.47 | 30.47 | 30.47 |
| Total | Total C/N | dB | 7.47 | 7.47 | 7.47 | 7.47 | 7.47 | 7.47 |
|  | Required C/N | dB | -9.48 | -6.66 | -1.80 | 2.42 | 6.37 | 10.27 |
|  | C/N Margin | dB | 16.94 | 14.12 | 9.26 | 5.04 | 1.10 | -2.80 |

Table 2.3-3: Link Budget Analysis for GSO Legacy Satellites

(VSAT UE EIRPMAX=76.2dBm)

| Carrier | CQI |  | 1 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modulation |  | QPSK | QPSK | QPSK | QPSK | 16QAM | 64QAM |
|  | Code Rate x 1024 |  | 78 | 120 | 308 | 602 | 490 | 466 |
|  | Channel Bandwidth | MHz | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 |
|  | Bit Efficiency | bps/Hz | 0.15 | 0.23 | 0.60 | 1.18 | 1.91 | 2.73 |
|  | Throughput | Mbps | 2.85 | 4.39 | 11.26 | 22.01 | 35.83 | 51.11 |
| Uplink | Uplink EIRP | dBW | 46.20 | 46.20 | 46.20 | 46.20 | 46.20 | 46.20 |
|  | Uplink path loss | dB | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 |
|  | Satellite G/T | dB/K | -2.00 | -2.00 | -2.00 | -2.00 | -2.00 | -2.00 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Uplink C/N | dB | -7.01 | -7.01 | -7.01 | -7.01 | -7.01 | -7.01 |
| Downlink | Satellite EIRP | dBW | 44.00 | 44.00 | 44.00 | 44.00 | 44.00 | 44.00 |
|  | Output back-off | dB | 8.34 | 8.34 | 8.34 | 8.34 | 8.34 | 8.34 |
|  | Downlink path loss | dB | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 |
|  | Rx Antenna G/T | dB/K | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Downlink C/N | dB | 18.91 | 18.91 | 18.91 | 18.91 | 18.91 | 18.91 |
| Total | Total C/N | dB | -7.02 | -7.02 | -7.02 | -7.02 | -7.02 | -7.02 |
|  | Required C/N | dB | -9.48 | -6.66 | -1.80 | 2.42 | 6.37 | 10.27 |
|  | C/N Margin | dB | 2.46 | -0.36 | -5.22 | -9.45 | -13.39 | -17.29 |

These results were calculated without considering interference, and it is expected that the outcomes would be even worse when taking into account interference that should be considered in satellite communications with GSO, such as adjacent satellite interference, cross-polarization interference, and inter-modulation.

In the case of using legacy GSO satellite, the footprint per beam is much wider than SDS satellites. This means that satellite EIRP and satellite G/T performance is lower than GSO SDS satellite. In the result, if the EIRPMAX for Ku-Band VSAT UE is specified to 76.2dBm(46.2dBW) per 13RBs, the bit efficiency can be achieved no less than 0.15bps/Hz and it is not reasonable performance.

To resolve the issue, the bigger EIRPMAX is required for Ku-band VSAT UE. As a sample, link budget analysis calculation in case EIRPMAX for Ku-band VSAT UE is assumed to 12dB higher than that for Ka-Band VSAT UE, that is 88.2dBm(58.2dBW) per 13RBs, is performed shown in the table below;

Table 2.3-4: Example of Link Budget Analysis for GSO Legacy Satellites

(VSAT UE EIRPMAX=88.2dBm)

| Carrier | CQI |  | 1 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modulation |  | QPSK | QPSK | QPSK | QPSK | 16QAM | 64QAM |
|  | Code Rate x 1024 |  | 78 | 120 | 308 | 602 | 490 | 466 |
|  | Channel Bandwidth | MHz | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 |
|  | Bit Efficiency | bps/Hz | 0.15 | 0.23 | 0.60 | 1.18 | 1.91 | 2.73 |
|  | Throughput | Mbps | 2.85 | 4.39 | 11.26 | 22.01 | 35.83 | 51.11 |
| Uplink | Uplink EIRP | dBW | 58.20 | 58.20 | 58.20 | 58.20 | 58.20 | 58.20 |
|  | Uplink path loss | dB | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 |
|  | Satellite G/T | dB/K | -2.00 | -2.00 | -2.00 | -2.00 | -2.00 | -2.00 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Uplink C/N | dB | 4.99 | 4.99 | 4.99 | 4.99 | 4.99 | 4.99 |
| Downlink | Satellite EIRP | dBW | 44.00 | 44.00 | 44.00 | 44.00 | 44.00 | 44.00 |
|  | Output back-off | dB | 8.34 | 8.34 | 8.34 | 8.34 | 8.34 | 8.34 |
|  | Downlink path loss | dB | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 |
|  | Rx Antenna G/T | dB/K | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Downlink C/N | dB | 18.91 | 18.91 | 18.91 | 18.91 | 18.91 | 18.91 |
| Total | Total C/N | dB | 4.82 | 4.82 | 4.82 | 4.82 | 4.82 | 4.82 |
|  | Required C/N | dB | -9.48 | -6.66 | -1.80 | 2.42 | 6.37 | 10.27 |
|  | C/N Margin | dB | 14.29 | 11.47 | 6.61 | 2.39 | -1.55 | -5.45 |

Observation 1: The EIRP required for a Ku-band VSAT UE to connect with a legacy GSO satellite is over 10 dB higher than what is needed for connecting to a GSO SDS.

In this case, although the analysis was performed for communication with a legacy transparent GSO satellite and GSO SDS only, the required EIRP is >10dB difference.  In addition, the NGSO case, the geographical distance between the VSAT UEs and the NGSO satellites are varied upon the NGSO constellation design, so there are many more possible combinations of antenna diameter and transmission power than in the GSO case. Therefore, to specify the maximum transmit power for Ku-band VSAT UE is not reasonable. If it is necessary to specify the maximum TRP, the parameter should be applied for many cases that exist, 12dB or higher value should be specified.

Proposal 3: To guarantee the quality of communication for both GSO and NGSO, it would be preferable not to specify a maximum EIRP requirement or to specify one that can be applied to various scenarios.

Proposal 4: The maximum EIRP requirement should account for the numerous possible combinations of antenna diameter and transmission power for Ku-band VSAT.

# Conclusion

Observation 1: The EIRP required for a Ku-band VSAT UE to connect with a legacy GSO satellite is over 10 dB higher than what is needed for connecting to a GSO SDS.

Proposal 1: RAN4 should take this into consideration, as even if the maximum EIRP value used in the co-existence study is not set as a requirement for the Ku-band VSAT UE, and no restrictions are specified on the maximum EIRP, practical co-existence is still expected to be feasible due to the frequency separation between TN and NTN.

Proposal 2: For power control function, to discuss whether it is possible to configure Max TRP and EIRP for each VSAT UE as variable parameters instead of defining Max TRP and EIRP as specifications in 3GPP standard, or whether it is possible to operate VSAT UE without Max TRP nor EIRP.

Proposal 3: To guarantee the quality of communication for both GSO and NGSO, it would be preferable not to specify a maximum EIRP requirement or to specify one that can be applied to various scenarios.

Proposal 4: The maximum EIRP requirement should account for the numerous possible combinations of antenna diameter and transmission power for Ku-band VSAT.

# References

[1] R4-2415679, Ku-band VSAT UE RF requirement analysis ; SKY Perfect JSAT, RAN#112bis

[2] R4-2416563, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF; Moderator (CHTTL), RAN#112bis

[3] R4-2419556, Discussion on VSAT UE parameter for Ku-band co-existence study; SKY Perfect JSAT, RAN#113