3GPP TSG-RAN WG4 Meeting #113	R4-2419576

# Orlando, US, November 18 – 22, 2024

Title:	On inter-band DC configuration grouping cleanup

Source:	ZTE Corporation, Sanechips

Agenda item:	9.1.1.4

Document for:	Approval

- 1.	Introduction

In last RAN4 meeting, several draft CRs on EN-DC configuration grouping were endorsed to organize the EN-DC configuration tables with different constituent bands [1-4]. It is based on the guidance of grouping DC configurations as specified in TR 38.846 as below.

-	Grouping of DC configurations is based on common band combination.

-	Common band combination is considered as the configuration having the same band sequence, such as DC_x-y-y_nz and DC_x-x-y_nz are not common band combination, while all configurations with DC_x-y_nz(*) having non-contiguous parts in band nz are considered as common band combination.

-	In case E-UTRA or/and NR has non-contiguous CA, it will be in a separate row compared to cases when DC configuration has only single carrier or contiguous CA operation.

-	If multiple UL DC configurations are indicated with multiple DL DC configurations, only UL DC configurations with the same or a lower number of carriers in the same fallback group are valid UL configurations.

Although the draft CRs in [1-4] are for EN-DC within FR1, the above guidance also applies to other inter-band DC types such as NE-DC and NR-DC according to TR 38.846. The rules make the DC configuration table clearer and reduce the configuration table size.

Observation 1.  It is observed that in the rules for grouping DC configurations not only apply to EN-DC configurations but also apply to NE-DC and NR-DC configurations.

In current RAN4 spec, apart from EN-DC within FR1, some other DC configurations also do not comply with the above grouping rules which should be corrected.

In this proposal, we proposed some follow-up CRs to cleanup the grouping of DC configurations with different DC types. Furthermore, the CRs include not only for inter-band DC within FR1, but also inter-band DC including FR1 and FR2.

Proposal 1.  It is suggested to approve the cleanup CRs in [ - ] for all types of DC configuration grouping.

- 2. Conclusion

In this paper, we discuss the remain issues about the grouping of inter-band DC configurations in RAN4 specifications. The following observation and proposal are provided.

Observation 1.  It is observed that in the rules for grouping DC configurations not only apply to EN-DC configurations but also apply to NE-DC and NR-DC configurations.

-	Grouping of DC configurations is based on common band combination.

-	Common band combination is considered as the configuration having the same band sequence, such as DC_x-y-y_nz and DC_x-x-y_nz are not common band combination, while all configurations with DC_x-y_nz(*) having non-contiguous parts in band nz are considered as common band combination.

-	In case E-UTRA or/and NR has non-contiguous CA, it will be in a separate row compared to cases when DC configuration has only single carrier or contiguous CA operation.

-	If multiple UL DC configurations are indicated with multiple DL DC configurations, only UL DC configurations with the same or a lower number of carriers in the same fallback group are valid UL configurations.

Proposal 1.  It is suggested to approve the cleanup CRs in [ - ] for all types of DC configuration grouping.

# 3. References

- R4-2416110, (DC_R18_2BLTE_1BNR_3DL2UL) Draft CR for TS 38.101-3 on 2L and 1N bands EN-DC configuration grouping, ZTE, Sanechips.

- R4-2416111, (DC_R18_xBLTE_1BNR_yDL2UL) Draft CR for TS 38.101-3 on (3-5)L and 1N bands EN-DC configuration grouping, ZTE, Sanechips.

- R4-2416112, (DC_R18_xBLTE_2BNR_yDL2UL) Draft CR for TS 38.101-3 on (1-4)L and 2N bands EN-DC configuration grouping, ZTE, Sanechips.

- R4-2416113, (DC_R18_xBLTE_yBNR_zDL2UL) Draft CR for TS 38.101-3 on (1-3)L and (3-5)N bands EN-DC configuration grouping, ZTE, Sanechips.

- R4-2419578, Cleanup on 38.101-3 for inter-band DC configuration grouping, ZTE, Sanechips

- R4-2419577, Cleanup on 38.101-1 for FR1 inter-band DC configuration grouping, ZTE, Sanechips