# 3GPP TSG-RAN WG4 Meeting #113	R4-2419624

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.23.4.2

Source: 	Samsung

Title: 	RRM impact for Rel-19 LP-WUS/WUR

Document for:	Discussion

- Introduction

During RAN#105 meeting, a revised WID [1] was approved on low-power wake-up signal and receiver for NR (LP-WUS/WUR). In RAN4#112bis meeting, RAN4 achieved some agreements in WF [3].

In this contribution, our views on LP-WUS/WUR and related analysis of RRM requirements are provided.

- Discussion

Issue 1-1-8: LP-WUR operating carrier frequency

A LS from RAN1 (R4-2414909) was sent to RAN2/RAN4 on operating frequency assumption for LR:

Following agreements reached in last RAN4 meeting:

Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. Issue for further discussion based on contribution driven: - For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

In our understanding, the operation frequency for LR shall be same carrier frequency as serving cell of MR otherwise it’s not feasible to support MR measurement offloading to LR if operating frequency from MR and LR is different. On supporting bands on LR receiver, LP-WUS/WUR is an independent feature introduced under Rel-19. The supporting bands of LR receiver shall be subsets of the bands which supported by UE. The detailed capability of LP-WUS reception and granularity can be discussed further in later stage. With above understanding, MR measurement offloading shall be applicable only on the bands which LR supported.

Proposal 1: From Rel-19 RAN4 RRM requirement perspective, LR shall be operated as the same carrier frequency of serving cell for MR measurement offloading and relaxation.

For LP-WUS monitoring, it’s not necessary to keep LR and MR on the same carrier. Meanwhile if LR and MR not on the same carrier, then wakeup MR, the sync information from LR can’t be used for MR and in worst case MR need to do sync procedure as unknow cell condition which increase interruption delay for MR wake up.

Observation 1: From LP-WUS monitoring perspective, LR and MR can be on different carriers/bands. And the delay for MR wake up will be increased with re-synchronization to operating carrier for MR required before MR is ready to monitor paging.

Another issue related to frequency is FR2 handling, in previous RAN4 meeting following agreement reached:

RAN4 agreements - RAN4#110bis Issue 2-1-1: Operation bands for LP-WUR Agreement: - Focus on FR1 licensed bands - FR2 is not precluded - FFS on which licensed bands will be chosen as example bands for band specific requirements. - RAN4#112 Issue 1-1-6: Specify FR2 LP-WUS RF requirements Agreements: - Wait to RAN1 progress on FR2 LP-WUS work..

In last RAN-P, following agreement endorsed:

RAN#105 WF: It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

With above agreements, FR1 and FR2 is the scope meanwhile FR2 will try to reuse FR1 design as much as possible from RAN1 perspective.

From RAN4 RF and RRM requirements perspective, supporting FR2 will double the effort in RAN4. To control RAN4 work load, we suggest to prioritize the discussion in FR1.

Proposal 2: From RAN4 requirement perspective, prioritize the work in FR1.

Issue 1-2-2: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

Following agreements were reached in RAN2 on the exit/entrance criteria

##### RAN2#127: Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ. ##### RAN2#127: RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. ##### RAN2#127bis: Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements. ##### RAN2#127: RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

With above agreements, we can summarize the status as following:

| Case | Entrance | Exit | Metric |
| --- | --- | --- | --- |
| LP-WUS monitoring | MR threshold and LR threshold | LR threshold | (LP-)RSRP and optional (LP-)RSRQ. |
| RRM measurement offloading (no serving cell measurement) | MR threshold LR threshold (can be considered) | LR threshold | (LP-RSRP) |

We can observe that:

Observation 2: For entrance condition of LP-WUS monitoring, both MR based and LR based threshold can be configured, and exit condition is based on LR based threshold.

Observation 3: For MR measurement offloading (case 1), entrance condition relies on MR measurement only or both MR measurement threshold and LR measurement threshold can be configured; and exit condition is LR based threshold.

Proposal 2: For MR measurement offloading, RAN4 specify the evaluating period on whether the threshold configured by network is fulfilled or not

- The evaluating period is same as the measurement period of MR and/or LR

Observation 4: For LP-WUS monitoring entrance, both MR threshold and LR threshold can be configured. If configured both, the both thresholds need to be meet.

Proposal 3: For LP-WUS monitoring entrance and exit, RAN4 specify the evaluating period on whether the threshold configured by network is fulfilled or not

- Evaluation period shall follow the measurement period of MR and//or LR

Another issue needs to be further discussed for entrance condition, how to consider when LR need to be on state and evaluate the criteria met or not. Three possible assumptions:

- Assumption 1: UE always turn LR on if both MR and LR threshold configured, this will bring UE power consumption issue.

  - Evaluation period can be max of measurement period of MR and LR

- Assumption 2: UE first check MR threshold by MR measurement and if MR measurement meet the condition and then UE turn on LR to check LR criteria

  - Evaluation period can be sum of measurement period of MR and LR

- Assumption 3: UE only apply MR measurement, and apply offset to get corresponding LR measurement hypothesis. This approach require UE to do calibration between MR measurement and LR measurement. it is worth noting that the RSRP measurement results difference between MR and LP-WUR should be stable and comparable to enable such option.

  - Evaluation period can measurement period of MR+ additional margin for calibration of MR and LR

Proposal 4: RAN4 need to further how to specify evaluation period for LP-WUS monitoring entrance if both LR and MR threshold configured.

Issue 1-2-3: On interruption related requirements for LP-WUR

Once LP-WUR detected LP-WUS indicating paging indication, then UE needs to wake up MR receiver and monitor paging on paging occasion.

The maximum interruption/processing time between UE being ready to receive Paging/PDCCH by MR and LP-WUS received “on” indication by LP-WUR consist of following factor (time offset between LP-WUS/paging indication to MR ready for Paging monitoring):

“LP-WUS detection processing time + MR wake-up time + MR sync time”

- MR wake-up time depending UE implementation can be 400 ms ~1s

- MR sync time pending on whether UE can utilize LR sync time/frequency information as starting point for MR i.e., if MR and LR sharing same of RF components e.g.  LO, and/or antenna then UE can utilize LR sync information for MR receiver:

  - For UE with shared architecture between LR and MR, LR sync info can be used

    - ~1~2 SSB required for MR sync time

  - Other wise

    - ~10# SSB required for MR sync time

RAN4 may need to specify the maximum interruption time between LP-WUS/LO to PO, which the UE expects to be ready for MR in Paging monitoring. And NW can configure PO offset to LO no less than above processing time/interruption time specified by RAN4.

Proposal 5: New requirements need to be specified for the processing/interruption time between LP-WUS received paging indication to MR ready for Paging monitoring with following factors to be considered

- “LP-WUS detection processing time + MR wake-up time + MR sync time”

Proposal 6: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement

Proposal 7: MR sync time, need to be further discuss with different side conditions

- Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g.  1~2 SSB sufficient

- Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

RAN2 is still on discussion for the neighbour cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In R16, relaxation factor 8 was introduced in RAN4 for UE inter-frequency/intra-frequency measurement when UE meet ‘not at cell edge’ criteria and or ‘low mobility’.

According to LP-WUR SI discussion, LP-WUS can achieve power saving gain when relaxation factor larger than 8.

Proposal 8: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation.

- If not possible to down-selection as single value, multiple values can be considered with delta on threshold.

- Conclusion

In this contribution, our view on RRM requirement is provided.

Issue 1-1-8: LP-WUR operating carrier frequency

Proposal 1: From Rel-19 RAN4 RRM requirement perspective, LR shall be operated as the same carrier frequency of serving cell for MR measurement offloading and relaxation.

Observation 1: From LP-WUS monitoring perspective, LR and MR can be on different carriers/bands. And the delay for MR wake up will be increased with re-synchronization to operating carrier for MR required before MR is ready to monitor paging.

Proposal 2: From RAN4 requirement perspective, prioritize the work in FR1.

Issue 1-2-2: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

Observation 2: For entrance condition of LP-WUS monitoring, both MR based and LR based threshold can be configured, and exit condition is based on LR based threshold.

Observation 3: For MR measurement offloading (case 1), entrance condition relies on MR measurement only or both MR measurement threshold and LR measurement threshold can be configured; and exit condition is LR based threshold.

Proposal 2: For MR measurement offloading, RAN4 specify the evaluating period on whether the threshold configured by network is fulfilled or not

- The evaluating period is same as the measurement period of MR and/or LR

Observation 4: For LP-WUS monitoring entrance, both MR threshold and LR threshold can be configured. If configured both, the both thresholds need to be meet.

Proposal 3: For LP-WUS monitoring entrance and exit, RAN4 specify the evaluating period on whether the threshold configured by network is fulfilled or not

- Evaluation period shall follow the measurement period of MR and//or LR

Proposal 4: RAN4 need to further how to specify evaluation period for LP-WUS monitoring entrance if both LR and MR threshold configured with possible options as following:

- Assumption 1: UE always turn LR on if both MR and LR threshold configured, this will bring UE power consumption issue.

  - Evaluation period can be max of measurement period of MR and LR

- Assumption 2: UE first check MR threshold by MR measurement and if MR measurement meet the condition and then UE turn on LR to check LR criteria

  - Evaluation period can be sum of measurement period of MR and LR

- Assumption 3: UE only apply MR measurement, and apply offset to get corresponding LR measurement hypothesis. This approach requires UE to do calibration between MR measurement and LR measurement. It is worth noting that the RSRP measurement results difference between MR and LP-WUR should be stable and comparable to enable such option.

  - Evaluation period can measurement period of MR+ additional margin for calibration of MR and LR

Issue 1-2-3: On interruption related requirements for LP-WUR

Proposal 5: New requirements need to be specified for the processing/interruption time between LP-WUS received paging indication to MR ready for Paging monitoring with following factors to be considered

- “LP-WUS detection processing time + MR wake-up time + MR sync time”

Proposal 6: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement

Proposal 7: MR sync time, need to be further discuss with different side conditions

- Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g.  1~2 SSB sufficient

- Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

Proposal 8: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation.

- If not possible to down-selection as single value, multiple values can be considered with delta on threshold

# References

RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony

3GPP TR38.869: “Study on low-power wake up signal and receiver for NR”

R4-2416861	WF for RRM core requirements for LP-WUSWUR, vivo