3GPP TSG-RAN WG4 Meeting #113	R4-2419631

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Adapting NGSO requirements for RedCap support in Rel-19

Agenda item:	7.26.4.1

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The most recent version of the WID can be found in [2]. It includes a handful of objectives such as:

- Downlink Coverage Enhancements

- Uplink Capacity/Coverage Enhancements

- Signaling of the intended service area of a broadcast service (MBS broadcast)

- Support of regenerative payload for NTN systems

- Support of Rel-17 and Rel-18 RedCap and (e)RedCap UEs in NTN systems

This document is related to the 5th topic on the list above: RedCap deployments over NTN. The discussions for this objective were initiated in the previous meeting RAN4 #112. And as the outcome of the discussion a way-forward (WF) document was approved which contains a set of agreements and other way-forward (for further discussion) guidance for currently identified open issues [3].  The discussions further ensued in RAN4 #112bis, which culminated in the WF found in [4].

In this document we provide our views, related to the RAN4 impact of the introduction of RedCap over NTN considering the scope of this WI and/or the open issues already identified in the WF.

Discussion

The present contribution will present our views in some of the currently identified open items presented in [4] .

On applicability of L3 measurements for (e)RedCap UE

In RAN4 #112, companies agreed to introduce L3 measurement requirements associated to mobility for (e)RedCap UEs. But, in RAN#112bis, some companies asked for more clarification on what exactly would be the applicability of said requirements. The following WF was then agreed:

#### Issue 2-2-2: Further discussion on L3 measurements for (e)RedCap UE with FR1-NTN In the last meeting, RAN4 has agreed to defined the requirements of NR intra-frequency L3 measurements and NR inter-frequency L3 measurements, the proposal is a further clarification on the supported types of L3 measurements for (e)RedCap UEs with FR1-NTN. <Way Forward>: - Proposal 1 (CATT): RAN4 to define the requirements of the following types of L3 measurements for (e)RedCap UEs with FR1-NTN: - Intra-frequency measurements without measurement gaps - Intra-frequency measurements with measurement gaps - Inter-frequency measurement without measurement gaps - Inter-frequency measurement with measurement gaps

From our point of view, it only makes sense if both, intra and inter frequency measurement gaps are considered. But also, in what regards the applicability or not of measurement gaps, they are a key part of the NTN operation, in order to enable measurements to be conducted across multiple satellites at once.

Therefore, at least intra and inter frequency measurements with measurement gaps have to be supported. But once the support for measurement gaps has been agreed, it goes without saying that UEs that have the capability to perform measurements without gap will have enhanced performance in the network. Therefore, we propose:

Confirm that L3 measurement requirements will be defined for intra and inter frequency measurements for both with/without measurement gaps.

On the relaxed measurements.

In what concerns the relaxation of measurements to be performed in idle mode by the UE, there is an ongoing discussion whether the relaxation of measurements shall follow “NR NTN” agreed relaxations (Rel-17) or whether the RedCap relaxations are to be defined.

#### Issue 2-3-1: RedCap R17 relaxed measurement Online agreement <Agreement>: - FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline. - If defined, the requirements apply provided that target cell is GSO

RedCap relaxation scenarios include more relaxation cases than those observed for NR NTN by, for example introducing low mobility and stationary criteria. Although this adds to the flexibility of relaxation of measurements both type of measurements are not particularly useful for NTN deployments.

First of all, as it has been discussed several times for Rel-17 and Rel-18, mobility evaluations based on RRM measurements (such as RSRP levels) are not good indicators of movement in NTN where the “near-far” effect in a cell is very limited (low variability of RSRP levels). Besides, if mobility levels are to be evaluated, for a NTN device (RedCap or not), this has to be preferred to be performed using GNSS measurements, once they are already enabled for such devices.

In NTN, mobility measurements based on RSRP conditions are not good indicative of movement due to limited “near-far” effect.

Therefore, we propose:

Adopt the R17 NR NTN Relaxations for GSO. Do not apply RedCap R18 relaxations.

On eDRX Configuration

In what relates to the applicability of eDRX configuration for RedCap UEs in NTN, companies agreed that for GSO scenarios, it would be ok to support all existing eDRX requirements, but there is still disagreement over the applicability of the requirements for LEO. +

#### Issue 2-3-2: eDRX configuration Online agreement <Agreement>: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO.

This disagreement is justified because both sides have valid points. In one end, some companies argue that the eDRX cycle of 2.56 s have no applicable requirements under earth-moving scenarios. On the other end, other companies argue that this does not preclude the applicability of this eDRX cycle for quasi-earth fixed scenarios.

The decision must be consistent across releases, such that NR NTN and REDCAP NTN UEs operate following the same logical principle. Therefore, no cycle length equal or superior to 2.56 s is to be supported in NTN for earth moving scenarios. But we agree that this does not preclude UEs in earth-fixed scenarios to support longer DRX Cycles.

Support longer (e)DRX Cycles for LEO scenarios at least for Quasi-Earth Fixed cells.

On the impact of NR satellite switching with re-synchronization due to HD-FDD.

In the previous meeting, a concern was raised about the impact of HD-FDD requirements for RedCap UEs. One of the features that could be potentially impacted is the satellite switching.

#### Issue 4-2-2: The impact on NR Satellite switching with re-synchronization due to HD-FDD <Way Forward>: - Proposal 1 (vivo): For requirements on NR Satellite switching with re-synchronization for Red-cap NTN, RAN4 to reuse the existing Satellite switching with re-synchronization requirements for NTN with adding the following applicability rules to adapt HD-FDD case: - For RedCap NTN UE with HD-FDD, the Satellite switching with re-synchronization requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

This happens, as the UE might not be capable of transmit and receive at the same time, and transmission periods at the UE side would preclude the UE to perform measurements. This would make the requirements for satellite switching more complex and difficult to evaluate, as well as increasing the uncertainty on the exact point in time the UE is available for scheduling in the new satellite.

In satellite switching operation, the UE does not perform RACH; increasing the uncertainty in the time the UE leads to perform the measurements will increase the uncertainty on the network scheduling side.

In order to avoid that, the simple scenario is to adopt only “hard satellite switch” for RedCap UEs, in this manner, in the period where the UE performs measurements, it is well known that no uplink transmissions are to be performed.

Only support hard satellite switching for RedCap UEs

On the specification structure and RedCap applicability

Another topic was discussed in the WF is related to the specification structure. There are currently  discussions whether the RedCap over NTN shall be captured under a new suffix (e.g. E) or whether it should be new subclauses to the NTN clauses in TS 38.133. This was discussed in Issue 6-8-1 in [3].

Among the proposed options, in order to minimize the specification effort and the size of the specification, but still maintaining coherence and completeness of the specification, we would prefer that RedCap subclauses are inserted as subsections of NTN section (suffix C).

Adopt RedCap subclauses under NTN sections (under Suffix C).

But another thing that needs to be discussed is the applicability of the requirements for RedCap operation under NTN. For a UE to support this feature (RedCap over NTN), it has to support RedCap features and NTN features. There are therefore, two options to declare such capabilities:

- Option A: Declare support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

- Option B: Declare explicitly – via a new capability – the support to RedCap over NTN.

Both options have their pros and cons (flexibility versus capability signalling overhead). Therefore, we would like to introduce this discussion in RAN4 and propose:

RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

Conclusion

This paper has discussed the open issues on the specification of requirements for RedCap operations in FR1-NTN. Based on our investigations the following observations and proposals were made:

Proposal 1: Confirm that L3 measurement requirements will be defined for intra and inter frequency measurements for both with/without measurement gaps.

Observation 1: In NTN, mobility measurements based on RSRP conditions are not good indicative of movement due to limited “near-far” effect.

Proposal 2: Adopt the R17 NR NTN Relaxations for GSO. Do not apply RedCap R18 relaxations.

Proposal 3: Support longer (e)DRX Cycles for LEO scenarios at least for Quasi-Earth Fixed cells.

Observation 2: In satellite switching operation, the UE does not perform RACH; increasing the uncertainty in the time the UE leads to perform the measurements will increase the uncertainty on the network scheduling side.

Proposal 4: Only support hard satellite switching for RedCap UEs

Proposal 5: Adopt RedCap subclauses under NTN sections (under Suffix C).

Proposal 6: RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2414037, "The WF on RRM requirements for Rel-19 NR NTN phase3", Moderator (CATT), RAN4 #112, Maastricht, Netherlands, Aug. 19-23, 2024.

- R4-2416865, “WF on RRM requirements for NR_NTN_Ph3_Part1”, Hefei, China, 14 – 18 October, 2024