3GPP TSG-RAN4 Meeting #113	R4-2419642

# Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on coexistence studies and results

Source:	Samsung

Agenda item:	7.8.2.1

Document for:	Approval

1	Introduction

In the previous meetings, the WF R4-2414452 [1] and R4-2217122 [2] had achieved the baseline co-existence assumptions and some optional assumptions for NTN HPUE co-ex study. And also, there’re already 7 companies who had provided their co-ex study results in #112 and #112bis meeting.

In this paper, we will contribute our own study results and also discuss the ACLR/ACIR from the submitted results from previous meetings.

2	Co-ex study results from Samsung

2.1 NR-NTN GEO HPUE

In this section, we provided our NR-NTN HPUE co-ex study between NR NTN PC2/1.5/1 and NR BS for scenario 4. And the baseline assumptions are following the agreements in [1] and [2].

2.1.1 Baseline assumption and isolation distance as 1.5km

Table 2.1.1-1. NR-NTN GEO HPUE interfering NR BS w. different ACIR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.07 | 0.03 | - | - | - | 0.29 | 0.04 | - | - | - |
| PC1.5 | 0.12 | 0.05 | - | - | - | 0.63 | 0.15 | - | - | - |
| PC1 | 0.17 | 0.07 | - | - | - | 0.9 | 0.24 | - | - | - |

Table 2.1.1-2. NR-NTN GEO HPUE interfering NR BS w. different ACLR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.01 | - | - | - | - | 0.01 | - | - | - | - |
| PC1.5 | 0.01 | - | - | - | - | 0.03 | - | - | - | - |
| PC1 | 0.01 | - | - | - | - | 0.02 | - | - | - | - |

2.1.2 Baseline assumption and isolation distance as 0km

Table 2.1.2-1. NR-NTN GEO HPUE interfering NR BS w. different ACIR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 1.57 | 0.79 | - | - | - | 11.38 | 5.5 | 2.78 | 1.42 | - |
| PC1.5 | 2.37 | 1.2 | - | - | - | 17.81 | 8.53 | 4.31 | 2.2 | 1.11 |
| PC1 | 3.03 | 1.56 | 0.77 | - | - | 23.01 | 11.18 | 5.45 | 2.77 | - |

Table 2.1.2-2. NR-NTN GEO HPUE interfering NR BS w. different ACLR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.26 | - | - | - | - | 1.86 | 0.89 | - | - | - |
| PC1.5 | 0.41 | - | - | - | - | 3.03 | 1.75 | 1.01 | - | - |
| PC1 | 0.54 | - | - | - | - | 4.04 | 2.2 | 1.29 | - | - |

2.2 NR-NTN LEO-1200 HPUE

In this section, we provided our NR-NTN HPUE co-ex study between NR NTN PC2/1.5/1 and NR BS for scenario 4. And the baseline assumptions are following the agreements in [1] and [2].

2.2.1 Baseline assumption and isolation distance as 1.5km

Table 2.2.1-1. NR-NTN LEO-1200 HPUE interfering NR BS w. different ACIR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.06 | 0.02 | - | - | - | 0.26 | 0.04 | - | - | - |
| PC1.5 | 0.11 | 0.04 | - | - | - | 0.56 | 0.11 | - | - | - |
| PC1 | 0.16 | 0.06 | - | - | - | 0.93 | 0.3 | - | - | - |

Table 2.2.1-2. NR-NTN LEO-1200 HPUE interfering NR BS w. different ACLR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0 | - | - | - | - | 0 | - | - | - | - |
| PC1.5 | 0.01 | - | - | - | - | 0 | - | - | - | - |
| PC1 | 0.01 | - | - | - | - | 0.02 | - | - | - | - |

2.2.2 Baseline assumption and isolation distance as 0km

Table 2.2.2-1. NR-NTN LEO-1200HPUE interfering NR BS w. different ACIR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 1.56 | 0.78 | - | - | - | 11.38 | 5.5 | 2.78 | - | - |
| PC1.5 | 2.32 | 1.18 | - | - | - | 17.47 | 8.33 | 4.23 | 2.09 | - |
| PC1 | 2.95 | 1.52 | 0.76 | - | - | 22.43 | 11.05 | 5.36 | 2.94 | - |

Table 2.2.2-2. NR-NTN LEO-1200HPUE interfering NR BS w. different ACLR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.26 | - | - | - | - | 1.86 | - | - | - | - |
| PC1.5 | 0.4 | - | - | - | - | 2.89 | 1.55 | 1.01 | - | - |
| PC1 | 0.51 | - | - | - | - | 3.56 | 1.85 | - | - | - |

2.3 IoT-NTN GEO HPUE

In this section, we provided our IoT-NTN HPUE co-ex study between IoT NTN PC2/1 and NR BS for scenario 4. And the baseline assumptions are following the agreements in [1] and [2].

2.3.1 Baseline assumption and isolation distance as 1.5km

Table 2.3.1-1. IoT-NTN GEO HPUE interfering NR BS w. different ACIR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.25 | 0.09 | 0.03 | 0.01 | - | 0.81 | 0.11 | 0.03 | 0.02 | - |
| PC1 | 0.44 | 0.17 | - | - | - | 2.19 | 0.63 | 0.09 | - | - |

Table 2.3.1-2. IoT-NTN GEO HPUE interfering NR BS w. different ACLR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.02 | - | - | - | - | 0 | - | - | - | - |
| PC1 | 0.04 | - | - | - | - | 0.03 | - | - | - | - |

2.3.2 Baseline assumption and isolation distance as 0km

Table 2.3.2-1. IoT-NTN GEO HPUE interfering NR BS w. different ACIR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 5.23 | 2.69 | 1.35 | 0.66 | - | 33.46 | 18.46 | 9.74 | 4.95 | 2.36 |
| PC1 | 8.26 | 4.42 | 2.26 | 1.12 | - | 50.07 | 29.22 | 15.66 | 8.05 | 4.11 |

Table 2.3.2-2. IoT-NTN GEO HPUE interfering NR BS w. different ACLR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.9 | - | - | - | - | 6.84 | 3.56 | 1.97 | 1.15 | - |
| PC1 | 1.46 | 0.76 | - | - | - | 10.48 | 5.57 | 3.19 | 2.1 | 1.67 |

2.4 IoT-NTN LEO-1200 HPUE

In this section, we provided our IoT-NTN HPUE co-ex study between IoT NTN PC2/1 and NR BS for scenario 4. And the baseline assumptions are following the agreements in [1] and [2].

2.4.1 Baseline assumption and isolation distance as 1.5km

Table 2.4.1-1. IoT-NTN LEO-1200 HPUE interfering NR BS w. different ACIR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.08 | 0.03 | 0.01 | - | - | 0.2 | 0.03 | 0 | - | - |
| PC1 | 0.12 | 0.04 | 0.02 | - | - | 0.4 | 0.08 | 0.01 | - | - |

Table 2.4.1-2. IoT-NTN LEO-1200 HPUE interfering NR BS w. different ACLR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.01 | - | - | - | - | 0 | - | - | - | - |
| PC1 | 0 | - | - | - | - | 0 | - | - | - | - |

2.4.2 Baseline assumption and isolation distance as 0km

Table 2.4.2-1. IoT-NTN LEO-1200HPUE interfering NR BS w. different ACIR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 2.45 | 1.22 | 0.6 | - | - | 17.2 | 8.84 | 4.41 | 2.1 | 0.76 |
| PC1 | 2.84 | 1.44 | 0.71 | 0.34 | - | 20.02 | 10.13 | 5.08 | 2.34 | 0.87 |

Table 2.4.2-2. NR-NTN LEO-1200HPUE interfering NR BS w. different ACLR

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC2 | 0.39 | 0.19 | - | - | - | 2.83 | 1.34 | 0.64 | 0.3 | - |
| PC1 | 0.4 | - | - | - | - | 2.9 | 1.4 | - | - | - |

3	Discussions for co-ex results

In last meeting, the WF [2] has formed the early observations from the existing results submitted by 7 companies. The observations are as follows:

Issue 1-8: NR-NTN ACIR/ACLR Agreement: - Consider following observations can be used as a starting point for RF requirements analysis/studies: - Candidate ACLR value for NR-NTN PC2/PC1.5/PC1, based on initial co-ex study results received in #112bis meeting, is 30 dBc - The current TN requirements are more stringent than the observations from co-existence results. Overlapping geographic coverage between NTN and TN is discussion in RAN4 in other WIs. - FFS for final ACLR requirements Issue 1-9: IoT-NTN ACIR/ACLR Agreement: - Take same approach in issue 1-8

The results from 7 companies are collected and submitted to this meeting in a separate tdoc [3]. In this section, we will discuss the observations from the results in [3].

Among all the submitted results, we have following observations and proposals in general.

Deployment scenario: Rural or Urban

Observation 1: We had agreed in #112 meeting to use Urban case as worst case over Rural and other deployment scenarios.

Observation 2: From the received results, it also aligns with the previous agreement that Urban results are worse than Rural results.

Proposal 1: For co-ex study result conclusion discussion, in principle, to use Urban results instead of Rural results which aligns with previous agreement and worst case.

NR BS type: AAS or non-AAS

Observation 3: From the received results, it indicated that the NR BS with AAS antenna has more impact than the results assuming BS with non-AAS antenna.

Proposal 2: For co-ex study result conclusion discussion, in principle, to use AAS results instead of non-AAS results as worst case.

In this section, we will apply the principles in Proposal 1 and Proposal 2 for discussion.

3.1 Results for NR-NTN PC2

Isolation: 1.5km

| TP Loss | Company | Isolation | Wrap-around | Statistics | SAN Orbit | ACLR/ACIR | Required results (dBc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average | Samsung | 1.5 | Y | TN cluster | GEO | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Ericsson | 1.5 | Y | Impact sectors | GEO | ACIR | 6 |
|  | Ericsson | 1.5 | Y | Impact sectors | LEO-1200 | ACIR | 6 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | ACLR | 15~20 |
|  | Mediatek | 1.5 | Y | TN cluster | GEO | ACLR | 19.77 |
|  | Mediatek | 1.5 | Y | TN cluster | LEO-1200 | ACLR | 19.77 |
|  | Thales | 1.5 | Y | TN cluster | GEO | ACIR | <13 |
|  | Thales | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <13 |
| 5%-tile | Samsung | 1.5 | Y | TN cluster | GEO | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Ericsson | 1.5 | Y | Impact sectors | GEO | ACIR | 2~4 |
|  | Ericsson | 1.5 | Y | Impact sectors | LEO-1200 | ACIR | 4~6 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | ACLR | 30~35 |
|  | Mediatek | 1.5 | Y | TN cluster | GEO | ACLR | 19.77 |
|  | Mediatek | 1.5 | Y | TN cluster | LEO-1200 | ACLR | 19.77 |
|  | Thales | 1.5 | Y | TN cluster | GEO | ACIR | <13 |
|  | Thales | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <13 |

Isolation: 0km

| TP Loss | Company | Isolation | Wrap-around | Statistics | SAN Orbit | ACLR/ACIR | Required results (dBc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average | Samsung | 0 | Y | TN cluster | GEO | ACIR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACIR | <20 |
|  | Samsung | 0 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | Ericsson | 0 | Y | Impact sectors | GEO | ACIR | 28 |
|  | Ericsson | 0 | Y | Impact sectors | LEO-1200 | ACIR | <32 |
| 5%-tile | Samsung | 0 | Y | TN cluster | GEO | ACIR | 25~30 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACIR | 25~30 |
|  | Samsung | 0 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | Ericsson | 0 | Y | Impact sectors | GEO | ACIR | 32~34 |
|  | Ericsson | 0 | Y | Impact sectors | LEO-1200 | ACIR | 34 |

Proposal 3: For NR-NTN PC2, the required ACIR is below 20 dBc when isolation distance of 1.5km assumed. The final exact required ACIR and ACLR from co-ex study can be derived from all submitted results.

3.2 Observations for NR-NTN PC1.5

Isolation distance: 1.5km

| TP loss | Company | Isolation | Wrap-around | Statistics | SAN Orbit | ACLR/ACIR | Required results (dBc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average | Samsung | 1.5 | Y | TN cluster | GEO | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Ericsson | 1.5 | Y | Impact sectors | GEO | ACIR | 8~10 |
|  | Ericsson | 1.5 | Y | Impact sectors | LEO-1200 | ACIR | 8~10 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | ACLR | 20~25 |
|  | Mediatek | 1.5 | Y | TN cluster | GEO | ACLR | 22.88 |
|  | Mediatek | 1.5 | Y | TN cluster | LEO-1200 | ACLR | 22.88 |
|  | Thales | 1.5 | Y | TN cluster | GEO | ACIR | <13 |
|  | Thales | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <13 |
| 5%-tile | Samsung | 1.5 | Y | TN cluster | GEO | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Ericsson | 1.5 | Y | Impact sectors | GEO | ACIR | 6~8 |
|  | Ericsson | 1.5 | Y | Impact sectors | LEO-1200 | ACIR | 8~10 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | ACLR | 30~35 |
|  | Mediatek | 1.5 | Y | TN cluster | GEO | ACLR | 22.88 |
|  | Mediatek | 1.5 | Y | TN cluster | LEO-1200 | ACLR | 22.88 |
|  | Thales | 1.5 | Y | TN cluster | GEO | ACIR | <13 |
|  | Thales | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <13 |

Isolation distance: 0km

| TP Loss | Company | Isolation | Wrap-around | Statistics | SAN Orbit | ACLR/ACIR | Required results (dBc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average | Samsung | 0 | Y | TN cluster | GEO | ACIR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACIR | <20 |
|  | Samsung | 0 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | Ericsson | 0 | Y | Impact sectors | GEO | ACIR | 30 |
|  | Ericsson | 0 | Y | Impact sectors | LEO-1200 | ACIR | <36 |
| 5%-tile | Samsung | 0 | Y | TN cluster | GEO | ACIR | 25~30 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACIR | 25~30 |
|  | Samsung | 0 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | Ericsson | 0 | Y | Impact sectors | GEO | ACIR | 36~38 |
|  | Ericsson | 0 | Y | Impact sectors | LEO-1200 | ACIR | 36~38 |

Proposal 4: For NR-NTN PC1.5, the required ACIR is below 20 dBc when isolation distance of 1.5km assumed. The final exact required ACIR and ACLR from co-ex study can be derived from all submitted results.

3.3 Observations for NR-NTN PC1

Isolation distance: 1.5km

| TP loss | Company | Isolation | Wrap-around | Statistics | SAN Orbit | ACLR/ACIR | Required results (dBc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average | Samsung | 1.5 | Y | TN cluster | GEO | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Ericsson | 1.5 | Y | Impact sectors | GEO | ACIR | >10 |
|  | Ericsson | 1.5 | Y | Impact sectors | LEO-1200 | ACIR | 10~12 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | ACLR | 20~25 |
|  | Mediatek | 1.5 | Y | TN cluster | GEO | ACLR | 24.88 |
|  | Mediatek | 1.5 | Y | TN cluster | LEO-1200 | ACLR | 24.8 |
|  | Thales | 1.5 | Y | TN cluster | GEO | ACIR | <13 |
|  | Thales | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <13 |
| 5%-tile | Samsung | 1.5 | Y | TN cluster | GEO | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | ACLR | 20~25 |
|  | Ericsson | 1.5 | Y | Impact sectors | GEO | ACIR | 8~10 |
|  | Ericsson | 1.5 | Y | Impact sectors | LEO-1200 | ACIR | 10~12 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | ACLR | 30~35 |
|  | Mediatek | 1.5 | Y | TN cluster | GEO | ACLR | 24.88 |
|  | Mediatek | 1.5 | Y | TN cluster | LEO-1200 | ACLR | 24.88 |
|  | Thales | 1.5 | Y | TN cluster | GEO | ACIR | <13 |
|  | Thales | 1.5 | Y | TN cluster | LEO-1200 | ACIR | <13 |

Isolation distance: 0km

| TP Loss | Company | Isolation | Wrap-around | Statistics | SAN Orbit | ACLR/ACIR | Required results (dBc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average | Samsung | 0 | Y | TN cluster | GEO | ACIR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACIR | <20 |
|  | Samsung | 0 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | Qualcomm | 0 | Y | TN cluster |  | ACIR | 25~30 |
|  | Qualcomm | 0 | N | TN cluster |  | ACIR | 5~10 |
|  | Ericsson | 0 | Y | Impact sectors | GEO | ACIR | 32 |
|  | Ericsson | 0 | Y | Impact sectors | LEO-1200 | ACIR | <36 |
| 5%-tile | Samsung | 0 | Y | TN cluster | GEO | ACIR | 30~35 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACIR | 30~35 |
|  | Samsung | 0 | Y | TN cluster | GEO | ACLR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | ACLR | <20 |
|  | Qualcomm | 0 | Y | TN cluster |  | ACIR | 40~45 |
|  | Qualcomm | 0 | N | TN cluster |  | ACIR | 15~20 |
|  | Ericsson | 0 | Y | Impact sectors | GEO | ACIR | 38~40 |
|  | Ericsson | 0 | Y | Impact sectors | LEO-1200 | ACIR | 38~40 |

Proposal 5: For NR-NTN PC1, the required ACIR is below 20 dBc when isolation distance of 1.5km assumed. The final exact required ACIR and ACLR from co-ex study can be derived from all submitted results.

3.4 Observations for IoT-NTN PC2

Isolation distance: 1.5km

| TP loss | Company | Isolation | Wrap-around | Statistics | SAN Orbit | SCS / UE# | ACLR/ACIR | Required results (dBc) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | Samsung | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | 15kHz / 9 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | <20 |
|  | Ericsson | 1.5 | Y | Impacted sectors | GEO | 15kHz / 9 | ACIR | 2~4 |
|  | Ericsson | 1.5 | Y | Impacted sectors | LEO-1200 | 15kHz / 9 | ACIR | 4~6 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | Multi tone | ACLR | 20~25 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | 15kHz / 9 | ACLR | 20~25 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | 20~25 |
| 5%-tile | Samsung | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | 15kHz / 9 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | <20 |
|  | Ericsson | 1.5 | Y | Impacted sectors | GEO | 15kHz / 9 | ACIR | 0 |
|  | Ericsson | 1.5 | Y | Impacted sectors | LEO-1200 | 15kHz / 9 | ACIR | 0 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | Multi tone | ACLR | 35~40 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | 15kHz / 9 | ACLR | 35~40 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | 35~40 |

Isolation distance: 0km

| TP Loss | Company | Isolation | Wrap-around | Statistics | SAN Orbit | SCS / UE# | ACLR/ACIR | Required results (dBc) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | Samsung | 0 | Y | TN cluster | GEO | 3.75kHz / 36 | ACIR | 20~25 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACIR | <20 |
|  | Samsung | 0 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACLR | <20 |
|  | Ericsson | 0 | Y | Impacted sectors | GEO | 15kHz / 9 | ACIR | 26~28 |
|  | Ericsson | 0 | Y | Impacted sectors | LEO-1200 | 15kHz / 9 | ACIR | 26~28 |
| 5%-tile | Samsung | 0 | Y | TN cluster | GEO | 3.75kHz / 36 | ACIR | 30~35 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACIR | 25~30 |
|  | Samsung | 0 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | 20~25 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACLR | <20 |
|  | Ericsson | 0 | Y | Impacted sectors | GEO | 15kHz / 9 | ACIR | 26~28 |
|  | Ericsson | 0 | Y | Impacted sectors | LEO-1200 | 15kHz / 9 | ACIR | 26~28 |

Proposal 6: For IoT-NTN PC2, the required ACIR is below 20 dBc when isolation distance of 1.5km assumed. The final exact required ACIR and ACLR from co-ex study can be derived from all submitted results.

3.5 Observations for IoT-NTN PC1

Isolation distance: 1.5km

| TP loss | Company | Isolation | Wrap-around | Statistics | SAN Orbit | SCS / UE# | ACLR/ACIR | Required results (dBc) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | Samsung | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | 15kHz / 9 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | <20 |
|  | Ericsson | 1.5 | Y | Impacted sectors | GEO | 15kHz / 9 | ACIR | 8~10 |
|  | Ericsson | 1.5 | Y | Impacted sectors | LEO-1200 | 15kHz / 9 | ACIR | 8~10 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | Multi tone | ACLR | 25~30 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | 15kHz / 9 | ACLR | 25~30 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | 25~30 |
| 5%-tile | Samsung | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACIR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | <20 |
|  | Samsung | 1.5 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | 15kHz / 9 | ACLR | <20 |
|  | vivo | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | <20 |
|  | Ericsson | 1.5 | Y | Impacted sectors | GEO | 15kHz / 9 | ACIR | 2~4 |
|  | Ericsson | 1.5 | Y | Impacted sectors | LEO-1200 | 15kHz / 9 | ACIR | 0~2 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | Multi tone | ACLR | 40~45 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | 15kHz / 9 | ACLR | 35~40 |
|  | ZTE | 1.5 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | 35~40 |

Isolation distance: 0km

| TP Loss | Company | Isolation | Wrap-around | Statistics | SAN Orbit | SCS / UE# | ACLR/ACIR | Required results (dBc) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average | Samsung | 0 | Y | TN cluster | GEO | 3.75kHz / 36 | ACIR | 20~25 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACIR | <20 |
|  | Samsung | 0 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | <20 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACLR | <20 |
|  | Ericsson | 0 | Y | Impacted sectors | GEO | 15kHz / 9 | ACIR | 32~34 |
|  | Ericsson | 0 | Y | Impacted sectors | LEO-1200 | 15kHz / 9 | ACIR | 30~32 |
| 5%-tile | Samsung | 0 | Y | TN cluster | GEO | 3.75kHz / 36 | ACIR | 35~40 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACIR | 30~35 |
|  | Samsung | 0 | Y | TN cluster | GEO | 3.75kHz / 36 | ACLR | 25~30 |
|  | Samsung | 0 | Y | TN cluster | LEO-1200 | 3.75kHz / 36 | ACLR | <20 |
|  | Ericsson | 0 | Y | Impacted sectors | GEO | 15kHz / 9 | ACIR | 30~32 |
|  | Ericsson | 0 | Y | Impacted sectors | LEO-1200 | 15kHz / 9 | ACIR | 34 |

Proposal 7: For IoT-NTN PC1, the required ACIR is below 20 dBc when isolation distance of 1.5km assumed. The final exact required ACIR and ACLR from co-ex study can be derived from all submitted results.

3.6 Other observations

Observation 4: The isolation distance is one of the most important factors which separate the results into two categories.

Observation 5: The 1.5km isolation distance was agreed as baseline assumptions.

Issue 1-1: Isolation distance Agreement: - The baseline is to use 2 x ISD (ISD = 750 meters) as the isolation distance

Proposal 8: To disucss co-ex study results separately for isolation distances assuming 1.5km and 0km.

4	ACIR and ACLR

The ACIR in co-ex study was used to describe the overall interference power considering both Tx leakage from aggressor and Rx rejection from victim.

Once co-ex study concluded a required ACIR, we usually used the following equation to derive ACLR. It’s worth noting that the derived ACLR from ACIR by using NR BS ACS through equation is a flat ACLR that has a reference bandwidth same as the victim bandwidth.

Figure 4-1: The ACLR models and its reference bandwidth

Proposal 9: For co-ex study result conclusion discussion, to clarify and align understanding that derived ACLR from agreed ACIR is an equivalent ACLR in victim bandwidth.

Proposal 10: For co-ex study result conclusion discussion, to follow similar principles in deriving ACIR as documented in TR 38.863 in Rel-17.

6.3.2 Methods and principle to process co-existence simulation results In order to process the co-existence simulation results received for all different scenarios and assumptions, the following steps are adopted: - Step 1: Discuss and agree on the most stringent scenario(s) for each scenario (Scenario 1, 2, 3…,6); - Step 2: Discuss and determine the required ACIR from results of the most stringent case(s) for each scenario; - Step 3: Use equation to derive corresponding ACLR or ACS from the agreed ACIR for each scenario It is noted that the averaged ACIR for the most stringent case in each scenario would be derived by taking the average among the interpolated ACIR results derived from each company’s results for that case. Moreover, the following considerations are adopted to deal with major disputes for the selected cases’ results in each scenario: - If the required ACIR results, from the contributor who did not participate or their results is still not well-aligned in calibration table, has a difference larger than 10 dB with most others, this result can be not considered in the discussion. - If the required ACIR results, from one contributor, has a difference larger than 10 dB with most others, this result can be not considered in the discussion. The processed results by adopting above principles and methods for scenarios 1 to 7 identified in Table 6.1-2 are captured in section 6.4. It is noted that due to the space limitation, only part of the simulation results for each case are presented, and the whole results for all studied options, as listed in Table 6.1-1 and section 6.2, can be found in Annex C.

5	Conclusion

For the NTN HPUE co-ex studies, we propose the following.

Observation 1: We had agreed in #112 meeting to use Urban case as worst case over Rural and other deployment scenarios.

Observation 2: From the received results, it also aligns with the previous agreement that Urban results are worse than Rural results.

Proposal 1: For co-ex study result conclusion discussion, in principle, to use Urban results instead of Rural results which aligns with previous agreement and worst case.

Observation 3: From the received results, it indicated that the NR BS with AAS antenna has more impact than the results assuming BS with non-AAS antenna.

Proposal 2: For co-ex study result conclusion discussion, in principle, to use AAS results instead of non-AAS results as worst case.

Proposal 3: For NR-NTN PC2, the required ACIR is below 20 dBc when isolation distance of 1.5km assumed. The final exact required ACIR and ACLR from co-ex study can be derived from all submitted results.

Proposal 4: For NR-NTN PC1.5, the required ACIR is below 20 dBc when isolation distance of 1.5km assumed. The final exact required ACIR and ACLR from co-ex study can be derived from all submitted results.

Proposal 5: For NR-NTN PC1, the required ACIR is below 20 dBc when isolation distance of 1.5km assumed. The final exact required ACIR and ACLR from co-ex study can be derived from all submitted results.

Proposal 6: For IoT-NTN PC2, the required ACIR is below 20 dBc when isolation distance of 1.5km assumed. The final exact required ACIR and ACLR from co-ex study can be derived from all submitted results.

Proposal 7: For IoT-NTN PC1, the required ACIR is below 20 dBc when isolation distance of 1.5km assumed. The final exact required ACIR and ACLR from co-ex study can be derived from all submitted results.

Proposal 8: To disucss co-ex study results separately for isolation distances assuming 1.5km and 0km.

Proposal 9: For co-ex study result conclusion discussion, to clarify and align understanding that derived ACLR from agreed ACIR is an equivalent ACLR in victim bandwidth.

Proposal 10: For co-ex study result conclusion discussion, to follow similar principles in deriving ACIR as documented in TR 38.863 in Rel-17.

6	Reference

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung

[2] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung

[3] R4-2419641, Collected NTN HPUE co-existence results, Samsung