3GPP TSG-RAN WG4 Meeting #113															R4-2419669

Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 		Further discussion on the FR2-NTN terminology consideration as FR2-1

Agenda Item:	8.2

Document for:	Discussion

# Introduction

Based on LS on FR2-NTN inclusion to specifications (R4-2414906; R1-2407406), discussion on the FR2-NTN terminology was initiated during RAN4#112-bis meeting, resulting in a WF (R4-2417185) extracted below.

In response to the LS on FR2-NTN inclusion to specifications (R1-2407406), RAN4 discussed the issue and reached the following agreements. - Whether FR2-NTN is regarded as FR2-1 when referenced from other specification RAN4 confirms to RAN1 that FR2-NTN follows FR2-1 procedure. - Whether and how to modify Note 2 in the table (38.101-5 and 38.105) RAN4 agrees to further consider the following options: - Option 1: Remove both Note 1 and Note 2 - Option 2: Keep Note 1 and remove Note 2 - Option 3a: Revise Note 2 (FR2 -> FR2-1) - Option 3b: Update NOTE 2 to “NTN bands within this frequency range are to follow FR2-1 band procedure when referenced from other specifications.” - Option 3c: Update NOTE 2 to: In other specifications, when FR2-NTN is not explicitly defined, NTN bands within this frequency range are regarded as FR2, or FR2-1 when there is a differentiation between FR2-1 and FR2-2.” - Option 3d: Update NOTE 2 to “NTN bands within this frequency range are regarded as a FR2 band when referenced from other specifications, if applicable.”

In this contribution, we provide further analysis of the FR2-NTN terminology in relation to the RAN1 LS from RAN4#112bis meeting.

# Discussion

## Rel-17 specifications

Based on the previous discussion, it was observed that Rel-17 versions of related specifications (i.e. TS 38.101-5 and TS 38.108) are not aligned, with the SAN specification TS 38.108 being already cleaned-up by removing outstanding [] from FR1-related Note. This can be seen from related extracts below:

Extract from TS 38.101-5, Rel-17:

Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN1; 410 MHz – 7125 MHz. NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.].

Extract from TS 38.108, Rel-17:

Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range (MHz). FR1-NTN (NOTE); 410 – 7125. NOTE: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications..

Proposal 1: Align TS 38.101-5 with the TS 38.108, by removing [] from FR1-related Note in TS 38/.101-5, Table 5.1-1.

## Rel-18 specifications

Issue highlighted by RAN1 LS (R4-2414906; R1-2407406) was related to Rel-18 onwards, where FR2-NTN terminology exists. Considering that there is related FR1 note already in Rel-17 version of specification, it is suggested not to consider Option 1 of the WF (i.e. Remove both Note 1 and Note 2). Furthermore, it is seen more useful to keep Note 2 (or its modification), as opposed to removing it instead. Therefore, it is suggested not to consider Option 2 of the WF either (i.e. Keep Note 1 and remove Note 2).

When it comes to options 3a – 3d, we have the following options available, based on the WF content:

| 3a | NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications. |
| --- | --- |
| 3b | NTN bands within this frequency range are to follow FR2-1 band procedure when references from other specifications. |
| 3c | In other specifications, when FR2-NTN is not explicitly defined, NTN bands within this frequency range are regarded as a FR2, or FR2-1 when there is a differentiation between FR2-1 and FR2-2. |
| 3d | NTN bands within this frequency range are regarded as a FR2 band when references from other specifications, if applicable. |

3a is the most simplistic, while still being in-line with the agreement on the RAN1 feedback (i.e. RAN4 confirms to RAN1 that FR2-NTN follows FR2-1 procedure.). Option 3b states the same as 3a, using modified wording.

Option 3c may be considered as alternative solution, while wording simplification would be still needed.

Option 3d does not seem to solve the issue at all.

With the above, we suggest to pursue with option 3a (while 3c can be considered as fallback solution, with further wording simplification needed):

Proposal 2: For the Note 2 modifications in the table (38.101-5 and 38.108), proceed with option 3a (i.e. (FR2  FR2-1 correction).

## RAN1 LS

There was no LS reply sent band to RAN1 last meeting. Once RAN4 conclusion is reached, feedback to RAN 1 shall be sent out:

Proposal 3: Sent reply LS back to RAN1, based on the agreements from RAN4#112-bis and RAN4#113 meetings.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Align TS 38.101-5 with the TS 38.108, by removing [] from FR1-related Note in TS 38/.101-5, Table 5.1-1.

CR related to the Proposal 1 above was submitted in [1].

Proposal 2: For the Note 2 modifications in the table (38.101-5 and 38.108), proceed with option 3a (i.e. (FR2  FR2-1 correction).

Proposal 3: Sent reply LS back to RAN1, based on the agreements from RAN4#112-bis and RAN4#113 meetings.

Related draft LS was attached in the Annex A.

# References

[1]		R4-2419670 (NR_NTN_solutions-Core) CR on FR1 note clarification in frequency ranges table, TS 38.101-5, Rel-17, Huawei, HiSilicon

[2]		R4-2419671 (NR_NTN_enh-Core) CR on FR2 note clarification in frequency ranges table, TS 38.101-5, Rel-18, Huawei, HiSilicon

[3]		R4-2419672 (NR_NTN_enh-Core) CR on FR2 note clarification in frequency ranges table, TS 38.108, Rel-18, Huawei, HiSilicon

# Annex A: Draft LS

3GPP TSG-RAN WG4 Meeting #113		R4-24xxxxx

Orlando, US, 18th – 22nd November, 2024

Title:	(Draft) Reply LS on FR2-NTN inclusion to specifications

Response to:	LS on FR2-NTN inclusion to specifications from RAN1

Release:	Rel-18

Work Item:	NR NTN (Non-Terrestrial Networks) enhancement (NR_NTN_enh-Core)

Source:	Huawei / RAN4

To:	RAN1, RAN2

Cc:

Contact person:	Michal Szydelko

michal.szydelko@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 thanks RAN1 for highlighting potential FR2-NTN related terminology issues in RAN4 specifications TS 38.101-5 and TS 38.108.

To address concerns raised in the original LS in R1-2407406, RAN4 has agreed to update wording of the Note 2 in the table 5.1-1 of TS 38.101-5 and TS 38.108, as highlighted in table below.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 7125 MHz |
| FR2-NTN (Note 2) | 17300 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications. |  |

RAN4 kindly asks RAN1 and RAN2 to further check applicability of FR2-NTN in their specifications, considering the above clarification in RAN4 specifications TS 38.101-5 and TS 38.108.

# 2	Actions

To RAN1, RAN2:

ACTION: 	RAN4 respectfully asks RAN1 and RAN2 to take the above information into account for their future work and to carefully check the applicability for FR2-NTN in their specifications.

# 3	Dates of next TSG-RAN WG4 meetings

RAN WG4 Meeting #114            	17th – 21st February, 2025			Athens, Greece

RAN WG4 Meeting #114-bis         7th – 11th	 April, 2025			China, CN