3GPP TSG-RAN WG4 Meeting #113															R4-2419678

Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 		Discussion on potential simplification of BS TRP test methods

Agenda Item:	7.11.4.3

Document for:	Discussion

# Introduction

One of the performance part objectives of the NR_BS_RF_req_evo WI is:

Investigate and if possible, simplify BS TRP test methods for FR1 and FR2 by improving the applicability and not removing any of the existing TRP test methods.

In this contribution, we provide initial analysis of the current TRP test methodology applicability in BS specifications.

# Discussion

The above referred TRP-related objective was triggered with the goal to look into TRP applicability aspects, considering potential issues related to test methodology equivalence.

## 3GPP TR 37.941

Referring to TR 37.941 clause 6.3.2, one can find the following description of TRP test methodology applicability to various RF requirements.

Different procedures can be used to evaluate the TRP estimate. These procedures can provide either an accurate assessment or a controlled overestimate of the TRP. The choice of methods is based also on the available test setup, measurement equipment, and the measurement time. This clause describes the methods which are suitable for each type of requirements. Other relevant methods are not precluded. For each method, the test purpose (accurate or overestimate) is pointed out. A summary of TRP measurement procedures and their applicability to different OTA BS requirements is shown in table 6.3.2.1-1. In the following clauses the measurement procedures for different parameters are described under the assumption of equal angle sampling. Similar procedures can be also used with other types of spherical grids, given that the proper reference steps Δθref and Δϕref are determined. NOTE: OTA FR2 transmit ON/OFF power is excluded from the table although the core requirement is specified as TRP because conformance is verified through EIRP measurements. Table 6.3.2.1-1: Applicability of TRP measurement methods and chambers to the type of emissions to be measured BS output power; Unwanted emissions: ACLR (Note 1); Unwanted emissions: SEM, OBUE; Spurious emissions (Note 7). Full sphere using reference steps (accurate); X; X; X; X (Note 2). Methods applicable to; Full sphere using sparse sampling (overestimate); X (Note 2, 3). anechoic chambers; Two cuts + Pattern multiplication (accurate) (Note 4); X; X; X. Two/three cuts (overestimate); X; X; X (Note 2, 3). Beam-based directions; X; X (Note 5); X (Note 5). Peak method; X; X (Note 2). Equal sector with peak average; X; X (Note 2). Pre-scan (Note 2); X; X; X. Reverberation chamber; X; X; X; X (Note 2). Note 1: Two TRP measurements are needed. Note 2: Pre-scan is needed to identify the frequencies of interest. Pre-scan can also be applied to ACLR, OBUE and SEM. Note 3: At harmonic frequencies the use of this method is not applicable. Note 4: Pattern multiplication is conditional. Note 5: Applicable if the directivity of corresponding requirement at the reference direction is equivalent to the directivity at the reference direction when BS emits Prated,c,TRP and Prated,c,EIRP. Note 6: If box is blank the method is not excluded but the methodology has not been described in clause 6.3.3 on angular alignment in TRP measurements; if a suitable analysis is shown the method may be applied. Note 7: Applicability of any chamber for the spurious emissions measurements is subject to the supported frequency range of the chamber, and specifically its upper frequency limit..

Different OTA test procedures can be used to evaluate the TRP estimate. None of suitable test methods are excluded, as long as the MU contributors of related test methodology can meet the requirement-specific MU budget (even if this condition is not met, alternative test methods can be still used, but then the requirements on EUT is tightened accordingly).

Consideration of various test methods was always used in RAN4 as principle to derive test requirements. This was done to avoid situation, where a particular test methodology would be standardized and used as competitive leverage against other (test) vendors. This particular aspect is also highlighted in the ETSI TR 103 974 on equivalence of measurement results with different test methods.

Observation 1: None of suitable TRP test method(s) was excluded from consideration in TR 37.941.

## 3GPP TS 38.141-2

Based on findings captured in TR 37.941 on various OTA chambers, NR BS technical specification TS38.141-2 captures various procedures for BS OTA TRP measurments in Annex I:

The annex describes various procedures for BS OTA TRP measurments. These procedures can provide either an accurate or an over-estimate of TRP values. The procedures for an accurate estimate can be applied to all TRP requirements. However, if a TRP requirement does not need accurate TRP estimate then the procedures for over-estimate of TRP may be used in order to have a reasonable OTA test time. Pre-scan does not provide an accurate TRP estimate or over-estimate of TRP. Pre-scan is a fast but coarse method that is used to identify the spurious emission frequencies with emission power as described in annex I.13. A sequential measurement is then made at the emission frequencies, to assess the TRP as described in annex I.2 to annex I.9.

Annex F in AAS BS test specification TS 37.145-2 captures same content.

Observation 2: TS 38.141-2 and TS 37.145-2 (for NR BS and for AAS BS, respectively) capture various TRP measurement methodologies descriptions.

## ETSI TR 103 974

ETSI ERM WGRM group is currently working on the work item on DTR/ERM-RM-284 [1], on the equivalence of measurement results with different test methods, with findings to be captured in the ETSI TR 103 974 [2]. This work was motivated by the fact that discussions on the validity of ETSI standards offering alternative test methods for a single radio parameter have led to delays in standardization. Therefore, the European Commission has triggered related ETSI work, by stating the following:

- One test method for each requirement is the preferred solution;

- However, more than one method may be proposed by ETSI:

- if this fosters innovation, or

- if this is needed to ensure technological neutrality.

If more than one method is proposed by ETSI, the EC wants assurances that the proposed methods:

- are equivalent in terms of accuracy,

- are equivalent in terms of reproducibility, and

- reflect the state of the art.

Related work ETSI TR 103 974 [2] was aiming to provide a discussion on the conditions for harmonised standards to offer alternative test methods.

Initially the work on ETSI TR 103 974 was planned to be concluded April 2024, but there is significant delay already. At the time of writing, version 0.0.4 of the TR was available (Early draft).

TR 103 974 [2] was aiming to discuss on the conditions for harmonised standards to offer different test methods for one requirement, with the focus on typical radio parameters. One particular question which was to be answered was how to ensure equivalent results of a radiated power measurement on different test sites, considering the following OTA test site types:

the Open Area Test Site (OATS),

the Semi-Anechoic Chamber (SAC),

the Fully Anechoic Room (FAR),

the Absorber Lined OATS (ATS),

the Transverse Electro-Magnetic (TEM) cell,

the Reverberation Chamber (RC), and

the Near Field Scanner (NFS).

Based on the lecture of the v0.0.4 of the TR 103 974, one can find the following statements, which were identified as worth highlighting from RAN4 discussion point of view:

Sentence #1: The standard must give clear guidance which method should be used for which measurement.

The above is at least partially fulfilled in TS 38.141-2, by text captured in the description of test procedure for TRP requirements (e.g. OTA BS output power in 6.3.4.2, ACLR in 6.7.3.4.2, Tx spur in 6.7.5.2.4.2, etc.). One example is extracted from clause 6.7.5.2.4.2 on Tx spur test procedure:

The following procedure for measuring TRP is based on directional power measurements as described in annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 4, 5, 7 and 10. When calibrated and operated within the guidance of 3GPP TR 37.941 [29] the measurement methods are applicable and selected depending on availability at the test facility.

In the above text extract, there is reference to Annex I, which contains 10 different TRP measurement procedures within anechoic chambers. Additionally, the RC is indicated as an alternative test method. While the above extract from TS 38.141-2 is technically correct, it seems it can be still further improved in order to address Sentence #1.

Proposal 1: in order to provide clear guidance on the TRP methods to be used for each requirement (in TS 38.141-2, TS 37.145-2) further review content of the TRP procedures sections (e.g. in TS 38.141-2: OTA BS output power in clause 6.3.4.2, ACLR in clause 6.7.3.4.2, Tx spur in clause 6.7.5.2.4.2, etc.) to improve guidance on the test methods to use per requirement.

Referring to the request from the EC, it has stated that one test method for each requirement is the preferred solution. However, more than one method may be in the standard, if

- this fosters innovation, or

- if this is needed to ensure technological neutrality.

TR 103 974 clarifies, that the innovation and technological neutrality mentioned is not of the equipment-under-test, but of the test methods, e.g. to foster innovation, it may be necessary to offer a newer, improved test method as well as the test method that is currently used. Concerning technological neutrality, two different technical implementations of the same test, that are both equally valid and equally accurate, may be offered as alternatives, as restricting the standard to one of them would then have the unwanted effect of selecting only one of these equally valid technologies.

Those observation on bullets a and b are considered to be in-line with legacy RAN4 work, where multiple test methods were considered.

TR 103 974 also captures section on the selection of preferred radiated test site(s). However, its content is still under construction.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: in order to provide clear guidance on the TRP methods to be used for each requirement (in TS 38.141-2, TS 37.145-2) further review content of the TRP procedures sections (e.g. in TS 38.141-2: OTA BS output power in clause 6.3.4.2, ACLR in clause 6.7.3.4.2, Tx spur in clause 6.7.5.2.4.2, etc.) to improve guidance on the test methods to use per requirement.

Referring to the report of the last ERMWGRM-Test methods meeting (2024.09.02) in [3], one can find that there is lack of consensus on the further steps towards conclusion of the work on this TR in ETSI ERM WGRM. Therefore, in order to reassure timely conclusion of related RAN4 discussion, it is suggested to send a LS to ETSI ERM WGRM, asking on the expected completion date.

Proposal 2: Send LS to ETSI ERM WGRM, asking on the expected completion date of the TR 103 974.

Related draft LS was submitted in attached in Annex A below.

# References

[1]		DTR/ERM-RM-284 	On the equivalence of measurement results with different test methods, Equivalence of test methods, ETSI ERM WGRM

[2]		ETSI TR 103 974, On the equivalence of measurement results with different test methods, v.0.0.4, Early draft

[3]		ERMRM(24)000021		ERMWGRM-Test methods meeting report (2024.09.02), https://docbox.etsi.org/ERM/ERMRM/05-CONTRIBUTIONS/2024/ERMRM(24)000021_meeting_report_2024-09-02.docx

# Draft LS

3GPP TSG-RAN WG4 Meeting #113															R4-24xxxxx

Orlando, US, 18th – 22nd November, 2024

Title:	Draft LS to ETSI ERM WGRM on the status of work on equivalence of test methods

Response to:

Release:	Rel-19

Work Item:	NR_BS_RF_req_evo-Perf

Source:	TSG RAN WG4

To:	ETSI ERM WGRM

Cc:	3GPP TSG RAN, ETSI ERM TFES

Contact person:	Michal Szydelko

michal.szydelko@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

3GPP RAN WG4 would like to inform ETSI ERM WGRM, that it is working on the rel-19 work item on NR base station RF requirement evolution for FR1/FR2 and testing (NR_BS_RF_req_evo; expected completion: March 2026). One of the objectives is to investigate and if possible, simplify BS TRP test methods for FR1 and FR2 by improving their applicability. This objective was motivated by the feedback received in relation to the Letter from DG GROW on Alternative testing methods and environmental profile.

It was identified, that one of ETSI ERM WGRM work item in DTR/ERM-RM-284 is related to the equivalence of measurement results with different test methods.

3GPP RAN WG4 would like to ask ETSI ERM WGRM for feedback on the progress of DTR/ERM-RM-284 work item on the equivalence of test methods, and in particular on the expected completion date of the ETSI TR 103 974.

# 2	Actions

To ETSI ERM WGRM

ACTION: 	3GPP RAN WG4 asks ETSI ERM WGRM to provide feedback on progress of the DTR/ERM-RM-284 work item on the equivalence of test methods, and in particular on the expected completion date of the ETSI TR 103 974.

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN4 Meeting #114			Athens, Greece				17-21 February, 2025

TSG-RAN4 Meeting #114-bis		China, CN					7-11 April, 2025