# 3GPP TSG-RAN WG4 Meeting # 113                                                         R4-2419696

# Orlando, USA, 18 – 22 November 2024

# Title:	Missing n85 3MHz RSD

# Agenda Item:	5.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents the missing 1Tx and 2Tx Reference Sensitivity Degradation (RSD) for band n85 3 MHz channel bandwidth (CBW).

# Discussion

In Rel-18, the 3 MHz CBW REFSENS requirements were added for band n85, and this band supports PC2. However, RSD requirements are only specified for CBW greater than 5 MHz. In this contribution, we propose to introduce the missing 3 MHz RSD requirements using the method presented in [1] by reverse engineering the interference levels that correspond to the agreed 1Tx PC3 REFSENS levels within 0.1 dB. We assume the following:

- For 1Tx RSD, we assume 9 dB interference imbalance between the Main and Diversity antenna ports, and that the PC2 interference is 3 dB higher than the PC3 interference.

- For 2Tx RSD, we assume 0.3 dB reverse IMD.

The band n85 RSD can be found in the Annex Figure 1.

Observation 1: 1Tx and 2Tx RSD requirements are missing in Rel-18 for band n85 3MHz CBW.

Proposal 1: For Rel-18, consider introducing the following missing 3 MHz CBW RSD for band n85: 1Tx RSD =0.3 dB, 2Tx RSD=0.5 dB.

# Conclusion

This paper captures our views on the band n28 RSD. We also propose to consider introducing the missing band n85 3MHz RSD in Rel-18 based on Observation 1.

Observation 1: 1Tx and 2Tx RSD requirements are missing in Rel-18 for band n85 3MHz CBW.

Proposal 1: For Rel-18, consider introducing the following missing 3 MHz CBW RSD for band n85: 1Tx RSD =0.3 dB, 2Tx RSD=0.5 dB.

# References

- R4-2405956 Guidelines for PC2 FDD Dual-TX MSD, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, Skyworks Solutions, Inc.

# Annex

Figure 1: Band n85 3MHz RSD.