# 3GPP TSG-RAN WG4 Meeting # 113                                                                 R4-2419699

# Orlando, USA, 18 – 22 November 2024

# Title:	1UL HPUE NR-CA MSD Simplifications for Rel-19

# Agenda Item:	7.1.1.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents basic rules to simplify the one uplink (1UL) HPUE MSD requirements based on observations captured in [1], and as a follow up to the agreed-upon WF [2].

# Discussion

As proponents since the RAN4 meeting #111 [3, 4, and 5], we support WF [2] Option 1. Option 1 describes the practice of specifying the HPUE MSD requirements for inter-band CA in TS 38.101-1 for Rel-19, and introduces the MSD requirements using PC3 to HPUE MSD look-up tables (LUTs), as captured in Proposals 1, 2, and 3 below.

The proposed LUTs are based on Option 2 of Observation 5 of [1], as follows:

- Worst-case 1Tx interference imbalance is assumed. Thus, 9 dB for PC2 2Tx and PC1.5.

- A PC3 MSD of less than 0.5 dB is ignored.

- To reduce complexity, an “A-MPR” type format is proposed, using simple “stair-case” equations with:

  - Entries: a maximum of two PC3 MSD ranges per LUT, thereby providing compact LUTs.

  - Accuracy: within +- 0.5 dB from the calculated PC2 1Tx/2Tx and PC1.5 – see Annex. The accuracy may be increased at the expense of additional entries. For example, such as those proposed in [3].

Proposal 1: Handling of the Rel-19 TS 38.101-1

- The legacy Rel-18 MSD test points are ported to Rel-19, in their current MSD table format, and are not modified for all power classes, and all types of 1UL and 2UL MSDs.

- For new Rel-19 inter-band CA combinations:

  - The default power class 1UL MSD is specified, using the legacy MSD tables.

  - The 1UL HPUE MSD requirements are specified. However, the MSD requirements are no longer captured in the legacy HPUE MSD tables. The 1UL HPUE MSDs are specified using PC3 to HPUE LUTs.

  - The 1UL HPUE LUTs are captured in TS 38.101-1, Clause 7.3A.2.3, Reference sensitivity power level for Inter-band CA. The following eligibility criteria should be captured in this clause’s core requirements:

The 1UL HPUE MSD LUT simplification applies to Rel-19 CA band combinations, for which:

  - A valid 1UL HPUE configurations is specified in the two-band inter-band CA configuration tables (Table 5.5A.3.1-1a ~ 5.5A.3.1-1n), and,

  - A default power class 1UL MSD test point is specified, and,

  - A 1UL HPUE MSD test point is not already specified in either Table 7.3A.4-2a, Table 7.3A.4-4a-1, Table 7.3A.4-4a-2, Table 7.3A.4-4b, Table 7.3A.6-1a-1, Table 7.3A.6-1a-2, or Table 7.3A.6-1b.

Proposal 2: Handling of the Rel-19 TPs for TR

- The procedure is unchanged for the default power class: the 1UL MSD must be studied, and, where applicable, the default PC MSD is captured in the TP.

- The 1UL HPUE MSD requirements are no longer studied. The 1UL HPUE MSD requirements may be captured by simply referring to the TS 38.101-1 LUTs.

Proposal 3: For new Rel-19 inter-band CA combinations within FR1, the 1UL HPUE MSD requirements, due to uplink harmonic, harmonic mixing, and cross-band isolation interference, are specified as:

- PC21Tx MSD = PC31Tx MSD + PC21Tx

- PC22Tx MSD = PC31Tx MSD + PC22Tx

- PC1.5 MSD = PC31Tx MSD + PC1.5

where PC21Tx, PC22Tx and PC1.5 are specified in the following LUTs:

| Agreed PC3 MSD | PC21Tx |
| --- | --- |
| PC3MSD ≥ 0.5dB | min(3, ceil(0.38*PC3MSD + 0.2,0.5)) |

| Agreed PC3 MSD | PC22Tx |
| --- | --- |
| PC3MSD > 5dB | min(7.9, ceil(0.3*PC3MSD + 3.6,0.5)) |
| 0.5dB ≤ PC3MSD ≤ 5.0dB | ceil[0.9*PC3MSD] |

| Agreed PC3 MSD | PC1.5 |
| --- | --- |
| PC3MSD > 5dB | min(10.9, ceil(0.55*PC3MSD + 4.3,0.5)) |
| 0.5dB ≤ PC3MSD ≤ 5.0dB | ceil(1.3*PC3MSD + 1.3) |

# Conclusion

This paper captures our views on simplifying the 1UL MSD requirements for HPUE NRCA inter-band combinations. The main benefits to our proposals are:

- For new Rel-19 CA combinations, RAN4 no longer needs to study the 1UL HPUE MSD requirements. Only the MSD of the default power class needs to be evaluated. This will help in optimizing  time for RAN4.

- Errors and inconsistencies observed in [1] are “automatically” removed:

  - For all MSD types: Harmonic interference, Cross-band isolation, or Rx harmonic mixing.

  - For all 1UL HPUE power classes: PC2 and PC1.5.

  - For all front-end architectures: 1Tx and 2Tx.

Proposal 1: Handling of the Rel-19 TS 38.101-1:

- The legacy Rel-18 MSD test points are ported to Rel-19, in their current MSD table format, and are not modified for all power classes, and all types of 1UL and 2UL MSDs.

- For new Rel-19 inter-band CA combinations:

  - The default power class 1UL MSD is specified using the legacy MSD tables.

  - The 1UL HPUE MSD requirements are specified. However, the MSD requirements are no longer captured in the legacy HPUE MSD tables. The 1UL HPUE MSDs are specified using PC3 to HPUE LUTs.

  - The 1UL HPUE LUTs are captured in TS 38.101-1, Clause 7.3A.2.3, Reference sensitivity power level for Inter-band CA.  The following eligibility criteria should be captured in this clause’s core requirements:

The 1UL HPUE MSD LUT simplification applies to Rel-19 CA band combinations for which:

  - A valid 1UL HPUE configurations is specified in the two-band inter-band CA configuration tables (Table 5.5A.3.1-1a ~ 5.5A.3.1-1n), and,

  - A default power class 1UL MSD test point is specified, and,

  - A 1UL HPUE MSD test point is not already specified in either Table 7.3A.4-2a, Table 7.3A.4-4a-1, Table 7.3A.4-4a-2, Table 7.3A.4-4b, Table 7.3A.6-1a-1, Table 7.3A.6-1a-2, or Table 7.3A.6-1b.

Proposal 2: Handling of the Rel-19 TPs for TR

- The procedure is unchanged for the default power class: the 1UL MSD must be studied, and, where applicable, the default PC MSD is captured in the TP.

- The 1UL HPUE MSD requirements are no longer studied. The 1UL HPUE MSD requirements may be captured by simply referring to the TS 38.101-1 LUTs.

Proposal 3: For new Rel-19 inter-band CA combinations within FR1, the 1UL HPUE MSD requirements due to uplink harmonic, harmonic mixing, and cross-band isolation interference, are specified as:

- PC21Tx MSD = PC31Tx MSD + PC21Tx

- PC22Tx MSD = PC31Tx MSD + PC22Tx

- PC1.5 MSD = PC31Tx MSD + PC1.5

where PC21Tx, PC22Tx and PC1.5 are specified in the following LUTs:

| Agreed PC3 MSD | PC21Tx |
| --- | --- |
| PC3MSD ≥ 0.5dB | min(3, ceil(0.38*PC3MSD + 0.2,0.5)) |

| Agreed PC3 MSD | PC22Tx |
| --- | --- |
| PC3MSD > 5dB | min(7.9, ceil(0.3*PC3MSD + 3.6,0.5)) |
| 0.5dB ≤ PC3MSD ≤ 5.0dB | ceil[0.9*PC3MSD] |

| Agreed PC3 MSD | PC1.5 |
| --- | --- |
| PC3MSD > 5dB | min(10.9, ceil(0.55*PC3MSD + 4.3,0.5)) |
| 0.5dB ≤ PC3MSD ≤ 5.0dB | ceil(1.3*PC3MSD + 1.3) |

# References

- R4-2419712 1UL HPUE MSD Observations, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions Inc.

- R4-2417098, WF on HPUE RF requirements, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Samsung, Skyworks, Nokia.

- R4-2407156, Guidelines for FDD PC2 Harmonic MSD Analysis, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Japan, Skyworks Solutions, Inc.

- R4-2413068 Update on Harmonic MSD simplification, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Skyworks Solutions. Inc.

- R4-2416252 HPUE CA/DC MSD Simplification, 3GPP TSG-RAN WG4 Meeting #112bis, Hefei, China, Skyworks Solutions, Inc.

# Annex

Figure 1: Left: PC2 1Tx LUT accuracy, Right: LUT-based PC2 1Tx (dots), calculated PC2 1Tx (line).

Figure 2: Left: PC2 2Tx LUT accuracy, Right: LUT-based PC2 2Tx (dots), calculated PC2 2Tx (line).

Figure 3: Left: PC1.5 LUT accuracy, Right: LUT-based PC1.5 (dots), calculated PC1.5 (line).