3GPP TSG-RAN WG4 Meeting #113 	R4-2419706

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.25.2

Source: 	Ericsson

Title:	Draft LS response on UE gaps and restrictions that are caused by RRM measurements

Document for:	Approval

# Introduction

The RAN4-related objectives of the Rel-19 XR Phase 3 WI are captured in [1]. The RAN4#112-bis WF is in [5]. The RAN4#112 agreements and WF are in [4].

RAN4 also received RAN1 LS [2], were RAN4 is requested to take the below information from [2] into account and decide whether or not to introduce any UE assistance information related to measurement occasions:

RAN1 Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

Yet another RAN1 LS was received in [3], on UE’s gaps/restrictions that are caused by RRM measurements, where RAN1 is asking to review the working assumption, provide feedback if there are any concerns, and decide the value(s) for the minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of the corresponding skipped gap/restriction occasion indicated by the DCI:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

Below we further discuss the RAN1 LSs and the issues related to RAN4 requirements impact for UE operating XR.

# Discussion

## Scenarios

The following was agreed in RAN4#112-bis [5] on the scenarios:

- Frequency ranges

  - Agree on FR1 and FR2-1

- Scenarios

  - Start the discussion for

    - NR-SA

    - Carrier aggregation

Frequency range FR2-2 does not seem to be so relevant for XR and therefore should not be considered in this WI.

- Proposal 1: FR2-2 is not considered in this WI.

In RAN4#112-bis, also DC deployments were discussed for this WI. We further note that, e.g., EN-DC deployment comes with many different inter-RAT measurements, which require MGs and may complicate MG cancelation procedure, but which are not very relevant for XR scenarios. Furthermore, the WID [1] mentions only NR SA and NR-DC, but not EN-DC or NE-DC:

- Observation 1: Only NR SA and NR-DC are explicitly supported for XR, according to WID.

- Proposal 2: EN-DC is not considered in this WI.

- Proposal 3: NE-DC is not considered in this WI.

## Measurement types

- Proposal 4: At least the following NR measurement types with respect to carrier frequency should be considered in the requirements discussions for XR:

  - NR inter-frequency with MG,

  - NR intra-frequency with MG.

- Proposal 5: The following inter-RAT measurements should be considered in the requirements discussions for XR:

  - NR – E-UTRAN FDD,

  - NR – E-UTRAN TDD.

- Proposal 6: If EN-DC scenario is supported in RAN4 XR requirements, then for UE in EN-DC, the XR-related requirements apply when the UE is not configured with the following inter-RAT measurements (can only be configured by E-UTRAN PCell):

  - E-UTRAN – UTRAN FDD/TDD,

  - E-UTRAN – GSM, GSM, cdma2000 1 x RTT and HRPD,

  - FFS: E-UTRAN – NR.

## DRX impact on MG cancellation

In addition to all the above considerations, we also note that a UE can be configured with DRX, which also has own periodicities (which can be longer or shorter than MGRP) and DRX ON duration, not to mention that DRX and XR can be configured by different network nodes. When configured with DRX, the UE is not required to perform measurements more than once per DRX cycle (which may make measurements less frequent than with MGRP without DRX), and the UE is not expected to monitor control channel outside DRX ON time period. Hence, not all MG occasions are relevant for cancelling in favour of XR.

- Proposal 7: DRX configuration needs to be considered in discussions on MG cancellation, e.g., because:

  - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided.

  - UE can be configured with more than one DRX configurations,

  - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations.

## Types of MGs

In [4], RAN4 ad hoc discussion was captured on MG types to be considered for Rel-19 XR enhancements. RAN4 needs to conclude on this issue. Type 1 MGs can be taken as a baseline for MG cancelling. Relevance for cancellation of other MG types and MG purposes needs further confirmation in RAN4. For example, since the configured PRS (positioning) measurements may be intended for emergency positioning, it is proposed that MGs associated with PRS measurements shall not be cancelled. As of concurrent gaps, it is difficult to discuss MG cancellation for UE configured with concurrent MGs, until the single MG pattern framework is settled.

Option 1 from [5] (“Option 1: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.”) is not agreeable either, because the requirements and scenarios are different, so it is better to focus on a baseline MG type.

- Proposal 8.1: RAN4 considers Type 1 MGs for MG cancelling in Rel-19 XR enhancements, as a baseline.

- Proposal 8.2: MGs associated with PRS (positioning) measurements shall not be cancelled.

- Proposal 8.3: MUSIM MGs shall not be cancelled.

- Proposal 8.4: FFS: Network controlled pre-configured MGs, at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 8.5: FFS: Concurrent MGs, at least until the core requirements impacts are settled for Type 1 MGs.

## RRM requirements enhancements for XR

This WI aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions reserved for RRM measurements, while keeping impact to mobility performance limited. This was studied in TR 38.835 B.1.7.

The RRM measurement core and performance requirements are tightly linked to the number of measurement occasions. If some of the measurement occasions are not available for the measurements, the existing requirements may not be met, at least at the existing side conditions. Hence, to maintain the RRM measurement performance, two approaches could be envisioned:

- Approach 1: The simplest approach would be to limit the XR operation to unused measurement occasions, e.g., in good radio conditions, when RRM measurements could be completed by UE faster. No extension of the RRM measurement period would be needed in this case, but the availability of the unused measurement occasions may not be always guaranteed, e.g., at the cell edge, resulting in no time-frequency resources available for XR for UEs in poor radio conditions, e.g., when the interference is high and/or along cell edges.

- Approach 2: A more practical approach is to compensate for the configured measurement occasions reallocated to the XR operation by extending the RRM measurement period during the XR operation (the compensation means a longer measurement period compared to the UE operation without XR), to ensure the availability of the sufficient number of measurement occasions to complete the RRM measurements at the required accuracy level for any UE, regardless of the experienced radio conditions.

- Approach 3: Combination of Approach 1 and Approach 2, i.e., selectively (upon the need) extend the measurement period only for UEs in poor radio conditions.

With Approach 2 or Approach 3, the extension of the measurement period can be fixed or pre-defined or can be dynamically extendable based on the availability of the measurement occasions during the UE XR operation. The choice of how the extension is defined depends on the RAN1/RAN2 solution and the specified signalling, but the dynamically extendable RRM measurement period may be more suitable for XR due to the dynamic nature of the XR traffic.

- Proposal 9: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

- Proposal 10: RAN4 to discuss the following enhancement methods to the legacy RRM measurement period requirements to account for the UE XR operation:

  - pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor),

  - dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

Even Type 1 MGs are configured by several configuration parameters, including MGL and MGRP and can be per-UE or per-FR. Not all MG configurations are equally important for XR, when MGs are cancelled to reallocate resource to XR.

- Proposal 11: RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL,

  - maximum MGL,

  - maximum MGRP (e.g., to avoid too long delays), etc.

At the same time, it is also noted that when some of the measurement occasions are used for the UE XR operation, there will also be impact on RRM measurement samples and how they are combined, since the samples may become non-equally spaced or disrupted, resulting in inaccurate or lost measurements, timing, etc., even if the total measurement period is met. This may result in the RRM measurement accuracy degradation.

- Proposal 12: RAN4 to discuss the impact on RRM samples and how they are combined with MG cancellation for XR, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be discussed.

- Proposal 13: The measurement accuracy requirements are not impacted by MG cancellation.

## On the time offset

RAN1 LS [3] suggests that RAN4 discusses the minimum time offset between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

- Proposal 14: RAN4 can confirm the RAN1 working assumption, with the understanding that the UE will expect XR data on the same carrier frequency, on which the corresponding DCI is received.

The maximum time offset, which needs to be considered at least when designing RAN4 test cases, is limited by the configured MGRP, considering that according to the RAN1 solution, the DCI indication applies to the closest MG after the minimum time offset. The minimum time offset must include at least the DCI decoding time, e.g., derived according to TS 38.214. We also note that the UE is configured with measurement gap timing advance (MGTA) and therefore can actually start the measurement gap even earlier by an amount of RF re-tuning time, i.e., 0.5 ms for measurements in FR1 and 0.25 ms for FR2. Hence, the minimum time offset must also account for the MGTA configuration, which may also depend on whether this is for FR1 or FR2.

- Proposal 15: RAN4 will define the minimum time offset between the end of the received DCI and the MG indicated by the DCI.

- Proposal 16: The minimum time offset can be different for FR1 and FR2.

- Proposal 17: The minimum time offset between the end of the PDCCH symbol carrying DCI and the first subframe of the target measurement gap includes:

  - DCI processing time,

  - measurement gap timing advance (MGTA) configuration,

  - FFS any other delays.

- Proposal 18: FFS The minimum time offset accounts for UL timing adjustment for UL XR data within the target MG.

- Proposal 19: FFS The minimum time offset accounts for a guard period for DL-to-UL switching for UL XR data within the target MG.

A draft LS is provided in the Annex below.

## On UE assistance information for XR

RAN1 has been discussing UE assistance data to suggest which and/or how RRM measurement occasions (e.g., in measurement gaps) are to be reallocated to XR. According to [2], RAN1 discussed whether the impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information:

- Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period,

- Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

As correctly also pointed out in [2], from the specification point of view, there is no mandated gNB behaviour in response to any of the UE assistance information. This means that there cannot be any explicit or implicit requirement on gNB behaviour upon receiving such assistance data.

- Proposal 20: RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI.

On the information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period: this is more related to RRM measurement performance and therefore can be pre-defined in the specification (TS 38.133). Furthermore, such number may depend on MGRP and DRX cycle length, so it can be more relevant to specify the maximum time between the remaining occasions available for RRM measurements.

- Proposal 21: RAN4 to discuss whether the information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period can be defined in TS 38.133.

On the information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable: RAN4 can discuss whether this problem can be solved by the above, e.g., by defining in the specification the maximum time between the occasions available for RRM measurements. Also, using patterns may limit network scheduling flexibility and at the same time not all UEs will have the same preferred patterns, while the network will have to support all UEs, while will ultimately result in high complexity in the network. This is in addition to that gNB should not be expected to follow the UE patterns, anyway.

With the above observations, it is not possible to specify UE requirements, based on any XR UAI, since this is an implicit requirement on the network to follow the UAI.

- Proposal 22: RAN4 will not specify any XR-related requirement, based on XR UAI.

As discussed in a previous section, there can be an impact of UE DRX configuration on MG cancellation for XR. For the same reason, there can also be some impact on UAI.

- Proposal 23: The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions.

A draft LS is provided in a companion contribution.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: FR2-2 is not considered in this WI.

- Observation 1: Only NR SA and NR-DC are explicitly supported for XR, according to WID.

- Proposal 2: EN-DC is not considered in this WI.

- Proposal 3: NE-DC is not considered in this WI.

- Proposal 4: At least the following NR measurement types with respect to carrier frequency should be considered in the requirements discussions for XR:

  - NR inter-frequency with MG,

  - NR intra-frequency with MG.

- Proposal 5: The following inter-RAT measurements should be considered in the requirements discussions for XR:

  - NR – E-UTRAN FDD,

  - NR – E-UTRAN TDD.

- Proposal 6: If EN-DC scenario is supported in RAN4 XR requirements, then for UE in EN-DC, the XR-related requirements apply when the UE is not configured with the following inter-RAT measurements (can only be configured by E-UTRAN PCell):

  - E-UTRAN – UTRAN FDD/TDD,

  - E-UTRAN – GSM, GSM, cdma2000 1 x RTT and HRPD,

  - FFS: E-UTRAN – NR.

- Proposal 7: DRX configuration needs to be considered in discussions on MG cancellation, e.g., because:

  - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided.

  - UE can be configured with more than one DRX configurations,

  - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations.

- Proposal 8.1: RAN4 considers Type 1 MGs for MG cancelling in Rel-19 XR enhancements, as a baseline.

- Proposal 8.2: MGs associated with PRS (positioning) measurements shall not be cancelled.

- Proposal 8.3: MUSIM MGs shall not be cancelled.

- Proposal 8.4: FFS: Network controlled pre-configured MGs, at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 8.5: FFS: Concurrent MGs, at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 9: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

- Proposal 10: RAN4 to discuss the following enhancement methods to the legacy RRM measurement period requirements to account for the UE XR operation:

  - pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor),

  - dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

- Proposal 11: RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL,

  - maximum MGL,

  - maximum MGRP (e.g., to avoid too long delays), etc.

- Proposal 12: RAN4 to discuss the impact on RRM samples and how they are combined with MG cancellation for XR, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be discussed.

- Proposal 13: The measurement accuracy requirements are not impacted by MG cancellation.

- Proposal 14: RAN4 can confirm the RAN1 working assumption, with the understanding that the UE will expect XR data on the same carrier frequency, on which the corresponding DCI is received.

- Proposal 15: RAN4 will define the minimum time offset between the end of the received DCI and the MG indicated by the DCI.

- Proposal 16: The minimum time offset can be different for FR1 and FR2.

- Proposal 17: The minimum time offset between the end of the PDCCH symbol carrying DCI and the first subframe of the target measurement gap includes:

  - DCI processing time,

  - measurement gap timing advance (MGTA) configuration,

  - FFS any other delays.

- Proposal 18: FFS The minimum time offset accounts for UL timing adjustment for UL XR data within the target MG.

- Proposal 19: FFS The minimum time offset accounts for a guard period for DL-to-UL switching for UL XR data within the target MG.

- Proposal 20: RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI.

- Proposal 21: RAN4 to discuss whether the information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period can be defined in TS 38.133.

- Proposal 22: RAN4 will not specify any XR-related requirement, based on XR UAI.

- Proposal 23: The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions.

# References

[1] RP-240791, WID on XR (eXtended Reality) for NR Phase 3, March 2024.

[2] R4-2411007/R1-2405736, LS on UE assistance information, RAN1, Aug./May 2024.

[3] R4-2414910/ R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN1 LS, Oct./Aug. 2024.

[4] R4-2414043, WF on RRM requirements for XR_Ph3, Nokia, Aug. 2024.

[5] R4-2416864, WF on RRM requirements for XR_Ph3, Nokia, Oct. 2024.

# Annex: Draft LS response on UE gaps and restrictions that are caused by RRM measurements

3GPP TSG-RAN WG4 Meeting #113 	  R4-241xxxx

Orlando, US, 18th – 22nd November, 2024

Title:	[DRAFT] Draft LS response on UE gaps and restrictions that are caused by RRM measurements

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Response to: 	R4-2414910, LS on gaps/restrictions that are caused by RRM measurements

Source:	3GPP TSG-RAN WG4

To:	3GPP TSG-RAN WG1

Contact Person:

Name:					Iana Siomina

E-mail Address:		iana.siomina @ ericsson.com

Attachments:	None

- Overall Description:

In RAN4#112bis, RAN4 has discussed the RAN1 LS on UE gaps and restrictions that are caused by RRM measurements in R4-2414910 (R1-2407561) and concluded the following.

- RAN4 can confirm the RAN1 working assumption,

  - with the RAN4 understanding that the UE will expect XR data on the same carrier frequency, on which the corresponding DCI is received.

- RAN4 will specify the minimum time offset in TS 38.133

  - The minimum time offset between the end of the PDCCH symbol carrying DCI and the first subframe of the target measurement gap to be cancelled will include:

    - DCI processing time,

    - measurement gap timing advance (MGTA) configuration,

    - FFS any other delays.

  - The minimum time offset can be different for FR1 and FR2.

2. Actions:

To RAN1 group:

ACTION: 	RAN4 kindly asks RAN1 to take into account the above information in its work during the WI.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #114 	Athens, GR 			February 17 – November 21, 2025

TSG-RAN WG4 Meeting #114-bis 	China, CN 			April 7 – April 11, 2025