3GPP TSG-RAN WG4 Meeting #113	R4-2419732

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Discussion on LP-WUS RRM Simulations

Agenda item:	7.23.4.1

Document for:	Discussion

Introduction

This contribution provides simulation results for LP-SS and proposes a simulation methodology

Discussion

Simulation methodology

RAN4 has progressed well on the simulation assumption side and for the next meeting, it would be good to coordinate a set of simulations which would be used to evaluate systematically and we can base our core-assumptions on. For instance.

Agree on a set of simulation scenarios each company provides to the next meeting.

LP-SS design impact on LP-RSRP measurement performance

Traditionally, RSRP is used as the measurement metric to determine the cell coverage/range. In the LP-WUS context, it is one of the possible reliable metric to determine whether the UE shall exit the LP-WUS mode. To study the LP-RSRP estimation accuracy, a time correlated fading of 1.28s/2.56s is used with 1e6 samples for the LP-RSRP evaluation under TDL-C 300ns channel.

In this evaluation, we tried to determine the value of X, which represents the number of measurement samples needed within 1.28s that are spaced at equal interval. We have determined LP-RSRP based on the signal energy received during the ON duration corrected with the (noise and interference) energy received in the OFF duration of LP-SS, i.e., ΔE=EON-EOFF.

RSRP estimation error is provided in the following table in relation to number of samples with different LP-SS lengths

| RSRP estimation error in dB |  | Averaging over LP-SS occasions X |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  |  | TDL-C 300ns |  |  |
| RSRP = ΔE |  | X = 2 | X = 4 | X = 8 | X = 2 | X = 4 | X = 8 |
| M = 1 | L = 4 | 1.37 | 1.04 | 0.82 | 6.22 | 4.16 | 2.83 |
|  | L = 6 | 1.16 | 0.9 | 0.73 | 6.27 | 4.19 | 2.87 |
|  | L = 8 | 1.04 | 0.82 | 0.68 | 6.31 | 4.22 | 2.9 |
|  | L = 12 | 0.91 | 0.74 | 0.62 | 6.21 | 4.17 | 2.88 |
| M = 2 | L = 8 | 0.92 | 0.74 | 0.62 | 6.74 | 4.45 | 3.06 |
|  | L = 12 | 0.81 | 0.67 | 0.57 | 6.67 | 4.43 | 3.03 |
|  | L = 16 | 0.75 | 0.62 | 0.54 | 6.71 | 4.46 | 3.05 |
|  | L = 24 | 0.67 | 0.57 | 0.5 | 6.66 | 4.44 | 3.04 |
| M = 4 | L = 16 | 1.08 | 0.9 | 0.78 | 7.86 | 5.25 | 3.67 |
|  | L = 24 | 0.95 | 0.81 | 0.72 | 7.77 | 5.2 | 3.64 |
|  | L = 32 | 0.89 | 0.77 | 0.69 | 7.78 | 5.22 | 3.64 |
|  | L = 56 | 0.79 | 0.7 | 0.64 | 7.76 | 5.2 | 3.64 |

Table 7 LP-RSRP for LP-SS periodicity of 320ms using X = {2,4,8} sample averages without inter-cell interference.

Table 7 and Table 8 compares the RSRP estimation with different values for M={1,2,4}. The comparison also includes the impact of pulse shaping and the use of equal energy over all OOK ON symbols. The RSRP measurement is performed by averaging LP-SS measurements over {0.64s,1.28s,2.56s} durations, i.e., by averaging over {2,4,8} LP-SS occasions, respectively. Even though, the sequence is generated with balanced number of zeros and ones using m-sequence, the concentration of ones and zeros in the resulting sequence introduces power variation at every NR CP-OFDM symbol, since the power is distributed only within a symbol.

| RSRP estimation error in dB |  | Averaging over LP-SS occasions X |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  |  | TDL-C 300ns |  |  |
| RSRP = ΔE |  | X = 2 | X = 4 | X = 8 | X = 2 | X = 4 | X = 8 |
| M = 1 | L = 4 | 1.91 | 1.43 | 1.12 | 6.33 | 4.32 | 2.96 |
|  | L = 6 | 1.58 | 1.21 | 0.97 | 6.39 | 4.28 | 2.92 |
|  | L = 8 | 1.4 | 1.1 | 0.89 | 6.33 | 4.24 | 2.93 |
|  | L = 12 | 1.21 | 0.97 | 0.8 | 6.3 | 4.26 | 2.94 |
| M = 2 | L = 8 | 1.28 | 1.04 | 0.88 | 7.1 | 4.65 | 3.22 |
|  | L = 12 | 1.13 | 0.94 | 0.8 | 6.98 | 4.65 | 3.21 |
|  | L = 16 | 1.04 | 0.87 | 0.76 | 6.92 | 4.62 | 3.19 |
|  | L = 24 | 0.94 | 0.8 | 0.71 | 6.93 | 4.63 | 3.19 |
| M = 4 | L = 16 | 1.52 | 1.27 | 1.11 | 8.25 | 5.58 | 3.92 |
|  | L = 24 | 1.37 | 1.17 | 1.04 | 8.16 | 5.52 | 3.92 |
|  | L = 32 | 1.28 | 1.11 | 1.1 | 8.11 | 5.5 | 3.9 |
|  | L = 56 | 1.14 | 1.02 | 0.94 | 8.07 | 5.47 | 3.89 |

Table 8 LP-RSRP for LP-SS periodicity of 320ms using X = {2,4,8} sample averages under SIR of 3dB with PDSCH data.

The impact of such a clustered 1’s and 0’s is depicted in Table 7 and Table 8, where the signal power variation is large due to uneven concentration of ON and OFF locations, if Manchester encoding is not used. However, if Manchester encoding like restriction is used, then the signal power remains same and as observed in Table 7 and Table 8, the RSRP estimation using both S and D technique performs the same. Finally, by averaging over multiple LP-SS occasions, the accuracy of RSRP can be improved as shown in Table 7 and Table 8. The performance of M={2,4} with averaging performs similar or even better than M=1 due to power boosting within a symbol.

Averaging over multiple LP-SS MOs improves the RSRP estimation accuracy when employed with uniform power distribution together with or without pulse shaping.

Uniform power levels on all OOK ON ensure reliable LP-RSRP estimation by the LR.

The final design is up to RAN1, but in RAN4 assumptions the LP-SS sequence length should be at least {12,16} for M = {2,4}, respectively, together with the averaging over ≥4 LP-SS observations to ensure reliable LP-RSRP.

LP-RSRP measurement metric for LP-WUR

Traditionally, RSRP is used as the measurement metric to determine the cell coverage/range. In the LP-WUS context, it is one of the possible reliable metrics to determine the RRM relaxation thresholds. To study the LP-RSRP estimation accuracy, a time correlated fading of 1.28s is used with 1e6 samples for the LP-RSRP evaluation under TDL-C 300ns channel. In this evaluation, we aimed to determine the value of X, which represents the number of measurements needed within 1.28s that are spaced at equal interval. The following table shows the percentile distribution of LP-RSRP averaging under different SNR operating point. We have determined LP-RSRP based on the signal energy received during the ON duration corrected with the (noise and interference) energy received in OFF duration of Manchester encoded LP-SS, i.e., EON-EOFF.  Table 1 outlines the LP-RSRP error distribution with different operating SNR condition. The “Delta” column highlights half the deviation between 90%ile and 10%ile value. As can be seen that the LP-RSRP deviation is approximately within ±2.5dB error margin for values with X having 320ms or less.

From the table we can notice that the impact of fading on the LP-RSRP estimation accuracy improves as the number of samples used for averaging, i.e., X, increases from 1→16. Furthermore, the LP-RSRP estimation is robust against the SNR as there is only a little variation across when looking at each row. The impact of M on the LP-RSRP estimation accuracy is also not that drastic to gain any obvious attention. However, as the value of X increase from 1→4, the LP-RSRP estimation accuracy improves noticeably, whereas increasing from 4→16, the gain is only marginal. This ensures that the effect of fading can be normalized by taking at least X≥4 samples that are spread across 1.28sec interval.

Table 1. Measured LP-RSRP percentile values for Manchester encoded OOK with M∈{2,4}  and X∈1,2,4,8,16.

| M | X | interval |  | SNR 0dB |  |  | Delta |  | SNR 6dB |  |  | Delta |  | SNR -3dB |  |  |  | Delta |  |  |  | SNR -9dB |  |  |  | Delta |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ms) |  | 10%ile | 50%ile | 90%ile |  |  | 10%ile | 50%ile | 90%ile |  |  | 10%ile | 50%ile | 90%ile |  |  |  |  | 10%ile |  | 50%ile | 90%ile |  |  |
| 2 | 16 | 80 |  | -2,15 | -0,39 | 1,28 | 1,715 |  | -2,1 | -0,36 | 1,29 | 1,695 |  | -2,22 | -0,45 | 1,24 | 1,73 |  |  |  | -2,41 |  | -0,6 | 1,1 | 1,755 |  |
| 2 | 8 | 160 |  | -2,31 | -0,44 | 1,36 | 1,835 |  | -2,25 | -0,4 | 1,37 | 1,81 |  | -2,38 | -0,48 | 1,33 | 1,855 |  |  |  | -2,59 |  | -0,65 | 1,18 | 1,885 |  |
| 2 | 4 | 320 |  | -3,26 | -0,58 | 1,72 | 2,49 |  | -3,17 | -0,54 | 1,75 | 2,46 |  | -3,32 | -0,64 | 1,71 | 2,515 |  |  |  | -3,56 |  | -0,82 | 1,6 | 2,58 |  |
| 2 | 2 | 640 |  | -4,81 | -0,9 | 2,35 | 3,58 |  | -4,7 | -0,87 | 2,36 | 3,53 |  | -4,93 | -0,96 | 2,32 | 3,625 |  |  |  | -5,18 |  | -1,14 | 2,18 | 3,68 |  |
| 2 | 1 | 1280 |  | -7,16 | -1,58 | 3,03 | 5,095 |  | -7,02 | -1,53 | 3,05 | 5,035 |  | -7,24 | -1,65 | 3,02 | 5,13 |  |  |  | -7,73 |  | -1,88 | 2,89 | 5,31 |  |
| 4 | 16 | 80 |  | -2,27 | -0,47 | 1,22 | 1,745 |  | -2,17 | -0,41 | 1,27 | 1,72 |  | -2,38 | -0,56 | 1,17 | 1,775 |  |  |  | -2,63 |  | -0,78 | 0,98 | 1,805 |  |
| 4 | 8 | 160 |  | -2,42 | -0,51 | 1,3 | 1,86 |  | -2,31 | -0,44 | 1,34 | 1,825 |  | -2,53 | -0,58 | 1,25 | 1,89 |  |  |  | -2,83 |  | -0,8 | 1,08 | 1,955 |  |
| 4 | 4 | 320 |  | -3,38 | -0,66 | 1,69 | 2,535 |  | -3,26 | -0,59 | 1,72 | 2,49 |  | -3,53 | -0,75 | 1,63 | 2,58 |  |  |  | -3,8 |  | -0,98 | 1,46 | 2,63 |  |
| 4 | 2 | 640 |  | -5,01 | -0,99 | 2,3 | 3,655 |  | -4,81 | -0,91 | 2,33 | 3,57 |  | -5,2 | -1,07 | 2,25 | 3,725 |  |  |  | -5,57 |  | -1,33 | 2,12 | 3,845 |  |
| 4 | 1 | 1280 |  | -7,5 | -1,69 | 2,97 | 5,235 |  | -7,22 | -1,61 | 3,02 | 5,12 |  | -7,67 | -1,81 | 2,97 | 5,32 |  |  |  | -8,64 |  | -2,04 | 2,79 | 5,715 |  |

The objective is to determine the number of distinct LP-SS based measurement samples that are spaced at equal interval within a period of 1.28s, to normalize the effect of channel fading. As the fading varies slowly in case of static UEs, it is evaluated over a period of 1.28s, which is several times greater than the coherence time of 3kmph UE speed. Thus, the estimated LP-RSRP is a true estimate of path loss, and thus normalizing the effect of fading channel. The following figure illustrates the LP-RSRP estimation error with ideal path loss. It can be seen that LR may need more than four measurements using LP-SS that are spread across 1.28s to obtain a reliable LP-RSRP estimate that characterizes the actual path loss.

Fig. 1 LP-RSRP evaluation over 1.28s using X sample averages on LP-SS in TDL-C 300ns.

The effect of OOK modulation order M is not visible in the LP-RSRP estimation accuracy. Thus, using the above evaluations, we may need four measurements that are equally spaced between 1.28s to obtain a reliable LP-RSRP estimation that limits the error ≤3dB in fading conditions.

Depending on the operating SNR, a LP-RSRP presentative of the pathloss can be obtained using X ≥4 LP-SS samples that are spread over 1.28s at equal interval.

A minimum of X≥4 LP-SS samples are required to estimate LP-RSRP reliably irrespective of the operating SNR.

Our simulations indicate that X≥4 LP-SS samples are required to estimate LP-RSRP reliably irrespective of the operating SNR.

Simulate LP-SS RSRP accuracy with 4 samples with 16bit LP-SS and M=2, 4

Conclusion

The following Observations and Proposals were made:

Proposal 1: Agree on a set of simulation scenarios each company provides to the next meeting.

Observation 1: Averaging over multiple LP-SS MOs improves the RSRP estimation accuracy when employed with uniform power distribution together with or without pulse shaping.

Observation 2: Uniform power levels on all OOK ON ensure reliable LP-RSRP estimation by the LR.

Observation 3: The final design is up to RAN1, but in RAN4 assumptions the LP-SS sequence length should be at least {12,16} for M = {2,4}, respectively, together with the averaging over ≥4 LP-SS observations to ensure reliable LP-RSRP.

Observation 4: Depending on the operating SNR, a LP-RSRP presentative of the pathloss can be obtained using X ≥4 LP-SS samples that are spread over 1.28s at equal interval.

Observation 5: A minimum of X≥4 LP-SS samples are required to estimate LP-RSRP reliably irrespective of the operating SNR.

Proposal 2: Our simulations indicate that X≥4 LP-SS samples are required to estimate LP-RSRP reliably irrespective of the operating SNR.

Proposal 3: Simulate LP-SS RSRP accuracy with 4 samples with 16bit LP-SS and M=2, 4